3GPP TSG-SA1 Meeting #109	S1-250347

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.101 | CR | 0602 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SMS to Emergency Centre |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, KPN, Orange, Viasat, Sateliot, Novamint, Google, SyncTechno Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed in SA-106, current SA1 requirements are not clear, preventing to properly scope potential stage-2 work. SA tasked SA1 to further investigate those service requirements. Furthermore, a LS from GSMA was received asking to generalize the requirements (applying to SMS over NAS as well) and clarify roaming aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to generalize current requirements (moved from sec 10.11 to a higher level section), clarify the applicabiility to SMS over IMS and/or SMS over NAS, simplify some aspects (e.g. removing the case without SIM) and add a new requirement on roaming support (based on the GSMA LS). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current requirements remain unclear and/or unnecessarily constrained. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.11, 36 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ================= First Change ======================

## 10.11	Void

## ================= Next Change ======================

# 36	Short Message Service to emergency centre

EPS and 5GS shall route a Short Message using the emergency numbers to a local emergency response centre (i.e. PSAP), according to the local regulations.

The Short Message shall be routed to the emergency response centre serving the location where the subscriber is located and replies routed back to the subscriber in accordance with national regulations.

The 3GPP system shall support a Short Message to be routed to different emergency response centres, depending on the type of emergency, e.g., as specified in 10.1.

The Short Message Service to emergency response centre shall be able to use the same identification methods as for emergency calls, e.g., as specified in 10.1.1.

The Short Message Service to emergency response centre shall be supported also for roaming UEs, where roaming subscribers’ SMS can be routed to a local PSAP (in the VPLMN).

NOTE: The above requirements can apply to SMS over IMS and/or SMS over NAS.