3GPP TSG-SA WG1 Meeting #109 	S1-250375

Athens, Greece, 17-21 February 2025

Source:	SK telecom, NVIDIA

New use case title:	New use case on Orchestration between RAN and AI workload

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.2

Document for:	Approval

Contact:	hyeonsoo.lee@sk.com, jaehyun_lee@sk.com, xingqinl@nvidia.com

Abstract: <provide a short description of the content>

This contribution proposes a new use case on Orchestration between RAN and AI workload.

It is the revised contribution of S1-250210 with adding co-source ‘NVIDIA” and editorial changes.

---------- Use Case template ----------

x.1	Use case on Orchestration between RAN and AI workload

x.1.1	Description

<Describe what the use case intends to achieve.>

Operators facing revenue stagnation due to demographic changes. Given the substantial investment is required for generational change, it is crucial to explore new monetization opportunities for operators beyond cost reduction, especially with the advent of the 6G era. One of the most promising approaches is leveraging existing Telco assets, ranging from short-term strategies such as providing colocation services to long-term initiatives that repurpose base stations as platforms for delivering AI services.

Base stations process large amounts of data, including physical layer signal processing and complex radio resource allocation in RAN. To enhance RAN performance, the use of xPUs (e.g., GPUs) is expected to increase, which can be the starting point for AI-native RAN. On the other hand, base stations may experience traffic surges in specific cells and time periods where users are concentrated. This makes it possible to utilize idle base station resources during off-peak hours for AI services, providing opportunities to generate additional revenue for operators.

AI services require real-time inference that demands low latency. Because base stations are located close to users, they are well-suited to meet these requirements. This strengthens the role of base stations as a common infrastructure for both RAN and AI workloads.

Therefore, resource orchestration between RAN and AI workloads is one of the core technologies for AI-native RAN. The key challenge in resource orchestration is to guarantee the RAN performance with or without AI workloads in resource management. Therefore, within a single base station, hardware resources such as GPUs should be allocated dynamically between RAN and AI workloads without any performance degradation on RAN, while ensuring the AI service requirement in the same time.

x.1.2	Pre-conditions

The operator's base stations are equipped with xPUs(e.g., GPU), enabling the processing of both AI workloads of application servers and RAN workloads

Each base station monitors the current HW resource usages for each workload, and an orchestrator in management layer collects this information from multiple base stations and distributes AI workloads accordingly.

Ensuring RAN performance consistency (e.g., bit error rate, latency, etc.)  is the highest priority for the operator's base stations.

x.1.3	Service Flows

- Base stations A and B provides AI workload as well as RAN workload and each base station monitors its own hardware resource usage.

- Base station A and B report monitoring information and their current network status, including traffic volume and patterns to the Orchestrator

- As mobile traffic increases at base station A, more hardware resources are needed to handle the additional RAN workload.

- To allocate more resources for RAN processing at base station A, the orchestrator decides to migrate the AI workload from base station A to base station B.

- Based on the orchestrator's decision, hardware resource allocation is updated at both base station A and B accordingly.

x.1.4	Post-conditions

None

x.1.5	Existing features partly or fully covering the use case functionality

None

x.1.6	Potential New Requirements needed to support the use case

The 6G system shall provide a framework for operating AI workloads possibly provided by 3rd party.

The 6G system shall orchestrate to allocate hardware resources of base stations for RAN and AI workloads adaptively.

The 6G system shall prioritize ensuring mobile network performance(e.g., bit error rate, latency, etc).