3GPP TSG-SA WG1 Meeting #109 	S1-250423

Athens, Greece, 17-21 February 2025	(revision of S1-250160, 408)

Source:	CATT

pCR Title:	Update on 5.18 Use Case

Draft Spec:	3GPP TR 22.887 v1.0.0

Agenda item:	7.3.1

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes an update to the requirement of 5.18 use case in TR22.887.

1. Introduction

The editor’s note was added to PR 5.18.6-001 and this contribution is proposed to solve the EN.

2. Reason for Change

The editor’s note was added to PR 5.18.6-001 for the remaining work on study the relation with multi-orbit. The whole use case need to be revisited and clarify what is the gap in current system to support the use case. Then reword the PR 5.18.6-001 for better understanding.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.887 v1.0.0

* * * First Change * * * *

5.18	Use case on using GEO to land data in a desirable geographical location

5.18.1	Description

NGSO satellites typically have a much smaller field of view (FOV) than a GEO satellite. Thus NGSO satellites have to either download traffic from a UE to a gateway within its FOV, or transfer via inter satellite links (ISL) the data to another NGSO satellite in order to land the data outside the FOV of the satellite connecting to the UE. These ISLs sometimes often do not have as much bandwidth as the primary links. ISLs may also be unavailable under various conditions due to satellite duty cycle constraints or routing conflicts.

It is also desirable for many customers to download data to network within a specific geographic areas. This may be for intellectual property, security, or national security requirements. Many customers may want to avoid areas that belong to adversarial nations or have geopolitical instability. We will refer to this as data with a preferred satellite ground station location.

Using a multi-orbit satellite system allows differentiated data landing based on customer needs. E.g., the UE can connect to a NGSO and then have its data sent to a GEO satellite from which the data can be landed anywhere in the field of view of the GEO. It may also be the case that the UE with data with preferred satellite ground station location, directly communicates with a GEO satellite bypassing the NGSO entirely.

5.18.2	Pre-conditions

- The network supports both GEO and NGSO satellite access

- The network supports the mobility between GEO satellite(s) and NGSO satellite(s)

- A network plan deconflicts frequency allocations between the GEO and NGSO satellites.

- The network is able to coordinate the user to the proper NGSO coverage beam or cell based on the location of a user in a sufficient granularity

- The network must be able to determine whether each satellite network connecting to the preferred satellite ground station within the desirable geographical location.

5.18.3	Service Flows

- UE intends to exchange the data of different services with the applications. Considering the regulation and/or the user preference and/or the operator’s policy, the traffic of each service is allowed to be delivered to specific geographical locations, where have satellite ground stations deployed. During communication, the network or UE determines that data will be exchanged via a selected satellite ground station.

- The network will route traffic through an optimal path between UE and the selected ground station located in a preferred geographical location.

5.18.4	Post-conditions

Data of a specific service is delivered to a specific geographical location via the preferred satellite ground station(s).

5.18.5	Existing features partly or fully covering the use case functionality

Existing 3GPP Rel-19 NTN features may  cover the use case partially.

5.18.6	Potential New Requirements needed to support the use case

[PR 5.18.6-001] Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support a mechanism to deliver specific data traffic of a UE related to specific service (s) to a preferred  geographical location.

NOTE: 	There are satellite ground stations deployed within the preferred geographical location. The link between satellite and satellite ground station is out of 3GPP scope.

* * * End of Changes * * * *