3GPP TSG-SA WG1 Meeting #109 	S1-250531

Athens, Greece, 17-21 February, 2025	(was S1-250179)

Title:	Use case on network controlled dynamic network slicing

Agenda Item:	8.1.1 (6G study - System and Operation Aspects)

Source:	Samsung

Contact:	DongEun Suh <dongeun.suh@samsung.com>

Abstract: This contribution proposes a use case on network-controlled dynamic network slicing.

Discussion

This contribution proposes a use case on network controlled dynamic network slicing.

Proposal

It is proposed to make the change listed below to TR 22. 870.

Begin changes

5.x	Support of network controlled dynamic network slicing

5.x.1	Description

Network slicing is a technology that enables customized virtual networks to be deployed and operated so as to meet various service requirements.

In 5G network slicing, an UE is required to support the network slice selection functionalities. That is, the UE selects the network slice that it would access/use among the list of network slices configured from the network. In a scenario where network slices for the UE need to be changed over time (e.g., due to location of the UEs, subscription update, change of policies, slice congestion, etc.), the UE needs to be signaled to receive the updated configuration. To support dynamic functionality, 5G network slicing relies on complex interactions between the UE and network.

Meanwhile, for simpler UEs, e.g. low-cost smartphones, IoT devices, the network slice selection functionalities in the UE and the signaling due to the change of the network slices is an unacceptable overhead, as it assumes substantial terminal awareness and functionality with respect to network functionality.

The form and timing of the interaction between the UE and the network is an architectural matter, out of scope of stage 1. The service scenario of a UE too simple or resource constrained to interact with the network during changing conditions is within scope of stage 1. This use case concerns the following scenario:

-	Dynamic conditions determining the desirability to change network slice (e.g. due to application requirements changing over time, the location of the UE, conditions of the network, etc.)

-	Inability or unsuitability of the UE to initiate Network Slice related operations, e.g. the UE is a simple device, the UE is a split terminal device without knowledge of application traffic carried by the upper layers, etc.

The corresponding 5G requirements from TS 22.261 below explicitly requires the UE to initiate change in network slice support in dynamic conditions.

- The 5G system shall support a mechanism for a UE to select and access network slice(s) based on UE capability, ongoing application, radio resources assigned to the slice, and policy (e.g., application preference).

In 6G, this use case extends support to enable dynamic network slice functionality autonomously by the network.

5.x.2	Potential New Requirements needed to support the use case

[PR.5.x.2-1]	The 6G system shall support a mechanism for the 6G network to select network slice(s) based on, e.g., UE capability, ongoing application, radio resources assigned to the slice, and policy (e.g., application preference).

End of changes