3GPP TSG-SA WG1 Meeting #109 	S1-250651

Athens, Greece, 17-21 February 2025	(revision of S1-250285)

Source:	Huawei, China Mobile

pCR Title:	Update of Use Case 7.6

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	Fangyuan Zhu< zhufangyuan@huawei.com >

Abstract: This document provides some updates for the use case 7.6.

1. Introduction

The aim of this contribution is to update the use case 7.6 by addressing the fowllowing Editor’ s Note:

Editor’s Note:	The definition of sensing target density and whether communication quality will be taken into account in the above PR is FFS.

2. Reason for Change

2. 1  Definiton

The definiton of sensing target density is proposed in the following:

sensing target density: total number of objects to be sensed per geographic area. It is a measure of how many objects the 3GPP system can detect, identify and/or track within a target sensing service area.

2. 2  KPI values of sensing target density for road digitalization

In order to monitor traffic volume constantly, this use case intends to detect and track all moving objects, including vehicles, bicycles and pedestrians effectively.

It is proposed to introduce different vehicle densities to represent a few typical urban transport scenarios based on vehicle speed and road environment (e.g., crossroad, highway).

According to the TR 37.885[1], three vehicle types are defined as follows:

-	Type 1 (passenger vehicle with lower antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 0.75 meters

-	Type 2 (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters

-	Type 3 (truck/bus): length 13 meters, width 2.6 meters, height 3 meters, antenna height 3 meters

Normally, the vehicle densities per area depends on the stopping distance between vehicles at different velocity, which allows driver to stop safely within the certain distance. The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is the average of the speed * 2 sec.

The sensing area and assumptions of this use case are described in the Figure 1 and Table 1.

Figure 1: Sensing area

Table 1: Asumptions

| Sensing target | Size (Length x Width x Height) | Velocity | Distance between vehicles[1] | Sensing area (Note 1) |
| --- | --- | --- | --- | --- |
| Vehicle | 5m x 2m x 1.6m | Up to 5km/h(Static/moving) | 3m | Crossroad (Note 1) |
|  |  | Up to 120km/h | 66m | Highway (Note 2) |
| Note 1: One crossroad area is approximated by 1000m dual three-lane carriageway(24m), by assuming 250m per direction at the crossroad. Note 2: Based on an assumption of dual three-lane carriageway (1000m x 24m). |  |  |  |  |

Therefore, the different values for sensing target density at different sensing area can be desribed in the  Table 2.

Table 2

| Sensing target density | Sensing target Velocity |
| --- | --- |
| 750 vehicles/Crossroad (Note 1) | Static/moving, up to 5km/h |
| 85 vehicles/Highway (Note 2) | Moving, up to 120km/h |
| Note 1: One crossroad area is approximated by 1000m dual three-lane carriageway(24m), by assuming 250m per direction at the crossroad. Note 2: Based on an assumption of dual three-lane carriageway (1000m x 24m). |  |

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.1.1.

4. Refernece

| [1] | 3GPP TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR |
| --- | --- |

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

…….

[r1]		3GPP TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR.

* * * Second Change * * * *

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

sensing target density: total number of objects to be sensed per geographic area. It is a measure of how many objects the 3GPP system can detect, identify and/or track within a target sensing area.

* * * Third Change * * * *

## 7.6	Use Case on road digitalization

### 7.6.1	Description

Many transportation and urban applications require real-time and citywide traffic flow estimation, which is the basis for transportation planning and traffic control. Estimated traffic flow is generally represented by the number of cyclists/vehicles/pedestrians passing a reference location per unit of time and can be virtualized by a third-party application.

In order to enable real-time navigation for automatic driving or traffic flow monitoring, it is necessary to define the upper limit of sensing object detection/tracking required by a 3GPP sensing service, since any 3GPP based sensing detection/tracking coexists with communication services and is not cost-free for operators by sharing. 3GPP system shall allocate limited but sufficient spectrum resources, power, base stations and/or road side units and etc for sensing operations. Such kind of necessity can be represented by sensing target density

For example, the number of vehicles that needs to be simultaneously detected and tracked at a crossroad may be up to 1000 cars per [km2], including all stationary and moving vehicles occupying that cross-road temporally:

The assumptions of this use case are described in the Table 7.6.1-1.

Figure 7.6.1-1: Sensing area

Table 7.6.1-1: Assumptions

| Sensing target | Size (Length x Width x Height) | Velocity | Distance between vehicles[r1] | Service area (Note 1) |
| --- | --- | --- | --- | --- |
| Vehicle | 5m x 1.75m x 1.5m | Up to 5km/h(Static/moving) | 3m | Crossroad (Note 1) |
|  |  | Up to 120km/h | 66m | Highway (Note 2) |
| Note 1: One crossroad area is approximated by 1000m dual three-lane carriageway(24m), by assuming 250m per direction at the crossroad. Note 2: Based on an assumption of dual three-lane carriageway (1000m x 24m). |  |  |  |  |

### 7.6.2	Pre-conditions

Good partnership and cooperation are established between Traffic Department A and Mobile Operator B. Traffic Department A subscribes the 3GPP wireless sensing service from Mobile Operator B for the real-time road digitalization.

In order to monitor traffic volume constantly, Mobile Operator B has deployed and activated base stations and road side units around traffic intensive areas, such as crossroads, urban roads and highways, in order to provide a wide sensing coverage and capability to detect and track all moving objects, including vehicles, bicycles and pedestrians effectively.

NOTE:	For the ease of elaboration, base station is acting as sensing transmitter and/or sensing receiver in this case. Other sensing modes can also be feasible and useful as well.

### 7.6.3	Service Flows

Figure 7.6.3-1

Figure 7.6.3-2

1.	Traffic Department A receives a lot of complaints from local residents about heavy traffic at crossroads and highways due to ongoing city construction or bad route planning.

2.	Traffic Department A requests Mobile Operator B to detect and track at least [100] sensing objects simultaneously including cyclists, pedestrians and vehicles at crossroads shown in Figure 7.6.3-1, and at least [50] sensing objects simultaneously including high-speed vehicles at overpasses shown in Figure 7.6.3-2. According to preferred sensing target density, base stations and road side units deployed by Operator B along these areas constantly monitor the road situation by appropriate sensing operations according to these requests.

3.	Mobile Operator B collects 3GPP sensing data from base stations and road side units, and non-3GPP sensing data from sensors (e.g. radar, cameras) if available. Then Mobile Operator B provides Traffic Department A with real-time traffic flow information, including vehicle volume of different directions within a unit time (e.g. every 10 minutes, every half hour), average queuing time for a vehicle/pedestrian as the crossroad, etc.

4.	For more demanding traffic monitoring, Traffic Department A requests Mobile Operator B to detect and track more objects simultaneously at given crossroads and overpasses during rush hours. Mobile Operator B adapts to the request by updating the configuration parameters of sensing operations.

5.	Based on sensing results exposed by Mobile Operator B for road digitalization, Traffic Department A figures out solutions to improve the utilization of public roads at real-time by finer spatial/temporal traffic control and traffic divergence.

### 7.6.4	Post-conditions

Thanks to the network-wide coverage, the bird's-eye-view of base station, road digitalization is enabled by capturing real-time information of the road environment.

### 7.6.5	Existing features partly or fully covering the use cast functionality

The capability to provide secure means for a trusted third-party to request the sensing target density object tracking is not covered.

### 7.6.6	Potential New Requirements needed to support the use case

[PR 7.6.6-1]	Subject to regulation and operator's policy, 6G system shall efficiently support sensing target density requested by the third party for a given sensing service.

[PR 7.6.6-2] The 6G system shall be able to support the following KPIs:

Table 7.6.6-1

| Scenario | Service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Sensing target density （Note 3） | Sensing target Velocity | Max sensing service latency [s] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |  |
| Road digitalization | Outdoor | 95 | 0.5(vehicle) 1（Pedestrian） | N/A | 0.5 | N/A | 0.2 | 0.2 m/s | ≤ 750 Vehicles Per 1000m x 24m ≤100 Pedestrian Per 1000m x 24m Note 1 | Static/moving, up to 5km/h | 1~5 | ≤ 0.1 | ≤5 | ≤5 |
|  | Outdoor | 95 | 3 | N/A | 3 | N/A | 1 | 1m/s | ≤85 vehicles Per 1000m x 24m Note 2 | Moving, up to 120km/h | 1~5 | ≤ 0.1 | ≤5 | ≤5 |
| Note 1: One crossroad area is approximated by 1000m dual three-lane carriageway (24m), by assuming 250m per direction at the crossroad. Note 2: Based on an assumption of dual three-lane carriageway (1000m x 24m). Note 3: The definition of sensing target density is described in Clause 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Changes * * * *