3GPP TSG SA WG 1 Meeting #109	S1-250735

Athens, Greece, 17-21 January, 2025	(revision of S1-240220 was4836)

Source:		OPPO, Huawei, Toyota, CATT, vivo

Title:	Use case of 6G system assisted target object detection

Agenda item:	8.1.7

Document for:	agreement

Contact:	Yang Xu (xuyang@oppo.com)

Abstract: This contribution provides a use case of target object detection to by leveraging computation in 6G

********** Start of Change *********

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	Zhang Z, Wang S, Hong Y, et al. Distributed dynamic map fusion via federated learning for intelligent networked vehicles[J]. arXiv preprint arXiv:2103.03786, 2021

[x2]	AECC: Operational Behavior of a High Definition Map Application – White Paper Version 1.0.0

5.x	6G system assisted target object detection

5.x.1	Description

Figure 5.x.1-1 6G system assisted data processing for a task

Aside from 6G system providing a data transmission (3GPP communication service), on top of the connectivity, some servers in the 6G CN or operator managed DN (e.g. edge compute domain) may be connected to provide data processing using its local computation resource. As the figure above shown, UE and one or more nodes in 6G CN, operator managed DN (e.g. edge compute domain) and Application server are involved. The 6G network can gurantee the e2e task latency considering the apportionment of communication latency as well as the processing latency.

NOTE: The 6G system may adjust the resource of  the processing server in the 6G CN or in the operator managed DN (e.g. edge compute domain) considering the transmission latency to satisfy the needed the overall joint latency (communication latency plus processing latency).

This contribution introduces a use case where the data processing is performed by 6G system for a target object detection task,

5.x.2	Pre-conditions

Figure 5.x.2-1 6G system assited target object detection

Vehicle:

support LiDAR sensor to sense the target car. It tgenerate data in in cloud-point format and transmit the data to RAN node;

RAN node:

support RF sensor to sense the target car. It generates the measurement data in cloud-point format and send the data to corenetwork.

NOTE: the RAN node has a overview inforamtion about the target and the vehicles surrounding the target car can get the detailed data about a specific direction to the target, which helps increase the inference accuracy.

Core network:

The core network is installed with aggregation model, which is used to aggregate the input data from UEs and RAN. Then transmit the aggregated data to the Edge server supporting inference model for Car positioning.

Edge Server:

Support the inference model. Perform the inference result of the shape and position of the target object based on the aggregated data from corenetwork and the video data from the infrastructure camera.

5.x.3	Service Flows

0) the aggregation model and inference model has been offline trained in cloud server and provisioned into CN node and edge computing server in 6G system.

1) When a car collison happens, the 6G system selects three vehicles which are surrounding the target object (i.e. the collided cars) and capable of doing LiDAR sensing service [x1]. As example, the selection may consider one or more of the aspects: capabilities and computation resource of UEs and/or networks, the network congestion status, and the certain task request (e.g. the accuracy of a sensing request). Then, the vehicles generate the LiDAR sensing data meanwhile the RAN node generates the RF sensing data, both the vehicles and RAN node transmit the data in cloud-point format. The serving core network node (e.g. connected behind the user-plane function) generates the output (aggregated data) using aggregated model based on the measurement input from vehicles and RAN node.

2) The edge computing server received the video data sent by the infrastructure camera and received the aggregated data from CN node. The edge server then do the inference using the inference model to derive the accurate shape and position of the target object.

3) The edge server generates the output i.e. inference result, which could be updated into HD map for auto-driving service. The edge server may send the updated HD map to all vehicles so that the vehicle can perform the correct driving decision based on the updated HD map [x2].

5.x.4	Post-conditions

Thanks to the in-time detection of the target object, it can quickly inference the accurate shape and position of the collided cars so that the HD map can be updated in time.

5.x.5	Existing features partly or fully covering the use case functionality

3GPP TS22.261 Clause 6.40 and 7.10 describe the communication functionality and KPIs for AIML model transfer in 5GS. Insted of assuming to be performed as a mere transmission pipeline, the 6G system may get involved for an AIML operation by providing computation resources, data, model and enhanced connections.

Thus, it requires more functionality and KPIs for 6G network to perform AIML operation as a native AI service.

5.x.6	Potential New Requirements needed to support the use case

Functionality Requirements:

[P.R. 5.x.6-1] Subject to user consent and operator policy, 6G system shall be able to select UEs and/or network entities (i.e. in the 6G core network, or as trusted edge server) to jointly complete a task.

[P.R. 5.x.6-2] Subject to user consent and operator policy, 6G system shall be able to guarantee an E2E joint latency.

Note: the E2E joint latency includes the time for data transmission and data processing.

Note: the above term does not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN (compared to 5G).

*******************End of Change*****************