3GPP TSG-SA WG1 Meeting #109 	S1-250824

Athens, Greece, 17-21 February 2025	(revision of S1-250166)

Source:	CATT

pCR Title:	Update on W.2 Use Case

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda item:	8.1.8

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes to introduce the update to the use case of Optimizing 6G Infrastructure Utilization in clause W.2, to capture additional potential requirements identified to TR22.870.

1. Introduction

The use case of Optimizing 6G Infrastructure Utilization via Resource Exposure was discussed and agreed in SA1#108 meeting. And some associated requirements were identified and agreed as well. Taking the above result as the basis, this contribution proposes to introduce additional potential requirements identified from the service flow in clause W.2 TR 22.870.

2. Reason for Change

As the use case mentions, the spare compute, storage and AI processing resources are possible to be monetized and exposed to 3rd party as a service depending on the request and the policy. Generally, compute resources can include generic compute resources and AI-specific compute resources that request additional built-in AI capabilities such as AI operating system, runtime environment, SDKs, and are dedicated for AI services. Corresponding, AI processing resources can refer to AI-specific resources with both computing capabilities and AI capabilities to support AI based applications, such as AI training capability, AI inference capability and etc. Thus, the potential requirements need to cover all the possible cases.

Observation 1: PR#1 has not covered all the possible resources discussed in the use case.

Moreover, regarding the service flow, 6G system needs to get the 3rd party’s request before initiate the service as illustrated in Step1 of the service flow. And the resources allocation need to be managed as illustrated in Step4 of the service flow. Those items are not covered by the defined potential requirements.

Observation 2: Existing defined PRs have not covered all the network capability illustrated in the service flow.

Therefore, the following changes are proposed to be introduced.

(1) More descriptive sentence about resources in Pre-condition;

(2) Align Step1 service flow with PR1 and cover all the possible offering in PR1;

(3) New PR for information exposure and resource allocation management;

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.870 v0.1.1

* * * First Change * * * *

W.2	Use case on Optimizing 6G Infrastructure Utilization via Resource Exposure in 6G

W.2.1 	Description

With the advent of advanced technologies such as Generative Artificial Intelligence (Gen AI), Large Language Models (LLMs), and AI-accelerated xPUs, wireless network operators are now equipped with substantial computational and storage infrastructures that often remain underutilized. These computation resources may be hosted at edge sites in a distributed manner. It is in the strategic interest of these operators to expose these dormant resources to third-party entities, allowing them to leverage the computational power and AI capabilities when required. In exchange, operators can monetize these assets by charging nominal fees for compute cycles, AI processing, and storage utilization by third parties.

This approach not only generates additional revenue streams for network operators but also ensures the optimal utilization of existing infrastructure. Consequently, it justifies the need for new requirements in 6G networks to expose and manage network resources effectively. By incorporating standardized interfaces, robust security protocols, and advanced resource management frameworks, 6G can facilitate efficient and secure access to these resources. This transformation would position 6G networks as comprehensive platforms for distributed computing and AI services, aligning with the evolving demands of the technology landscape and provide resource as a service. This unique network-aware resource exposure capability is enabled by the 6G system’s centralized policy framework.

The 6G system will be built on the cloud, optimized for ubiquitous computing, and natively provide computing services (e.g., in forms of infrastructure services, platform services, and software services). [5.1X]

Figure W.2.1-1: Illustration of network resource usage and exposure to 3rd party

NOTE:	The figure above is just an illustration to depict where computation resources could be deployed e.g. at an edge site.

W.2.2	Pre-conditions

Network Operator ‘A is equipped with significant compute, storage, and AI processing resources. The compute resources can be used for generic computing of various types of conventional applications, or AI-specific computing with additional installed AI capabilities such as AI operating system, runtime environment, SDK and etc., for AI services. AI processing resources can refer to the dedicated resources with AI compute capabilities (e.g. GPU, accelerators), AI capabilities (e.g. operating environment, AI models) and/or AI services (e.g. AI training services, AI inference services)to support AI based applications. Some of these resources are used by NFs or OAM etc. However, resource utilization varies based on network load, and during periods of low demand—often at certain times of day or night—up to 40% of these resources remain idle. These idle resources represent untapped potential, lying unused with no opportunity for additional utilization.

Meanwhile, Operator ‘A’s customer, a gaming company called ‘X,’ experiences high demand for computational resources during nighttime hours when many gamers are online on both computers and mobile devices. As games become increasingly complex and distributed, companies like ‘X’ are integrating large language models (LLMs) into their platforms to enhance gameplay. Offloading some of this processing to external infrastructure would be beneficial, particularly if ‘X’ could access these idle resources at a competitive rate. Being an existing customer of Operator ‘A,’ company ‘X’ could potentially secure these resources at favourable rates.

W.2.3	Service Flows

1	A third-party requests access to specific compute, storage or AI processing resources via an API provided by Operator X

2	The 6G network verifies the request against defined policies and authorizes access if conditions are met.

3	The 6G network checks with the policy framework, which in turns may confirm with orchestration system if the resources are available, how many of them are used for NF and if the load is expected to increase based on history of network usage. Upon approval, the 6G system allocates the requested resources and establishes secure access for the third-party. This unique network-aware resource exposure capability is enabled by the 6G system’s centralized policy framework.

4	The operator could also add its own rules and policy to manage the resource allocation with the help of policy framework.

5	The system continuously monitors resource usage and records data for charging and reporting purposes.

6	The 3rd party is also allowed to analyse the usage in real time.

W.2.4	Post-conditions

1	Third-party entity completes its tasks using the allocated resources.

2	The 6G network releases the resources and updates the availability status in real-time

3	Usage data is logged, and relevant charges are generated and sent for billing.

4	Policy function is in control of entire operation.

W.2.5	Existing features partly or fully covering the use case functionality

None

W.2.6	Potential new requirements needed to support the use case

[PR-W.2.6-1] Based on operator policy, the 6G network shall provide suitable APIs to allow authorized third parties to retrieve availability information about computational resources (e.g., storage, AI processing resources, xPUs etc.) inside operator managed data network (e.g., similar to Service Hosting Environment, Edge Compute domain) and to utilize these computational resources for running workloads on demand.

NOTE: AI processing resources can refer to the dedicated resources with AI compute capabilities (e.g. GPU, accelerators), AI capabilities (e.g. operating environment, AI models) and/or AI services (e.g. AI training services, AI inference services) to support AI based applications

[PR-W.2.6-2] The 6G network shall support charging for the usage of network resources by third parties.

[PR-W.2.6-3] The 6G network shall support detailed monitoring and reporting of computational resource usage in the operator managed data network (e.g., similar to Service Hosting Environment, Edge Compute domain).

[PR-W.2.6-4] The 6G network shall provide means for an operator to acquire the information about the request and preference of computational resources (e.g. types, AI capabilities, etc.) from the authorized 3rd parties and to allocate proper computational resources based on the operator’s policy.

* * * End of Changes * * * *