3GPP TSG SA WG 1 Meeting #110 	S1-252015

19-23 May 2025, Fukuoka, Japan	(revision of S1-25xxxx)

Source:	6G Rapporteurs

pCR Title:	Pseudo-CR on Service Continuity (5.2.x)

Draft Spec:	3GPP TR 22.870v0.2.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Initial service continuity text

1. Introduction

At the last meeting, there were diverse opinions on how to progress 6G service continuity requirements.  The rapporteurs offer this text as a basis to find consensus on this topic.

2. Reason for Change

- TR 22.905 defined “service continuity” in Clause 3:

Service Continuity: The uninterrupted user experience of a service that is using an active communication (e.g. an ongoing voice call) when a UE undergoes a radio access technology change  or a CS/PS domain change without, as far as possible, the user noticing the change.

NOTE:	In particular Service Continuity encompasses the possibility that after a RAT / domain change the user experience is maintained by a different telecommunication service (e.g. tele- or bearer service) than before the RAT / domain change.

- TS 22.261 modified that definition in Clause 3 as follows:

service continuity: the uninterrupted user experience of a service that is using an active communication when a UE undergoes an access change without, as far as possible, the user noticing the change.

NOTE 8:	In particular service continuity encompasses the possibility that after a change the user experience is maintained by a different telecommunication service (e.g. tele- or bearer service) than before the change.

NOTE 9:	Examples of access changes include the following. For EPS: CS/PS domain change. For EPS and 5G: radio access change, switching between a direct network connection and an indirect network connection.

- In TS 22.261, service continuity is mentioned 39 times (including table of contents and clause titles) for various services and network access types.  With this proposal, the Rapporteurs try to find consensus to consolidate the essential points made concerning service continuity (e.g., uninterrupted customer experience, seamless mobility, minimized packet loss and interruption time) and using the normative text in TS 22.261 as the basis for further discussions.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870v0.2.1.

* * * * First Change * * * *

Insert as a new clause.

### 5. X	Service Continuity

Improving the subscriber experience is an on-going concern for network operators, and service continuity  during changes in access or network connectivity, is an important user experience requirement.  The 6G system is expected to support, at a minimum, the existing 5G service continuity requirements as defined in [14] except for clause 6.18 and requirements on inter-system change (e.g. between 5G and 4G/3G).

NOTE: 	The support and/or modifications of the service continuity requirements in [14] related to inter-system changes between 6G and legacy systems are captured in the Interworking chapter in clause 5.2.2.

* * * * End of Change * * * *