3GPP TSG-SA WG1 Meeting #110 	S1-252162

19-23 May 2025, Fukuoka, Japan	(revision of S1-25xxxx)

Source:	SoftBank Corp., NTT DOCOMO INC., KDDI Corporation, Deutsche Telekom AG, Gilat

New use case title:	New use case on Seamless Coastal and Near-Shore Maritime Connectivity via HAPS

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.5

Document for:	Approval

Contact:	Shiro Fukumoto (shiro.fukumoto01@g.softbank.co.jp)

Abstract: This proposal introduces a use case where HAPS provides seamless mobile broadband connectivity extending from land over coastal and near-shore maritime areas (e.g., up to 200 km). It focuses on bridging the gap between terrestrial and satellite coverage to serve maritime activities like tourism, fishing, and offshore energy platforms, offering potentially higher performance than satellite solutions in these zones.

---------- Use Case template ----------

8.x	Use case on Seamless Coastal and Near-Shore Maritime Connectivity via HAPS

8.x.1	Description

This use case leverages HAPS to extend seamless, high-performance mobile broadband connectivity from land over coastal waters and near-shore maritime areas. This serves various applications, including tourism (cruise ships, ferries, yachts), fishing, offshore energy platforms (wind farms, oil/gas rigs within HAPS coverage), environmental monitoring, and coastal security/surveillance operations. HAPS provides a bridge between terrestrial network coverage ending at the coastline and the deep-sea coverage potentially offered by satellites, offering potentially higher throughput and lower latency than satellite solutions in these near-shore zones (e.g., up to 200 km from the coast).

8.x.2	Pre-conditions

- HAPS platform(s) are deployed with coverage overlapping coastal and near-shore maritime areas.

- UEs (on vessels, platforms, or individual marine users) are within the HAPS service link range.

- Spectrum is available for maritime/coastal usage via HAPS.

- Backhaul connectivity is established.

- Roaming agreements or service provisioning allows users to connect via HAPS.

8.x.3	Service Flows

- UE moving from terrestrial coverage towards the sea performs cell reselection/handover to a HAPS cell as terrestrial signal fades.

- UE initiates connection request to the HAPS network.

- HAPS authenticates UE and establishes data/voice session via its backhaul link.

- User consumes mobile broadband services (internet access, streaming, communication, vessel monitoring data).

- HAPS manages beams and resources to serve multiple users across the maritime area.

- As UE moves back towards land, handover/reselection to terrestrial network occurs.

- For fixed platforms (e.g., wind turbines), HAPS provides continuous connectivity.

8.x.4	Post-conditions

- Users maintain seamless connectivity when transitioning between terrestrial and near-shore maritime areas covered by HAPS.

- Reliable broadband service is available for maritime activities within the HAPS footprint.

8.x.5	Existing features partly or fully covering the use case functionality

- Mobility Management (Handover, Cell Reselection).

- QoS framework.

- Standard authentication and security procedures.

8.x.6	Potential New Requirements needed to support the use case

[PR 8.x.6-1]: The 6G system shall support optimized beamforming and resource allocation strategies that consider over-water propagation characteristics and high mobility of maritime vessels.

[PR 8.x.6-2]: The 6G system shall enable seamless and efficient handover between terrestrial networks and HAPS-based NTN when UEs crosses the coastline.

[PR 8.x.6-3]: The 6G system shall provide QoS mechanisms tailored to maritime applications, including support for real-time monitoring data from offshore platforms.

[PR 8.x.6-4]: The 6G system shall accommodate high user density in localized maritime hotspots such as ferry routes, and popular anchorages.

[PR 8.x.6-5]: The 6G system shall offer enhanced location-services accuracy for UEs operating over water via HAPS.

[PR 8.x.6-6]: The 6G system shall include interference management capability between HAPS maritime cells and adjacent coastal terrestrial networks.