3GPP TSG-SA WG1 Meeting #110 	S1-252256

19-23 May 2025, Fukuoka, Japan

Source:	NTT DOCOMO, NTT

New use case title:	New use case on Digital Identity Management for Digital asset container

Draft TS/TR:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.6

Document for:	Approval

Contact:	Kenta Yamauchi, kenta.yamauchi.xe<at>nttdocomo<dot>com
Naoki Higo, naoki.higo<at>ntt<dot>com

Abstract: This document proposes use cases for Digital Identity (Digital Credential) management utilizing Digital Asset Containers, as defined in Rel.19 TS 22.156. These use cases describe the applicability of Digital Asset Management beyond the Metaverse, particularly focusing on identity verification, accounting, QoS control, and interoperability with external Digital Identity wallets.

1. Introduction

Please see the use case below.

2. Reason for Change

This pCR introduce use case and requirements on Digital Identity Management for Digital asset container for FS_6G-REQ.

3. Conclusions

Define the requirements.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.2.1.

* * * First Change * * * *

## 9.X	Use case on Digital Identity Management for Digital asset container

### 9.X.1	Description

This use case focuses on the digital asset container's functionality related to digital identity and proposes to expand the use cases related to the linkage with the 3GPP system.

The widespread use of digital identities is currently expected around the world. A digital identity is a representation of a user's credentials and attributes in a digital format. Specifically, digital identities can be issued by public institutions, such as driver's licenses and employment certificates, or by educational institutions, such as student ID cards and diplomas. These formats and issuance/presentation protocols are being standardized by standards bodies such as W3C and IETF, and digital identity wallets are being developed to store these digital identities.

The technological impact of the spread of digital identity means that data formats related to users, which were previously defined in a closed manner for each industry, will be standardized, and systems can work together across sectors. In other words, user information issued by systems in different industries can be verified and used by communication systems, and vice versa.

Figure 9.X.1-1: Use of digital identity

The 3GPP defines a digital asset container component for the metaverse use case, which is defined as a function to manage virtual currencies, NFTs, and digital identities. However, digital identities are not limited to the metaverse use case but can be used in many other use cases.

### 9.X.2	Pre-conditions

Figure 9.X.2-1: Functions to manage of digital identity

Digital asset container (DAC) interfaces with the network functions of the 3GPP system and has functions to manage digital identity. DAC may be a part of 3GPP system.

Third party Digital identity system (TPDIS) interfaces with the digital asset container and has functions to manage digital identity.

Sub use case 1: The system changes network control based on information provided by the DAC

There will be cases where QoS control and billing control are based on digital identity.

Examples of specific scenarios are:

- Control charging based on digital identity presented

- When presented with a digital identity for membership in a particular VoD service, QoS is matched to video traffic for communications to that VoD service.

- If you present a digital identity issued by the administration, it provides communication to a slice that leads to administrative services and allows you to use the communication for free, albeit with low QoS communication, to operate the administrative service portal, for example.

Figure 9.X.2-2: Network control based on information provided by the DAC

Sub use case 2: TPDIS provides services based on information provided by the system to DAC.

It is conceivable that UE status, location information, and communication status could be issued as certificates.

An example of specific scenarios is:

- Based on location information provided by network operators, TPDIS may decide whether or not to approve credit card transactions.

Figure 9.X.2-3: Third party services based on information provided by the system to DAC

Sub use case 3: DAC and TPDIS synchronize digital identity

In a specific scenario, a user uses a digital identity wallet provided by TPDIS as a non-DAC digital identity wallet and synchronizes digital identities between the DAC and TPDIS.

Figure 9.X.2-4: Synchronization of digital identities

### 9.X.3	Service Flows

Sub use case 1: The system changes network control based on information provided by the DAC

- The user's terminal connects to the network.

- The system verifies the digital identity stored in the DAC or retrieves one from a TPDIS.

- Based on the verification result, the system applies relevant charging and QoS control settings.

- The system provides communication service in accordance with the applied settings.

Sub use case 2: TPDIS provides services based on information provided by the system to DAC.

- The user requests access to a TPDIS.

- The TPDIS prompts the user to present their digital identity.

- If an appropriate digital identity is not available in the DAC, the DAC retrieves it from the system.

- The digital identity is securely transmitted to the TPDIS.

- The TPDIS verifies the credential and grants access if authentication is successful.

Sub use case 3: DAC and TPDIS synchronize digital identity

- The user links their DAC with a TPDIS.

- The user selects which identities should be maintained in sync.

- The synchronized credentials are updated automatically across platforms.

### 9.X.4	Post-conditions

Coordination of DAC, TPDIS, and 3GPP system provides suitable services for subscribers based on their identities.

### 9.X.5	Existing features partly or fully covering the use case functionality

General requirements for digital asset management are defined in TS 22.156.

### 9.X.6	Potential New Requirements needed to support the use case

[PR x.1.6-1] Subject to operator policy and user consent, the 6G system shall enable charging and QoS control based on digital identities issued by a third party.

[PR x.1.6-2] Subject to operator policy and user consent, the 6G system shall support verification of digital identities issued by a third party.

[PR x.1.6-3] Subject to operator policy and user consent, the 6G system shall support retrieval of digital identities issued by a third party.

[PR x.1.6-4] Subject to operator policy and user consent, the 6G system shall support exposure of digital identities issued by the system to a third party.

[PR x.1.6-5] Subject to operator policy and user consent, the 6G system shall enable secure transmission of digital identities to a third party.

* * * End of the Change * * * *