3GPP TSG-SA WG1 Meeting #110 	S1-252319

Fukuoka, Japan, 19-23 May 2025	(revision of S1-25xxxx)

Source:	Nokia

New use case title:	New use case on powering network by mobile energy systems

Draft TS/TR:	3GPP TR 22.870 v.0.2.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	<name and email address>

Abstract: This use case proposes to enable mobile energy systems such as electric vehicles to power the 6G network on demand.

---------- Use Case template ----------

## 5.6.x	Use case on powering network by mobile energy systems

### 5.6.x.1	Description

In recent years, there is a trend for sustainable road transportation and increased carbon emission regulations, electric vehicles (EVs) become more and more popular. The EVs will become an immense storage device of energy in the future.

Along that trend, the energy grid is also undergoing a transition towards renewable energy production and cellular energy networks, which are much more fluctuating and dynamic in terms of energy demand/supply. Under such conditions, the energy market may change to “on-demand-behaviour” (e.g., customers may (re)charge their EVs during times of excess grid energy and discharge the stored energy in the battery of the EV when there are power shortages in the grid). Such behaviour may be further applied to any Mobile Energy Storage (MES) system, e.g., objects equipped with energy storage and bi-directional charging capabilities which can (autonomously) move in 3-dimension (3D) space, such as bikes, scooters, cars, trucks, buses, planes, UAVs, etc. To achieve such “on-demand-behaviour”, information exchange between the MES and a mobile network is required to allow the network to be charged via a MES, (e.g., for business or emergency situations) or in some cases to allow the MES to be charged by network (or another MES).

In addition, as the stored energy in MES as well as the energy supply and demand in the electrical grid are dynamically changing, a standardized information exchange at network-level (avoiding proprietary OTT solutions) is necessary. For example, the information exchanged between the network and UE (i.e., MES) on whether, where and when to either charge the MES or be charged by the MES may be critical especially in the scenarios of grid overload, grid outages or disaster relief. The exchanged information is associated with current and predicted energy demand and supply in a certain geographical area and the capability of acting as a supplier or a consumer of electricity.

Potential sustainability impacts of the use case

|  | (the UN SDGs/GDC matching goals of each aspect within 3GPP context) | Potential benefits of the use case (added value) | Potential areas of attention of the use case (risks to be mitigated) |
| --- | --- | --- | --- |
| Environmental sustainability aspects (UN SDGs 12, 13, 14, 15 and indirectly 6, 7 & 11. UN GDC “Develop principles for environmental sustainability of digital technologies”) | Energy resources (UN SDG 7, 11, 12) | - Helping reducing network energy demand and dependency to the grid - Helping reducing risks of energy shortage and/or disruptions | - Extra cost for running charging/discharging sockets |
|  | Material resources (UN SDG 11, 12) |  | - Extra resources for building charging/discharging sockets |
|  | Emissions (UN SDG 6, 7, 11, 12, 13, 14, 15) | - Increase the number of charging stations for MES and also let the MES owners get revenue by discharging their MES to network which can encourage more people to use MES and, therefore, help to reduce air pollution and carbon emission generated by non-electric vehicles - Charging the network with MES which are already charged by a green energy resource can reduce carbon emission. | - Charging the network with MES which are already charged by a non-green energy resource may increase carbon emission. |
| Socio-economic sustainability aspects (UN SDGs 2, 3, 4, 5, 8, 9, 10, 11, 16 & 17 and indirectly 12. UN GDC “Closing Digital Divides and Accelerating SDG Progress” & “Expanding Digital Economy Inclusion” & “Fostering an Inclusive, Safe Digital Space”) | Health (UN SDG 3) | - Increase the number of charging stations for MES and also let the MES owners to get revenue by discharging their MES to network which can encourage more people to use MES and, therefore, help to reduce air pollution which directly impacts on people health (e.g., negative impact on some people with specific diseases such as heart issue or the air pollution is an important cause of many diseases). |  |
|  | Trustworthiness (UN SDGs 11 and indirectly 3 & 17) | - It can increase operation resilience and service availability of the network in case of adversarial events such as attacking the grid network |  |
|  | Work & income (UN SDG 8 and indirectly 12) | - Gives this opportunity to the MES owners to get revenue by discharging their MES energy to the network. |  |
|  | Infrastructure (UN SDG 9) | - Lowers the pressure on the grid infrastructure by relying on batteries of MES to provide energy to the network |  |
|  | TCO reduction (UN SDGs 8, 9 and 12) | - The operator can lower its dependency to the grid and get charged possibly at lower cost by MES |  |

### 5.6.x.2	Pre-conditions

  - EV1 from brand X has its UE registered to Network A

  - EV2 from brand Y has its UE registered to Network B

  - EV1 and EV2 are configured to provide information to their networks about their capability and availability to recharge other entities. When registering, they have both indicated this capability.

  - Mobile network A is configured to inform (nearby) terminal devices about its capability to be charged (e.g., presence of electric sockets at some gNB locations) and its current or future availability of being charged or need to be charged (e.g. when/if its available energy is below a certain threshold). Network A is also configured to inform about its capability and/or availability of charging an MES (e.g., when/if its available energy is above a certain threshold)

  - Mobile network B is configured to inform (nearby) terminal devices (i.e., MES including EV 2) about its capability to be charged but not to charge an MES. Network B is also configured to inform about other energy storages’ current or future availability of being charged or need to be charged (e.g. public or third party assets/infrastructure and/or other MESs).

### 5.6.x.3	Service Flows

- When the stored energy of an offgrid-powered base station of Network B is under a threshold (e.g., the base station is undergoing unusual bad weather conditions over several days preventing to recharge its local battery storage via its solar panels) or when there is a grid issue for a grid-connected base station (e.g., grid outage or grid disconnection because of a disaster) in Network B, the base station sends an information to its served UEs (including EV2) and informs its need to receive energy from UEs (i.e., MES) to recharge its backup batteries in order to maintain its service. Other cells in the nearby area (including grid-powered) also send the same energy information request to their served UEs. The information sent by the network can contain the minimum amount of energy or power needed and rewarding information.

- When EV2 receives the information from Network B, since it has good battery status and no long trip planned, it sends back an indication that it is ready to supply energy to the network.

- When the network (e.g., here, Network B) receives the indications from UEs (e.g., here, EV2), it selects one or more UEs (e.g., here, EV2 is also selected) and sends a response to them.

- If a selected UE (e.g., here, EV2) accepts the condition (e.g., the discharge reward), it sends an acceptance to the network (e.g., here, Network B) and, then, UE’s energy will be discharged to the network B.

- In the meantime, EV1 is getting low on battery and Network A has some extra energy. While the brand X app can provide the location of the nearest charging station, EV1 also sends a request to Network A that it could need to be charged by the network.

- Since Network A has high charge status, it send its availability to EV1 to supply energy. If EV1 accepts the charging conditions offered by Network A (e.g., energy price), it will be charged by Network A.

### 5.6.x.4	Post-conditions

EV2 goes to the location indicated by Network B to charge the network.

EV1 goes to the location indicated by Network A and get recharged.

### 5.6.x.5	Existing features partly or fully covering the use case functionality

None.

### 5.6.x.6	Potential New Requirements needed to support the use case

[PR 5.6.x.6-001] Subject to operator policy, the 6G network shall be able to inform UEs about its availability and its need to receive energy at certain locations.

[PR 5.6.x.6-002] Subject to operator policy, the 6G network shall be able to inform UEs about its availability to provide energy at certain locations.

[PR 5.6.x.6-003] Subject to user consent, the 6G system shall enable UEs to provide information to the network about their availability to provide energy.