3GPP TSG SA WG 1 Meeting #110 	S1-252572

19-23 May 2025, Fukuoka, Japan	(revision of S1-252553,2135,2066)

Source:	Airbus, ESA, Fraunhofer IIS

pCR Title:	Pseudo-CR on “8.4	Use case on resilient positioning in satellite networks”

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	José A. del Peral-Rosado, jose_antonio.del_peral_rosado@airbus.com

Abstract: The Editor’s Note in clause 8.4.6 is proposed to be replaced by a performance requirement in compliance to regulatory requirements and operator policy.

1. Introduction

The use case on resilient positioning in satellite networks includes in clause 8.4.6 an Editor’s Note on the KPIs related to regulatory requirements and operator policy, which needs to be replaced by performance requirements.

2. Reason for Change

Requirements related to the determination of the UE location in TS 22.261 [14] clause 6.46.2 do not provide any performance requirements. Since these performance requirements may change between regions and operators, the proposed change explicitly associates the performance on the determination of the UE location (i.e., UE location accuracy, availability and latency) to be in compliance to the applicable regulatory requirements and operator policy.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.2.1.

* * * Change * * * *

### 8.4.6	Potential New Requirements needed to support the use case

[PR 8.4.6-1]	The 6G system with satellite access shall be able to provide the UE with positioning service with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS) with the following KPIs:

Table 8.4.6-1: Performance requirements for satellite-based positioning services

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| Airplane en-route | [50] m | [50] m | [99 %] | [1] s | Outdoor | Up to [1500] km/h | Airplane mounted |
| Airplane landing | [10] m | [10] m | [99 %] | [1] s | Outdoor | Up to [350] km/h | Airplane mounted |
| UAV positioning | [1] m | [1] m | [99 %] | [1] s | Outdoor | Up to [160] km/h | UAV mounted |
| NOTE 1: Multiple satellites can be used to support 3GPP positioning technologies. |  |  |  |  |  |  |  |

[PR 8.4.6-2]	Subject to regulatory requirements,  operator policy, and to user’s consent, the 6G system with satellite access shall be able to determine the UE location.