3GPP TSG SA WG 1 Meeting #110 	S1-252657

19-23 May 2025, Fukuoka, Japan	(revision of S1-252190)

Source:	CATT

pCR Title:	Move W.1 UC to Clause 9 Immersive Communication

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.9

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes to move the agreed use case W.1 to Clause 9 Immersive Communication.

1. Introduction

Use case of coordinating computing and communication for XR rendering was approved in SA1 #108 meeting and included in TR as W.1 before how to organize computing relevant cases was clear. According to the study progress, the computing relevant topics were all discussed in the associated service use cases, thus this contribution proposes to move W.1 UC to clause 9 immersive communication following the same principle.

2. Reason for Change

W.1 content is specific to XR service, so it’s proposed to move W.1 UC to clause 9 Immersive Communication.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.870 v0.2.1

* * * First Change * * * *

9.X	Use case on coordinating computing and communication for XR rendering

9.x.1	Description

The exploration of ultimate user experience in consumer products, and the digital and intelligent transformation in the industrial fields have promoted the wide use of computing-intensive applications such as AR/XR, cyber-physical systems, industry robots and etc. However, the balance between computing capabilities and the size and cost of the devices raises a big challenge for the deployment of entire applications. The ITU-R report [39] points out that several emerging technologies are being envisioned to address the challenges. One trend is to process data at the network edge close to the data source for real-time response, low data transport costs and energy efficiency by edge computing technologies, while another trend is to scale out device computing capability beyond its physical limitations by splitting computing workload over reachable computing resources. With the development of each technology trend, the independent control, management and orchestration of communication and the computing resources may cause significant negative impact on the performance of the end-to-end solution.

The 6G system is expected to change the situation by coordinating the communication services and computing resource utilization in the stage of system architecture design. In this way, the 6G network can make the ubiquitous single-point computing resource node connected and participate in the scheduling of various types of computing resources such as UE, MEC server, cloud server. For example, the 6G network can select appropriate computing resources based on service characteristics to achieve optimal resource usage. On the other hand, the data transmission for the computation can be more flexible and efficient with additional information (e.g. workload, available capability) about the selected computing resources.

The real-time rendering of 3D scenes in a large-scale XR application is a typical case of a computing-intensive application, which requires large amounts of calculations to handle the complicated model and texture mapping. Sometimes limited to the capability of hardware, it is impossible for a single device to render the scene individually, and the distribution of the computing workload to other computing resources can solve the issue. Sometimes the duplicated rendering of the same XR scene which is being requested by multiple users can be avoided if choose to execute the rendering in the cloud server. However, the XR application can have little information on the status of computing resources so that it is difficult for the planned data transmission and computation to work together efficiently.

This use case illustrates how the 6G system enables better user experience of split XR rendering via the coordination of computing resources and communication resources.

Figure 9. x.1-1: Coordinating Computing and Communication for XR rendering

9.x.2	Pre-conditions

NOTE:	The term “6G core network” used in this use case does not imply any architectural assumption, e.g. whether 6G core network is a new or evolved core network (compared to 5G).

Several computing resources (e.g. MEC servers, cloud computing centre) for XR services are deployed distributed in different locations and have been enrolled in 6G core network. MEC Servers A, B and C are provisioned as a part of rendering pipelines to execute render engine, engine adaption, rendering acceleration for the rendering task. The cloud computing centre is capable of all the functions for XR rendering. XR Application Platform provides the render services to various XR applications.

Task Management Server is responsible for task analysis, sub-task planning, graphic composition and etc.

The UE supporting the XR application is the subscriber of the 6G network. It has registered to the 6G network and was authorized for the computing coordination service.

9.x.3	Service Flows

1.	Regarding the XR application's need, the UE sends a request of XR rendering to the XR Application Platform.

2.	The XR application platform decomposes the UE's request into different tasks and dispatches the render task to the Task Management Server through the IP network.

3.	The Task Management Server analyses the render task and plans the computing task based on the capability of the computing resources. Then it sends the requests of computing coordination service and XR communication service to the 6G core network, including the information of computing coordination service QoS (e.g. requested computing capabilities, response time) and communication service QoS (e.g. latency).

4.	The 6G core network selects MEC Server B and cloud computing centre based on the requested QoS of the computing coordination service, and the status information of all enrolled computing resources, and then sends feedback to the Task Management Server about the coordination result (e.g. selected computing resources). Meanwhile, the 6G core network establishes the communication paths between the UE and MEC Server B and the cloud computing centre to transmit data for the rendering based on the requested QoS of communication service.

5.	The Task Management Server distributes the necessary data and associated information to all the selected computing resources via the IP network.

6.	MEC Server B and the cloud computing centre execute individual rendering sub-tasks, sync-up with the Task Management Server about the progress of its sub-tasks and sync-up the status information (e.g. computing workload, congestion status) with the 6G core network.

7.	The 6G core network detects the overload of MEC Server B, and informs the Task Management Server to replace MEC Server B with MEC Server C.

8.	The Task Management Server collects the rendering graphics from MEC Server B, MEC Server C and the cloud centre after the sub-tasks are finished and composes the XR image for the XR Application Platform. Then the XR Application Platform returns the rendered XR image to the UE.

9.x.4	Post-conditions

The render task is efficiently completed with the collaboration of several edge computing servers under the coordination of the 6G core network.

The rendered XR image is successfully provided to the UE as expected.

9.x.5	Existing features partly or fully covering the use case functionality

None.

9.x.6	Potential New Requirements needed to support the use case

[PR 9.x.6-1] The 6G network shall support mechanisms to acquire and maintain the information of trusted computing resources (i.e. edge server(s)) for coordinating the usage of computing resource on demand

NOTE: The information can be related to computing capabilities (e.g. xPUs, storage), service capabilities (e.g. supported applications, status), and network capabilities (e.g. allowed bandwidth).

[PR 9.x.6-2] The 6G network shall support mechanisms to collect status information (e.g. computing workload, congestion information, available capability, power consumption) of trusted computing resources (i.e. edge server(s) or cloud server(s)) on-demand or periodically.

[PR 9.x.6-3] The 6G network shall provide mechanisms to expose to an authorised 3rd party the information (e.g. computing capability, the location, allowed service types, status, power consumption, utilization) of computing resources (i.e. edge server(s) or cloud server(s)).

[PR 9.x.6-4] Subject to operator's policy, application needs or both, the 6G network shall support the selection of computing resource(s) (i.e. edge server(s) or cloud server(s)) based on e.g. requested computing capabilities, and support routing of data traffic between a UE and the selected computing resource(s) based on required communication service QoS.

* * * Next Change * * * *

W	Other Use Cases

Editor's Note: If there are any use cases that does not fall into any sections in this TR, these use cases could be placed in this clause, subject to further discussion. How this clause will be used is FFS.

* * * End of Changes * * * *