3GPP TSG SA WG 1 Meeting #110 	S1-252713

19-23 May 2025, Fukuoka, Japan	(revision of S1-252095 and S1-252096)

Source:	OTD_US

pCR Title:	Pseudo-CR on Additional Requirements for Roaming

Draft Spec:	3GPP TR 22.870v0.2.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Selvam Rengasami (selvam@trideaworks.com)

Abstract: This paper adds a requirement for a 6G system to enable a VPLMN to be able to identify: 1) a roamed-in user’s permanent subscription identifier and 2) a roamed-in user’s Home PLMN.

1. Introduction

In the case of roaming, there is a need to add a system requirement for a capability for a VPLMN to be able to:

- Identify the roamed in user’s subscription permament identifier, and

- Identify the HPLMN of a roamed-in user.

Such requirements are needed to meet regulatory requirements.

2. Reason for Change

Since the inception of Mobile Networks that supported roaming, the VPLMN has always been able to identify the roamed in user’s permanent subscription identifier as well as identify their HPLMN.

The permanent identifier was provided by the UE directly to the VPLMN (e.g., IMSI) which validated it with the HPLMN and provided service to the roamed in user. This capability has been supported in 2G, 3G, as well as 4G.

Although the capability for the VPLMN to identify the permanent subscription identifier and the HPLMN of the roamed in user was supported in those generations, no corresponding Stage 1 requirement was ever written down in any Stage 1 text.

Due to the lack of these Stage 1 requirements, in the initial Stage 2 work for 5G, an approach was taken to not follow the previous practice and only keep the permanent identifier in the HPLMN. However, this approach did not meet regulatory requirements that required the VPLMN to have access to the subscription permanent identifier as it had for the previous 3 generations. Once, it was clear that regulatory requirements needed to be supported by the system, the Stage 2 approach was modified to “de-conceal” the concealed subscription permanent identifier in the VPLMN. One of the reasons that the initial 5G approaches were considered is that there were no explicit Stage 1 requirements for this capability.

To ensure that regulatory requirements can be met, this contribution proposes to add explicit Stage 1 requirements so that the VPLMN continues to have access to both the permanent subscription identifier and identity of the HPLMN of the roamed in user.

One requirement is for the 6G system to enable a VPLMN to be able to identify a roamed in user’s permanent subscription identifier (e.g., SUPI in 5G) even when it is initially presented with a temporary or concealed identifer (e.g., SUCI in 5G). The other requirement is for the 6G system to enable a VPLMN to be able to identify the HPLMN of a roamed in user.

The following are existing requirements in Clause 5.1.2.1 of TS 22.261. This contribution proposes an additional requirement to address the regulatory requirements.

Clause 5.1.2.1	Interworking between 5G systems

The 5G system shall support a UE with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the UE's 5G Home Mobile Network.

The 5G system shall enable a Visited Mobile Network to provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

The 5G system shall enable a Visited Mobile Network to provide support for services provided in the home network as well as provide services in the visited network. Whether a service is provided in the visited network or in the home network is determined on a service by service basis.

The 5G system shall provide a mechanism for a network operator to limit access to its services for a roaming UE, (e.g. based on roaming agreement).

The 5G system shall provide a mechanism for a network operator to direct a UE onto a partnership network for routing all or some of the UE user plane and associated control plane traffic over the partnership network, subject to an agreement between the operators.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870v0.2.1.

* * * First Change * * * *

### 5.5.3	Identifying the HPLMN of a Roamed In User

#### 5.5.3.1	Description

Since the inception of Mobile Networks that supported roaming, the VPLMN has always been able to identify the roamed in user’s permanent subscription identifier as well as the their HPLMN (e.g., MNC code). The permanent subscription identifier was provided by the UE directly to the VPLMN, which validated it with the HPLMN and provided service to the roamed in user. The capability of the VPLMN identifying the permement subscription identifier of the roamed in user has been supported in 2G, 3G, as well as 4G. The capability of the VPLMN identifying the HPLMN has been supported in 2G, 3G, 4G, and 5G.

Although these capabilities were supported in those generations, no corresponding Stage 1 requirement was ever written down in any Stage 1 text. To ensure that regulatory requirements can be met, two explicit requirements need to be added to the Stage 1 requirements for 6G.

One requirement is for the 6G system to enable a VPLMN to be able to identify a roamed in user’s permanent subscription identifier (e.g., SUPI in 5G) even when it is initially presented with a temporary or concealed identifer (e.g., SUCI in 5G). The other requirement is for the 6G system to enable a VPLMN to be able to identify the HPLMN of a roamed in user.

#### 5.5.3.2	Potential New Requirements

[PR 5.5.3.2-001] The 6G system shall enable a Visited Mobile Network to be able to identify the roamed-in user’s 6G permanent subscription identifier including being able to map a home network issued temporary identifier to the home network assigned permanent subscription identifier. This shall include the ability to de-conceal a concealed permanent subscription identifier.

[PR 5.5.3.2-002] The 6G system shall enable a Visited Mobile Network to be able to identify the roamed-in user’s HPLMN. This shall include the ability to de-conceal a concealed HPLMN identifier.

* * * * End of Change * * * *