3GPP TSG SA WG 1 Meeting #110 	S1-252715

19-23 May 2025, Fukuoka, Japan	(revision of S1-252019)

Source:	6G Rapporteurs

pCR Title:	Pseudo-CR Clause 5.3.3 Revision (FBS)

Draft Spec:	3GPP TR 22.870v0.2.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Editorial revision to reformat clause into two sub-clauses

1. Introduction

Clause 5 includes a mix of formats.  This proposed revision reformats the information from a full use case format to a two-clause format (description, potential new requirements).

2. Reason for Change

Improved readability.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870v0.2.1.

* * * First Change * * * *

### 5.3.3	Use case on UE selecting a Base Station after assessing its legitimacy

#### 5.3.3.1	Description

The majority of the efforts in the mobile communication system security has been focused on delivering secure communication once connected, whereas security related to bootstrapping those connections are relatively less explored. A UE can interact with a Base Station without assessing its legitimacy, which opens way for diverse attacks. False Base Station (FBS) attacks are one such example and its potential to cause active and passive impacts are an alarming concern, worldwide. Addressing fundamental security issues is important and can entail a system level reappraisal of risk and expectations. This is most feasible to undertake at the beginning of a new mobile generation. Attacks mounted by a FBS is a major security flaw in the existing communication system. There is an important opportunity to address this concern in the next generation wireless systems. The sooner that security threats are addressed in the process of interaction between components of the system, the more risks can be reduced to the 3GPP system.

By increasing the level of security between the radio network entities and the UE during the initial network selection procedure, False Base Station attacks can be prevented at earlier stages of communication establishment. One such example is the introduction of an efficient Base Station authenticity verification mechanism such that the UE can verify the authenticity of the Base Station prior to starting the 6G registration procedure

#### 5.3.3.6	Potential New Requirements needed to support the use case

[PR 5.3.3.6-1]	Subject to operator policy and regulatory requirements, the 6G system shall support a means for a UE to be able to distinguish a False Base Station from an authentic Base Station.

* * * End of Change * * * *