3GPP TSG SA WG 1 Meeting #110 	S1-252855

19-23 May 2025, Fukuoka, Japan	(revision of S1-252744, 2179)

Source:	6G Rapporteurs

pCR Title:	Pseudo-CR on addressing intent definition and solving Editor’s Note

Draft Spec:	3GPP TR 22.780 v0.2.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: <provide a short description of the content>

1. Introduction

The meaning of intent, how does it relate to AI agent and what’s the difference compared to SA5 definition were discussed.

2. Reason for Change

Solving the existing Editor’s note in 6.5, and providing an aligned understanding for intent.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	QIAN LI et all: "6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies" in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

......

[x]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

* * * Next Change * * * *

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Al Agent: an automated intelligent entity capable of e.g interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, executing tasks (autonomously or in collaboration with other Al Agents) to achieve a specific goal.

Editor’s Note: This definition could be updated as the study goes on to align with the potential new understand of AI agent.

AI service: an 6G service (e.g., AI model inference) to support AI-related activities, which is provided by 6G core network to the 6G user, considering the required quality of the service (e.g., inference accuracy, latency).

NOTE 1:	The above term does not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN (compared to 5G).

Editor’s Note:	The definition of AI service is FFS.

Digital Twin: A real-time representation of physical assets in a digital world.

NOTE 2: This definition was taken from ITU-T Recommendation Y.3090 [113].

Intelligent Communication Assistant: The virtual intelligent communication assistant locates in operator network and interacts with the users through voice, video, text, gestures or other modalities. The assistant can be customized for each particular user by accessing user data stored in the network, with user’s consent. It can provide various communication services and support individual users based on user’s intention and requirement. One subscriber can have one or more Intelligent Communication Assistants.

intent: Expectations including requirements, goals and constraints without specifying how to achieve them. [x]

NOTE 3: Intent can be used for 6G services as well as OAM.

Editor’s note: NOTE 3 is FFS.

Editor’s note: this definition is FFS for further enhancement along the study goes on. If more detail regarding Intent is necessary to support the use cases in this TR it may be introduced in an Annex.

Network Digital Twin: virtual replica of (part of) a mobile network to emulate (or simulate) the behaviour of the actual network.

Editor’s Note: it is FFS to update this definition.

Personal Data: any information relating to a user or subscriber that can be used to, either directly or indirectly, identify that user or subscriber, or to distinguish that user or subscriber from others.

Sensing target density: total number of objects to be sensed per geographic area. It is a measure of how many objects the 3GPP system can detect, identify and/or track within a target sensing area.

* * * Next Change * * * *

6.5	Use Case on 6G AI Agent Collaboration with Third-Party AI using LLM

6.5.1 	Description

AI Agents play a crucial role in modern telecommunications by enabling intelligent automation, decision-making, and adaptive network management. These agents are software-driven entities that leverage artificial intelligence, including machine learning and natural language processing, to interact with users, applications, and network components. In a 6G environment, AI agents enhance network efficiency by dynamically optimizing resources, predicting network conditions, and facilitating providing seamless communication between services. By integrating LLMs, AI agents can understand complex requests, translate them into actionable insights, and orchestrate 3GPP services (e.g., communication service, sensing service, AI-related) network capabilities and functions autonomously, ultimately improving user experience when consuming the 3GPP services, operational efficiency, and service innovation. [80]

In this use case, a third-party application (e.g., a smart city traffic management system) AI Agent sends a text-based request or query to the 6G network. The request is processed by an AI agent in the 6G network that leverages LLMs and the network's advanced capabilities (e.g., sensing, real-time data processing, telemetry, analytics, and others) to provide a response or perform an action. This interaction mimics how users interact with chatbots like ChatGPT, but it is tailored for network-specific tasks and applications.

The 6G network AI agent acts as an intelligent intermediary, interpreting the text-based request, gathering necessary data, and returning a response or executing a task.

6.5.2 	Pre-conditions

The third-party application (e.g., smart city traffic management system) has access to 6G network’s AI agent services.

The 6G network has deployed LLM-based AI agents capable of understanding and processing natural language queries.

The 6G network has access to real-time data sources (e.g., traffic sensors, weather data, network performance metrics) to fulfil the requests.

The third-party application and the 6G AI agent have established secure communication channels (e.g., authenticated APIs or message brokers).

The 6G network supports edge computing to process requests locally and reduce latency.

6.5.3 	Service Flows

Operator R, which operates a 6G network, provides sensing-based services to authorized third parties, such as a smart city authority. Instead of deploying its own sensors (e.g., cameras and other monitoring equipment) to detect traffic conditions, the smart city can procure these services from Operator R.

Additionally, Operator R can enhance these sensing-based services by integrating advanced analytics, such as UE mobility patterns. This enriched information is then made accessible to authorized third parties through AI Agent(s), enabling optimized urban management and smarter decision-making.

The third-party AI agent (e.g., Smart city traffic controller) sends a request to the 6G network AI Agent. For example:

"Why is the traffic congested on Highway A, and what adjustments can be made to optimize traffic flow?"

The 6G network's AI agent receives the request and uses an LLM to interpret the query. The LLM breaks down the request into two parts:

Part 1: Retrieve the current traffic congestion level on Highway A.

Part 2: Provide recommendations for optimizing traffic flow.

The AI agent queries real-time data sources (e.g., sensing infrastructure, etc.) to gather the current traffic congestion level on Highway A.

The gathered data is used by the AI agent to infer and thus generate analytics insights to determine that Highway A is experiencing high congestion due to an accident. The data may also be used to update (improve) the LLM at the AI agent, e.g. self-learning capability of AI agent.

Based on the analysis, the AI agent generates traffic optimization recommendations, such as:

Increasing green light duration on alternate routes.

Reducing green light duration on Highway A to divert traffic.

Deploying dynamic signage to inform drivers of alternative routes.

These recommendations are sent to the third-party AI agent managing the smart city infrastructure.

The AI agent generates a response to the third-party AI agent:

"Highway A is currently experiencing high congestion due to an accident. Recommended actions include adjusting traffic light durations on alternate routes and deploying dynamic signage to inform drivers. Expected congestion reduction in 10 minutes."

The response is sent back to the third-party application, which displays it to the user (e.g., a traffic management controller or city official) or uses it for further decision-making. The final implementation of the recommendations remains under the control of the third-party AI agent or human operators.

6.5.4 	Post Conditions

The third-party application receives the response and takes appropriate action (e.g., notifying city officials or updating a public traffic dashboard).

The traffic light optimization reduces congestion on Highway A.

The 6G AI agent logs the request, response, along with any additional contextual meta data for future reference (e.g. memorization capability of AI agent) and continuous improvement of the agent’s underlying LLM.

6.5.5	Existing features partly or fully covering the use case functionality

None.

6.5.6	Potential new requirements needed to support the use case

[PR 6.5.6-1] Based on operator policy, the 6G network shall be able to support secure means to expose its services to the authorised third-party AI agent based on its intent.