3GPP TSG-SA WG1 Meeting #111	S1-253174

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	vivo

pCR Title:	New use case on Real-Time Video Super-Resolution Service

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.3	Artificial Intelligence

Document for:	Approval

Contact:	Hank YU, yuhang.txyjy@vivo.com

Yanchao Kang, kangyanchao@vivo.com

Pucong Wang, pucong.wang@vivo.com

Abstract: This paper introduces a use case in which the 6G network offers real-time video super-resolution services for live streams to UEs. After receiving a task to enhance the video resolution for a live stream, the 6G network can interpret and divide this task (i.e. an integrated 3GPP service) into several subtask (i.e. several 3GPP services like AI service and computing service), and then coordinates the communication nodes, AI nodes, and computing nodes to achieve this task. This approach, utilizing the computing resources inside the 6G network, not only aids in reducing end-to-end latency and jointly coordinating communication and computing resources, but also supports small and medium-sized developers/video providers in lowering costs and alleviating the workload associated with application development.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

(… text not shown for clarity …)

[257]	3GPP TS 28.538: “Management and orchestration; Edge Computing Management”.

[xx1]	Liu H, Ruan Z, Zhao P, et al. Video super-resolution based on deep learning: a comprehensive survey[J]. Artificial Intelligence Review, 2022, 55(8): 5981-6035.

[xx2]	K. C. K. Chan, X. Wang, K. Yu, C. Dong and C. C. Loy, "BasicVSR: The Search for Essential Components in Video Super-Resolution and Beyond," 2021 IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), Nashville, TN, USA, 2021, pp. 4945-4954, doi: 10.1109/CVPR46437.2021.00491.

* * * Next Change * * * * (all new text)

6.x	Use case on Real-Time Video Super-Resolution Service

6.x.1	Description

With the growing demand for immersive and high-quality video services (e.g. short videos, live streaming, XR content), end-users increasingly expect a consistently high-resolution visual experience, irrespective of device capabilities or network conditions. Video resolution has advanced from the Standard Definition (SD, such as 480p) to High Definition (HD, such as 720p), progressing to Full High Definition (FHD, such as 1080p), Ultra High Definition (UHD, such as 4k), and ultimately Full Ultra High Definition (FUHD, such as 8k). Better video resolution is synonymous with an enhanced user experience. However, limited uplink bandwidth, mobile device processing constraints, and battery limitations often result in video content (e.g., from low-end mobile devices or in remote scenarios) being transmitted at suboptimal resolutions (e.g., 480p or 720p). As a result, viewers are unable to enjoy high-quality live broadcasts and cannot clearly see video details, leading to a lack of engaging interactions and significantly diminishing the user experience.

Video Super-Resolution is an effective technology for enhancing video resolution. It processes video to convert low-resolution streams into high-resolution output while maintaining low latency and high frame rates [xx1]. This technology also aims to enhance details while minimizing blurring and artifacts. Figure 6.x.1-1 shows the comparison of super-resolution on an old video. The left side displays the frame before applying super-resolution, whereas the right side presents the frame afterward. The post-super-resolution frame reveals significantly clearer details and a more diverse content display.

Figure 6.x.1-1: Comparison of video super-resolution on old video

Both traditional algorithms and AI models can help to achieve video super-resolution. Neural network-based video super-resolution models (e.g., BasicVSR [xx2]) have achieved significant breakthroughs and have become mainstream. The effectiveness of video super-resolution can vary depending on the specific model used or the type of video content. For instance, an AI model optimized for sports-related live streaming might perform poorly when applied to virtual character live streaming. To ensure high-quality video for users, it is important to select the appropriate AI model/ algorithm by analyzing video content, possibly through AI-driven content detection.

High-quality videos (e.g., high resolution) are becoming increasingly popular. The user can select different clarity on their applications via the User-Interface (UI) to satisfy user experiences as shown in figure 6.x.1-2.

Figure 6.x.1-2: Resolution selection on APP via user interface

Video super-resolution technology can be carried out in the form of network-based video super-resolution or mobile-based video super-resolution. In practice, the computing resources deployed on the network side primarily play a proactive role by employing video super-resolution techniques to improve the quality of received video, i.e., network-based video super-resolution. In recent years, with advancements in user terminal computational power, especially in high-performance mobile phones, the consumer-level UE can also undertake video super-resolution with small-sized AI models, i.e., mobile-based video super-resolution.

Video super-resolution technology applies to a variety of practical use cases, such as local city live video streaming, video conferencing, real-time monitoring of high-definition video, real-time processing of drone aerial footage, cloud gaming, and video calls.

This use case specifically addresses scenarios where the 6G network offers real-time video super-resolution services for live streams to UEs. Here 6G network performs network-based video super-resolution by utilizing computing resources deployed in the 6G network. It offers several benefits to video service providers, UEs, and 6G operators:

1. Lower Costs for Video Service Providers. A 6G network can expose computing resources deployed in the 6G network to video providers to help perform video super-resolution. When applying network-based video super-resolution, especially to some small and medium-sized developers/video service providers, they may lack the capacity to deploy a large number of servers with GPUs/NPUs, as this would entail significant deployment costs and maintenance expenses. Using network computing resources can help these providers with video super-resolution at lower expenses.

2. Reduced UE power consumption. It enables video service providers to rely more on network-based video super-resolution, rather than mobile-based video super-resolution, when enhancing video resolution. This approach helps lower power consumption on terminal devices, reduces heat generation during video viewing, and imposes minimal requirements on terminal devices, such as computing power.

3. Reduced Application Development Workload. User terminals differ widely in form and operating system (e.g., Android, iOS, Linux), so video super-resolution on UEs would require adapting and developing multiple applications for invoking different super-resolution models. Consequently, employing unified network-side video super-resolution can reduce the workload involved in UE application development.

4. Minimized End-to-End latency. Traditionally, video super-resolution for live streams involves first transmitting the video data to the video service provider's servers, performing video super-resolution there, and then sending the high-resolution video back to the UE. However, these servers are often situated at a considerable distance from the UE. By utilizing the computing resources deployed on the 6G network to process these videos, user latency can be significantly reduced, enabling more real-time interaction on the UE side.

5. Enhanced Joint Coordination of Network Resources. Both communication control nodes, computing control nodes, and computing resources are deployed within the 6G network. 6G operators can utilize unified policy assurance and resource scheduling mechanisms to jointly manage communication and computing resources, instead of managing them separately. A globally optimized scheduling mechanism is clearly superior to two locally optimized ones, which helps 6G operators improve the utilization efficiency of communication and computing resources within the network.

6. New Service Opportunity. Beyond offering communication services, positioning services, and integrated sensing services, 6G operators can provide computing resources to applications on the UE, third parties, and even operator-owned video services to improve video resolution. 6G operators can also enhance high-resolution for the user when providing video call services (e.g., Video over 6G). These new resources and services introduce a new revenue stream.

6.x.2	Pre-conditions

The 6G network supports AI functions and deploys certain computing resources, enabling it to provide 6G computing service, AI service and communication service for video super-resolution.

Due to deployment cost constraints, the video service provider has decided to rent computing resources within the network to perform real-time video super-resolution, thereby offering users live streaming services with high video resolution.

The 6G operator or 3rd party video service provider can pre-configure and store the following information inside the 6G network to help perform video super-resolution: AI models/algorithms for video super-resolution, AI models/algorithms selection rules, video service provider information and so on. These AI models/algorithms and selection rules can also be further updated to improve the service experience.

Alice is a live-streaming host for outdoor events, and Bob watches Alice's stream on the application on his UE. Both of them are using the same video application. Bob, as a live streaming viewer, has selected a high-resolution preference (e.g., FHD) of the video via the user interface on his mobile terminal. Such user preference is delivered to the 6G network and 6G network will take user preference into account when improving video resolution.

6.x.3	Service Flows

Figure 6.x.3-1: Real-Time Video Super-Resolution Service provided by 6G Network

The service flow for the real-time video super-resolution service offered by 6G network is shown in Figure 6.x.3-1.

1.	The third-party video service providers or 6G operator can pre-configure specific AI model/algorithm selection rules and related AI models/algorithms tailored for different types of content, such as scenery, sports, or games. The same AI model/algorithm applied to different types of video content can produce varying effects. Besides, the 6G network can pre-train AI models for real-time video super-resolution and store them within the network.

2.	Alice is streaming video outdoors via a 6G network, but due to limitations with her user equipment and network uplink bandwidth, the video is captured in standard or suboptimal resolution (e.g., 540p).

3.	Alice's UE transmits the standard or suboptimal resolution live streams to the 6G network.

4.	When the video stream is received, the 6G network considers the user preferences provided by Bob, indicating a higher resolution than the current one, and therefore, the 6G network decides to apply video super-resolution. 6G network coordinates the AI nodes, computing nodes, and communication nodes to offer real-time video super-resolution services for this video stream.

The 6G network can determine network-based or mobile-based for the real-time video super-resolution process, and further select an appropriate AI model/algorithm considering following aspects:

a) Network aspect: considering different network downlink communication performance and network computing load, either network-based video super-resolution or mobile-based video super-resolution can be performed. For example, when communication links performance between the 6G network and Bob is optimal, the 6G network can handle real-time video super-resolution and deliver high-definition content directly. If communication links performance between the 6G network and Bob is suboptimal, low-resolution videos will be received from the network and maybe enhanced on the UE/client side utilizing the local computing resources;

b) Video processing aspect: a suitable AI model/algorithm should be selected for each specific video type, as the same AI model/algorithm has different effects on video content (e.g. scenery, sports, or games) regarding the colour, block size, and details of the video;

c) User aspect: for different user preferences (e.g. FHD, UHD) on video resolution, 6G network can accordingly choose different AI models or traditional algorithms to satisfy the user experience.

5.	The 6G network selects and applies an appropriate AI model/algorithm for the real-time video super-resolution service.

6.	The 6G network select the most suitable computing resources considering factors like computing load, node location and other potential information. Utilizing available computing resources, the network applies the AI model/algorithm to enhance the video resolution.

7.	The enhanced high-resolution live streams, such as 1080p, are transmitted to viewers, including Bob.

8	The video is decoded and presented to the user, Bob.

9	6G network monitors the performance of real-time video super-resolution and periodically sends reports of real-time video super-resolution outcomes to the manager. Such reports may encompass various metrics, including AI model inference accuracy, measured by calculating pixel differences between the upsampled video and the original high-definition video. They may also include Motion Estimation, which assesses whether the model effectively restores details in fast-moving scenes, as well as Quality of Experience (QoE) metrics. Such reports can be used by an operator or 3rd party video service provider to further update or retrain AI models, or update rules or algorithms in order to enhance the user experience for video services.

6.x.4	Post-conditions

Bob can enjoy high-resolution live streaming. The video service provider offers real-time video super-resolution services to users at a reasonable cost. The 6G operator can offer real-time high-resolution video to users when providing operator-managed video services.

6.x.5	Existing features partly or fully covering the use case functionality

None.

6.x.6	Potential New Requirements needed to support the use case

[PR 6.x.6-1] Subject to operator policy, the 6G network shall be able to manage and coordinate various network subtasks (e.g. AI model training/selection, algorithm selection, computing resource selection, communication performance monitoring) when providing 6G services (e.g., AI service, 6G computing service) to enhance user experience for applications.

NOTE:	Several subtasks (i.e. 3GPP services like communication service, AI service, or 6G computing service) are components essential for achieving a given task (i.e. integrated 3GPP service that combines such as AI service and 6G computing service).

[PR 6.x.6-2] Subject to operator policy, the 6G network shall be able to receive a task, interpret this task, and determines several subtasks to achieve this task (e.g., real-time video super-resolution).

[PR 6.x.6-3] Subject to operator policy, the 6G network shall be able to store and select AI/ML models received from the 3rd party or trained by the 6G network itself.

[PR 6.x.6-4] Subject to operator policy, the 6G network shall be able to support a mechanism to guarantee the user experience when providing 6G services (e.g., AI service, 6G computing service) to enhance user experience for applications.

[PR 6.x.6-5] Subject to operator policy, the 6G network shall be able to monitor the task performance (e.g., AI model inference accuracy, QoE of tasks) and report them to the 3rd party.

* * * End of Change * * * *