3GPP TSG SA WG 1 Meeting #111 	S1-253402

25-29 August 2025, Goteborg, Sweden	(revision of S1-253179r1)

Source:	NTT DOCOMO, NTT, Deutsche Telekom

pCR Title:	Pseudo-CR on updates to digital identity management for digital asset container (9.11)

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Kenta Yamauchi, kenta.yamauchi.xe@nttdocomo.com

Naoki Higo, naoki.higo@ntt.com

Abstract: <provide a short description of the content>

1. Introduction

pCR on updates to digital identity management for digital asset container (9.11).

2. Reason for Change

There are some issuses which need to be fixed in use case 9.11 as below.

- Missing references for Figure 9.11.1-1 and Figure 9.11.2-1

- Editor’s Note in PR 9.11.6-1

Also, there was a discussion at SA1#110 that this use case could be considered as one of security use cases in the TR, not one of immersive communication use cases.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP 3GPP TR 22.870 v0.3.1

Changes from S1-253179

- Add Deutsche Telekom as a cosigner

- Reword the requirement based on the offline discussion with companies

* * * First Change * * * *

## 5.5.X	Use case on digital identity management for digital asset container

### 5.5.X.1	Description

This use case focuses on the digital asset container's functionality related to digital identity and proposes to expand the use cases related to the linkage with the 3GPP system.

The widespread use of digital identities is currently expected around the world. A digital identity is a representation of a user's credentials and attributes in a digital format. Specifically, digital identities can be issued by public institutions, such as driver's licenses and employment certificates, or by educational institutions, such as student ID cards and diplomas. These formats and issuance/presentation protocols are being standardized by standards bodies such as W3C and IETF, and digital identity wallets are being developed to store these digital identities.

As depicted in Figure 5.5.X.1-1, the technological impact of the spread of digital identity means that data formats related to users, which were previously defined in a closed manner for each industry, will be standardized, and systems can work together across sectors. In other words, user information issued by systems in different industries can be verified and used by communication systems, and vice versa.

Figure 5.5.X.1-1: Use of digital identity

The 3GPP defines a digital asset container component for the metaverse use case, which is defined as a function to manage virtual currencies, NFTs, and digital identities. However, digital identities are not limited to the metaverse use case but can be used in many other use cases.

### 5.5.X.2	Pre-conditions

Figure 5.5.X.2-1: Functions to manage of digital identity

Figure 5.5.X.2-1 shows functionalities in this use case to manage user digital identities.

Digital asset container (DAC) interfaces with the network functions of the 3GPP system and has functions to manage digital identity. DAC is application hosted in Service Hosting Environment, which means it is under operator control.

Third party Digital identity system (TPDIS) interfaces with the digital asset container and has functions to manage digital identity. TPDIS is one type of third-party applications.

John has a UE with a subscription to MNO#A.

### 5.5.X.3	Service Flows

The following description illustrates a service flow for QoS control based on digital identities. While the example focuses on QoS control, the same procedure is also applicable to other network functions, such as charging control.

- John's UE registered to the network, and internet connection gets available.

- John has a contract for subscription-based streaming services, which are certified in the form of digital certificates, and enjoys subscription-based streaming services. The certificates are issued by subscription-based streaming services.

- John wants to be provided with communication quality (delay, bandwidth, etc.) suitable for subscription-based streaming services, and keeps the certificates issued by subscription-based streaming services in a DAC managed by MNO#A.

- Based on his consent, MNO#A uses the certificates in the DAC for service authorization combined with the subscription information stored in the network and authorizes changes the communication QoS for the subscription-based streaming services.

### 5.5.X.4	Post-conditions

John could get suitable communication quality for subscription-based streaming services based on the verification of the certificates by MNO#A.

### 5.5.X.5	Existing features partly or fully covering the use case functionality

General requirements for digital asset management are defined in TS 22.156 [28].

User identity features are defined in TS 22.101 [58].

### 5.5.X.6	Potential New Requirements needed to support the use case

[PR 5.5.X.6-1] Subject to operator policy and user consent, the 6G system shall support means to provide users with differentiation of QoS and charging based on users’ digital identity information issued by a third party and users’ subscription information.

* * * End of Changes * * * *