3GPP TSG-SA WG1 Meeting #111 	S1-253178r1

Gothenburg, Sweden, 25-29 August 2025

Title:	Use case on  advancement of network exposure service

Agenda Item:	8.1.2

Source:	CEWIT, Tejas Network

Contact:	Ankita Lachhwani

ankita.lachhwani@cewit.org.in

Abstract: This contribution proposes enhancement to the RAN exposure service which is currently supported in 5G

* * * First Change * * * *

## 5.7 	6G enhancements of legacy/existing services and capabilities

Editor’s Note:  Text to be provided.  This clause to address potential (new) requirements for new (6G) aspects of existing (5G and earlier) services or capabilities.

### 5.7.X	Use case on advancement of Network Exposure service

#### 5..7.X.1	Description

The exposure of network information has been a cornerstone for enabling applications to fetch data from the 5G system. This not only allows operators to monetize network insights but also empowers applications to take proactive actions, thereby improving service availability and quality.

In 5G, the core network shall expose information related to RAN such as Cell ID and helps applications make informed decisions. However, for 6G, this level of exposure will not be sufficient. New types of information beyond the current set will need to be made available from the network.

Non-sensitive context data like the ability to meet Quality of Service (QoS) requirements.

For instance, in 5GS, when the IMS (IP Multimedia Subsystem) initiates an audio or video call, it requests QoS resource allocation to the Core Network. This involves a series of service requests across multiple 5G entities, including the RAN, UE and user plane functionality of core network. If the any entity  rejects the QoS setup based on its prevailing network resource capabilities, all previously executed steps must be rolled back resulting in inefficiency in the network.

To address such scenarios in 6G, it is crucial that the IMS or any third-party application is made aware in advance of the likelihood of a successful QoS setup at 6G network. This requires the 6G network to expose its capability or readiness to fulfil the QoS request. Such foresight would significantly reduce unnecessary signalling and improve response times.

Moreover, in certain scenarios, applications may tune their services to specific UEs based on current network conditions of the UEs exposed by 6G.

Other scenarios could be

- Predictive Network State Sharing

  - Provide predictive insights to third parties based on AI/ML models, not just real-time changes.

  - Example: Forecasted RAN signal degradation, cell congestion, or expected handovers.

  - Advantages: Enables third parties to proactively reconfigure services before degradation occurs.

- Semantic Information Sharing

  - Move beyond raw data.

  - Example:

    - “Network conditions suitable for 8K video streaming.”

    - “Cell coverage expected to degrade in 20s due to weather.”

    - “Network slice operating within guaranteed QoS parameters”

    - Access selection based on prevailing radio conditions

  - Advantages: Reduces complexity for third parties and enables smarter application behaviour.

Such exposure capabilities will empower the 6G system to support specific customizations, optimized resource usage and support emerging use cases like XR, IoT, and NTN.

By enabling such exposure in 6G system to authorized third-party applications, we move toward a new era of proactive and adaptive service delivery, enhancing user experience and network efficiency. This opens the door for a new class of intelligent, network-context-aware applications.

### 5.7.X.2	Existing features partly or fully covering the use case functionality

Table 5.7.X.2-1: Gap analysis for this use case

| Specifications and clauses | Examples of existing Requirements | Gap Analysis |
| --- | --- | --- |
| 22.261 Clause 6.10.2 | The 5G system shall provide a means by which an MNO can inform authorised 3rd parties of changes in the - RAT type that is serving a UE. - cell ID. - RAN quality of signal information. - assigned frequency band. This information listed above shall be provided with a suitable frequency via OAM and/or 5G core network. NOTE 6: The information aids the third party user to take proactive actions so that it can achieve high service availability in delivery of its services. | These parameters exposed are limited and do not cater to extensive needs of the applications The predictive and semantic information was not present along with new information related to 6G requires to be address. |

#### 5.7.X.3	Potential New Requirements needed to support the use case

[PR 5.7.X.3-1] Subject to regulation, operator(s) policy and user consent, the 6G system shall provide means by which an MNO can expose predictive network state and semantic information of the network to an authorized third party application.