3GPP TSG-SA WG1 Meeting #111 	S1-253276

Goteborg, Sweden, 25-29 August 2025

Title:	Update Use case 5.6.4 on disaster risk-based network resilience to solve ENs

Agenda Item:	8.1.2

Source:	Nokia

Contact:	Laurent-Walter Goix

Abstract: This contribution proposes to update the use case and its requirements to solve the related ENs.

R1

- Replace “immune” by “not impacted”

- Added example of live video streaming (uplink) in the service flows

R2

- Removed details on providing assistance information to UEs [Samsung]

- Clarified gap with PWS [Futurewei]

- About PR2 removal [Futurewei, Huawei]: this PR solves the 2nd EN to address the case across multiple networks during disasters, while PR1 addresses the case within a single network

---------- Use Case template ----------

### 5.6.4	Use case on disaster risk-based network resilience

### 5.6.4.1	Description

Handling disaster situations in mobile networks is a critical aspect of ensuring robust and resilient communication systems. 3GPP standards incorporate mechanisms for rapid network recovery and continuity during emergency situations, such as natural disasters or large-scale outages. These mechanisms include the implementation of emergency communication procedures, prioritization of critical services, and the deployment of mobile base stations and temporary network infrastructure to restore connectivity swiftly. Additionally, the interoperability and coordination among different network operators and emergency services can facilitate seamless communication and efficient disaster response. By adhering to these standards, mobile networks can maintain essential communication services, support rescue operations, and provide timely information to affected populations, thereby enhancing overall disaster resilience and response capabilities.

Some operators have developed risk scores that have been integrated into existing mobility planning tools. Operators use “risk scores” in their site selection process. The risk score includes for example drought, wildfire, inland flooding, coastal flooding, and wind data.

The occurrence of certain disaster conditions such as tornadoes, wildfires or floodings can be predicted by the advanced weather forecasting etc. or generally be known in advance in a given geographical area. In today’s networks, emergency procedures exist to preconfigure some list of networks available in case of disaster (e.g. based on MINT) and to notify UEs about upcoming or ongoing disaster (e.g. Public Warning System (PWS) [62] / Earthquake and Tsunami Warning System (ETWS) messaging).

However, various elements/sites of the network may be sensitive/exposed to different risks e.g. depending on their physical location (e.g. roof-mounted base stations may be immune to floodings, indoor base stations immune to tornadoes etc). In case of an (upcoming) occurrence of a specific disaster, it can be anticipated upfront that part of the network may be affected in an area, while other parts of the network would not.

Potential sustainability impacts of the use case

Table 5.6.4.1-1: Potential sustainability impacts

|  | (the UN SDGs/GDC matching goals of each aspect within 3GPP context) | Potential benefits of the use case (added value) | Potential areas of attention of the use case (risks to be mitigated) |
| --- | --- | --- | --- |
| Environmental sustainability aspects (UN SDGs 12, 13, 14, 15 and indirectly 6, 7 & 11. UN GDC “Develop principles for environmental sustainability of digital technologies”) | Energy resources (UN SDG 7, 11, 12) | - Optimization of network resources based on the type of resources, with increased energy efficiency |  |
| Socio-economic sustainability aspects (UN SDGs 2, 3, 4, 5, 8, 9, 10, 11, 16 & 17 and indirectly 12. UN GDC “Closing Digital Divides and Accelerating SDG Progress” & “Expanding Digital Economy Inclusion” & “Fostering an Inclusive, Safe Digital Space”) | Health (UN SDG 3) | - Increased availability of the network to provide service to populations affected by the disaster |  |
|  | Inclusion & Equality (UN SDGs 11, 10, 4, 5 and indirectly 3, 16 & 17) | - Increased availability of the network to provide service to populations affected by the disaster |  |
|  | Trustworthiness (UN SDGs 11 and indirectly 3 & 17) | - Increase availability & resilience of the system based on the specific impact on the network depending on the type of disaster |  |
|  | Infrastructure (UN SDG 9) | - Increased availability & resilience of the network to provide service to infrastructures affected by the disaster |  |
|  | TCO reduction (UN SDGs 8, 9 and 12) | - Reduced service downtime and unwanted traffic on impacted network nodes reducing in further maintenance or costs to handle the disaster in the network |  |

### 5.6.4.2	Pre-conditions

The network (e.g. a 6G base station) is subscribed to obtain information about a disaster event or disaster prediction from an external source such as advanced weather forecasting service or public safety service.

### 5.6.4.3	Service Flows

- The network gets information about a possible disaster in an area in the near future (e.g. 1 day in advance) and starts to broadcast announcement to UEs in that area, which indicates the likelihood of the disaster situation in advance (e.g. in day(s)) with a prediction of the impact to (some of) the network/cells based on a specific type of disaster.

- The UE stores the information and performs internal mapping of the given disaster type with some action it may perform if the disaster happens. This can be based on predefined rules provided by the operator (e.g. indicating to move to some nearby cells which should not be impacted by the disaster).

- As the probability of occurring the disaster increases over time, the network broadcasts an indication of the imminent disaster situation with the anticipated risk-based network assistance information such as specific cells-at-risk or preferred (i.e. likely not impacted) cells, disaster risk score/level, anticipated time of the disaster, pre-configured UE actions etc.

- For certain type of disasters where serving network of UE is vulnerable to those disaster types but the disaster area is served by other (at least partially available) network(s), assistance/configuration information related to other network(s) is provided.

- The UE determines whether and what UE initiated action (e.g. conditional handover, cell reselection) it needs to perform based on this indication and the previously received information provided by the network. UE may decide to perform no action if it considers it as the best option. For example, if the UE is currently in a video-streaming live session to broadcast the impact of the disaster, it may anticipate a handover to a target cell of another network which will not be impacted in order not to affect the streaming (which would otherwise take several tens of seconds to reconnect to the remaining available network after its serving network went down)

NOTE: It could be possible that both information are sent at the same time (e.g., in case of short notice).

### 5.6.4.4	Post-conditions

The network can increase its stability by properly managing the unavailability of part of its network depending on the type of disaster, thus instructing UEs in advance on how to handle a specific situation.

UEs can better keep connectivity and service continuity even in case of disaster by anticipating the actions to remain connected to the parts of the network still available during the disaster.

### 5.6.4.5	Existing features partly or fully covering the use case functionality

Disaster roaming and MINT techniques allow cells to provide/broadcast a configuration for disaster roaming. The UE is then configured with the list of PLMN(s) to be used in the event of a disaster. In addition, UAC for disaster roaming UEs is also defined: a network should be able to bar UEs doing disaster roaming more aggressively than non-disaster roaming UEs. Specifically, TS 22.261 [14] clause 6.31 defines the following requirements:

6.31.2.2	Disaster Condition

The 3GPP system shall enable UEs to obtain information that a Disaster Condition applies to a particular PLMN or PLMNs.

NOTE:	If a UE has no coverage of its HPLMN, then obtains information that a Disaster Condition applies to the UE's HPLMN, the UE can register with a PLMN offering Disaster Roaming service.

The 3GPP system shall support means for a PLMN operator to be aware of the area where Disaster Condition applies.

The 3GPP system shall be able to support provision of service to Disaster Inbound Roamer only within the specific region where Disaster Condition applies.

The 3GPP system shall be able to provide efficient means for a network to inform Disaster Inbound roamers that a Disaster Condition is no longer applicable.

Subject to regulatory requirements or operator’s policy, the 3GPP system shall support a PLMN operator to be made aware of the failure or recovery of other PLMN(s) in the same country when the Disaster Condition is applies, or when the Disaster Condition is not applicable.

6.31.2.3	Disaster Roaming

The 3GPP system shall be able to provide means to enable a UE to access PLMNs in a forbidden PLMN list if a Disaster condition applies and no other PLMN is available except for PLMNs in the forbidden PLMN list.

The 3GPP system shall provide means to enable that a Disaster Condition applies to UEs of a specific PLMN.

The 3GPP system shall be able to provide a resource efficient means for a PLMN to indicate to potential Disaster Inbound Roamers whether they can access the PLMN or not.

Disaster Inbound Roamers shall perform network reselection when a Disaster Condition has ended.

The 3GPP system shall minimize congestion caused by Disaster Roaming.

The 5G system and EPS shall support a mechanism for the HPLMN to control whether a UE, with HPLMN subscription, should apply Disaster Roaming when a Disaster Condition arises (in the HPLMN or a VPLMN).

3GPP system shall be able to collect charging information for a Disaster Inbound Roamer with information about the applied disaster condition.

In addition, TS 22.261 [14] has defined requirements for network sharing-based disaster condition handling (also known as “disaster sharing”) via the following requirements in clause 6.21:

Subject to regulatory requirements or operator policy, the 5G network shall support a PLMN operator to be made aware of the failure or recovery of NG-RAN and/or core network in other PLMN(s) in the same area when the Disaster Condition applies, or when the Disaster Condition is not applicable anymore.

Subject to regulatory requirements or operator policy, the 5G network shall support a PLMN operator to be made aware of the availability of other PLMN(s) as Hosting NG-RAN Operator(s) via Indirect Network Sharing in the same area when the Disaster Condition applies.

Subject to the agreement between the hosting and participating operator, the 5G system shall support a means to enable a UE of the Participating NG-RAN Operator to:

-	access their subscribed PLMN services when accessing a Shared NG-RAN, and/or,

-	obtain its subscribed services, including Hosted Services, of participating operator via a Shared NG-RAN.

NOTE 4: The above requirement is applicable to Disaster Condition via a Shared NG-RAN.

Based on operator policy, the 5G network shall minimize network congestion caused by Indirect Network Sharing when a Disaster Condition applies.

NOTE 5: Population density in the different areas where Disaster Condition apply needs to be considered.

The 5G network shall be able to enable Indirect Network Sharing only when the Disaster Condition applies in a specific area and disable it when no longer applicable.

NOTE 7: It is assumed operators can have sharing agreement for Disaster Condition in the area.

NOTE 8: It is assumed that during a Disaster Condition, previous network communication is temporarily disabled.

The 5G network shall be able to provide a means for a UE to return to the PLMN used prior to Indirect Network Sharing, when a Disaster Condition is no longer applicable.

The 5G network shall be able to collect charging information for a UE accessing a Shared NG-RAN using Indirect Network Sharing in Disaster Condition.

Furthermore, PWS [62] messages are designed to deliver emergency alerts to users in a specific geographic area. PWS includes systems like ETWS and Commercial Mobile Alert System (CMAS). PWS messages contain essential information about the emergency, such as the type of event, the affected area, instructions for safety, and any other relevant details. The messages are typically short and concise to ensure quick delivery and easy comprehension. They may include text, symbols, or other formats depending on the capabilities of the network and devices. PWS messages are broadcasted using the Cell Broadcast Service (CBS), which allows messages to be sent to all devices within a specific geographic area simultaneously. This ensures that the message reaches a large number of users quickly.

ETWS is specifically designed for rapid dissemination of warnings related to earthquakes and tsunamis. CMAS, also known as Wireless Emergency Alerts (WEA) in the United States, is a broader system that covers various types of emergency alerts, including natural disasters, severe weather, AMBER alerts, and other public safety messages.

While PWS addresses users, MINT addresses UEs. However, MINT does not provide the granularity needed to identify the parts of the network(s) (e.g. cells) which can or cannot be affected depending on the type of disaster.

### 5.6.4.6	Potential New Requirements needed to support the use case

[PR 5.6.4.6-1] Subject to operator policy and regulatory requirements, the 6G system shall be able to minimize service interruption or degradation during disasters taking into account the affected / not affected cells impacted by a specific disaster (e.g. in locations more sensitive to windstorms, flooding areas etc).

[PR 5.6.4.6-2] Subject to operators’ policies, agreements and regulatory requirements, the 6G system shall be able to minimize service interruption or degradation during disasters taking into account the affected / not affected cells of multiple networks serving the disaster area impacted by a specific disaster.

NOTE:	It is assumed that under disaster conditions, the remaining available parts of the networks in the disaster area can serve the UEs of other networks.