3GPP TSG SA WG 1 Meeting #111	S1-253334r2

25-29 August 2025, Goteborg, Sweden	(revision of S1-253161)

Source:	Qualcomm Technologies

pCR Title:	Use Case on UAV Detection, Classification and Counting

Draft Spec:	3GPP TR 22. 870 v0.3.1

Agenda item:	8.1.4

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: This contribution introduces a use case on using 6G sensing service for UAV detection, classification and counting.

1. Introduction

Current 3GPP specifications do not support UAV counting based on micro-Doppler analysis in multipath-rich environments. While existing ISAC use cases address general object detection and tracking, they do not address the unique challenges of distinguishing multiple airborne targets in cluttered RF environments.

Micro-Doppler refers to time-varying Doppler shifts caused by the micro-motions of a target. Micro-Doppler analysis can be employed to identify characteristic patterns of different target micro-motions, such as the rotation of UAV propellers. In addition to exhibiting distinct micro-Doppler features compared to birds, UAVs of varying sizes and types also present subtle differences in their micro-Doppler signatures. Even for UAVs of the same model, variations in motion patterns and certain random factors can lead to differences in their micro-Doppler signatures. Micro-Doppler analysis is widely adopted in radio frequency (RF) sensing/radar research.

2. Reason for Change

This proposal introduces a novel sensing capability that leverages 6G systems to:

- Enhance UAV detection reliability in urban and indoor environments.

- Support identification of UAV-specific characteristics (e.g., rotor count) to assist in classification tasks (e.g., quadcopter vs. hexacopter).

- Enable accurate UAV counting by distinguishing true targets from multipath artifacts.

- Address operational needs in scenarios such as airports, city centres, and copyright-sensitive events (e.g., concerts, sports games) with a scalable and cost-effective solution.

3. Conclusions

Micro-Doppler-based UAV counting represents a significant enhancement to the ISAC capabilities of 6G systems. By exploiting the unique motion signatures of UAVs, this method enables accurate and robust sensing in environments where traditional techniques fail. It supports a wide range of applications, including airspace management, public safety, and event security, and aligns with the goals of 3GPP to support advanced sensing services.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 22.870 v031

* * * First Change * * * *

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	QIAN LI et all: "6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies" in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022. (6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies | IEEE Journals & Magazine | IEEE Xplore).

### ….

[x]   	Crona, Michael, et al. "Low-Speed Propeller for UAV Applications, From Design to Experimental Evaluation." ICAS 2024 Congress Proceedings, Chalmers University of Technology, 2024, https://www.icas.org/icas_archive/icas2024/data/papers/icas2024_1157_paper.pdf.

* * * Second Change * * * *

### 7.x.1	Use Case on UAV Detection, Classification and Counting

### 7.x.1	Description

In increasingly complex sensing environments—such as urban centers, airports, and large-scale public events—the accurate detection, classification, and counting of UAVs (Unmanned Aerial Vehicles) is critical for airspace safety, security, and regulatory compliance. Moreover, in many real-world applications, UAV sensing involves the simultaneous detection and tracking of multiple UAVs rather than a single target. This is especially common in scenarios where UAV usage is dense and diverse, such as logistics, infrastructure inspection, or public event monitoring. Therefore, the ability to accurately count and distinguish multiple UAVs becomes a fundamental requirement.

This capability is particularly relevant considering the growing interest in low-altitude airspace applications, where UAVs are expected to play a central role in logistics, inspection, emergency response, and aerial mobility. In some regions, such as China, the concept of a “low-altitude economy” is gaining policy-level support and industrial momentum, further highlighting the importance of robust UAV sensing and counting technologies to enable safe and efficient airspace operations.

Traditional RF sensing methods often assume a dominant direct or line-of-sight (LoS) path between the UAV and the sensing infrastructure. However, in real-world deployments—especially in urban environments—high-rise buildings introduce significant multipath effects by reflecting sensing signals. These multipath reflections lead to two major challenges:

- False Alarms: Reflected paths may appear as additional targets, causing over-counting if detection is based solely on signal strength thresholds.

- Localization Errors: Secondary reflections may be misinterpreted as direct paths, resulting in large position estimation errors.

Initially, UAVs can be distinguished from environmental clutter and other aerial targets such as birds by analysing their unique micro-Doppler signatures. To further address the challenges posed by multipath environments, this use case proposes leveraging micro-Doppler spectrograms extracted from 6G ISAC (Integrated Sensing and Communication) signals. Micro-Doppler signatures capture the fine-grained, periodic motion characteristics of UAVs (e.g., rotor blade rotations), which remain consistent across different multipath reflections of the same UAV. These signatures are largely invariant to propagation distance, reflection angle, or the number of bounces.

The distinct micro-Doppler signatures can be used to:

- Detect UAVs among various aerial objects and environmental clutter (e.g., distinguish UAVs from birds).

- Differentiate UAVs not only across different types (e.g., quadcopter, hexacopter) but also among individual UAVs of the same type.

By identifying and clustering paths with consistent micro-Doppler signatures, the system can:

- Detect and classify UAVs among multiple detected objects.

- Associate multiple reflections with a single UAV in multi-target scenarios.

- Reduce false alarms and improve localization accuracy.

- Accurately count the number of UAVs in a given airspace.

This capability is enabled by 6G RAN entities and UEs (e.g., smartphones, IoT devices, RSUs) acting as distributed sensors, combining their observations through data fusion to enhance robustness and coverage. The use case is particularly relevant for low-altitude airspace applications, where UAVs are expected to play a central role in logistics, inspection, emergency response, and aerial mobility. In some regions, such as China, the concept of a “low-altitude economy” is gaining policy-level support and industrial momentum, further highlighting the importance of robust UAV detection, classification and counting technologies. Upon request, the UAV detection, classification and counting sensing results can be provided by the 6G mobile network operator to trusted third-party applications, such as UAV service operators, regulatory agencies, Unmanned Aerial System Traffic Management (UTM) systems, and the UAV itself.

Figure 7.x.1-1: UAV detection, classification and counting by 6G system

### 7.x.2	Pre-conditions

In this use case, a USS (Unmanned Aerial System Service Supplier)/UTM (Unmanned Aircraft System Traffic Management) provides UAV services, such as package delivery, aerial photography, flight management services, within an area covered by a 6G network. These services may lead to a high-density UAV scenario in certain areas due to busy UAV traffic.

Network Operator NN provides 6G sensing service for UAV supervision service, including illegal UAV intrusion detection, UAV flight trajectory tracing, and UAV collision prediction etc. NN can make use of sensing entities (e.g. network nodes and UEs) to sense the airspace within their coverage area and report the sensing results (including detected and classified UAV) to the USS/UTM.

Network Operator NN provides the sensing result that may comprise multiple objects of multiple types (e.g., UAVs, birds, other aerial targets, environmental clutter). The system is required to detect and distinguish UAVs among detected objects even in challenge wireless conditions. The results may also include UAV classification and counting of each class to enable more fine-grained sensing.

The Company MM uses the USS/UTM to supervise the low-altitude UAVs and manage potential illegal intrusion into the restricted areas. MM has proved its restricted area information to the USS/UTM. The USS/UTM uses 6G sensing service provided by the 6G network operator NN for UAV supervision.

The USS/UTM provides specific details to the 6G network operator NN, including the characteristics of the UAV that will be detected. This information includes types (e.g., quadcopter, hexacopter) and sizes of target UAV. This information may also include the characteristics of the environment clutter and other aerial objects.

The 6G Network Operator NN provides the sensing service by  collecting, processing and storing  sensing data with the network providing sensing results to the USS/UTM.

### 7.x.3	Service Flows

Based on the request from USS/UTM, the 6G network operator NN activates the UAV sensing service in the designated airspace, which includes multiple UAVs, other aerial objects (e.g., birds), and various buildings and structures. These objects and structures contribute significant environmental clutter that can make sensing challenging.

Once the sensing service is activated, RAN entities and UEs within the operator NN network are triggered to perform RF sensing, transmitting and/or receiving sensing signals that are reflected from UAVs, birds, and environmental clutter.

The received sensing signals are processed to extract sensing data depicting micro-Doppler spectrograms. Using the micro-Doppler signatures of UAVs and birds—which are typically unique and remain consistent even across multipath reflections—the network is able to distinguish UAVs from birds and clutter.

With further processing, the network is able to:

- Classify the detected UAVs by type (e.g., quadcopter, hexacopter) and, where possible, distinguish individual UAVs of the same type.

- Count the number of UAVs in the area, including type-level (e.g., number of propellers) counting, and report the results to authorized stakeholders (e.g., airspace managers, event organizers).

The sensing results (e.g., different types of UAVs and the count of UAVs in the area, including type-level counting) and contextual information (e.g., time and locations of UAVs) are exposed to the authorized USS/UTM.

### 7.x.4	Post-conditions

The UAVs of interest in the designated airspace is accurately detected, classified, counted, and exposed to the trusted third parties.

### 7.x.5	Existing features partly or fully covering the use cast functionality

TR 22.837 [9] and TS 22.137 [6] present use cases and requirements for 5G system integration of sensing and communication, including detection and tracking of UAVs and other objects:

- The 5G system shall be able to provide sensing service to detect, and/or track one or more objects (e.g., UAVs, birds) and the environment around the object(s).

5G wireless sensing requirements address general object detection and tracking, but do not explicitly support, identification of UAV-specific characteristics and counting. The  use case, potential requirements and KPIs presented here are expected to facilitiate robust discrimination between UAVs and other aerial objects (e.g., birds), identification of characteristics and counting in many scenarios, especially, under challening wireless conditions.

### 7.6.6	Potential New Requirements needed to support the use case

[PR 7.x.6-1] Subject to operator policy, the 6G network shall be able to provide sensing service to detect, classify and count one or more sensing targets (e.g. detect UAVs, distinguish UAVs from birds, identify  specific UAV characteristics.).

NOTE: Classification refers to the identification of specific characteristics of the detected target objects and grouping together of the detected target objects with similar characteristics. For example, once a UAV is detected, the sensing service can be used to determine the type of UAV it is (e.g. if a detected UAV has 3 or 4 propellers) and then group together UAVs with similar characteristics.

.

Table 7.x.6-1: Performance requirements for UAV Detection, Classification and Counting

| Scenario | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Sensing Target Density | Max sensing service latency [ms] | Refreshing rate (Hz) | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s] |  |  |  |  |  |  |
| UAV detection, classification and counting | 95 | 1 Note 1 | 1 Note 1 | 1 Note 1 | 1 Note 1 | [0.3 – 1] Note 2 | 1 Note 1 | N/A Note 1 | ≤1000 Note 1 | ≤1 Note 1 | 2 Note 1 | 2 Note 1 | outdoor |
| NOTE 1: Most of the requirements presented here are related to the sensing category 3 in [6] NOTE 2: Detection, classification of UAV features such as propellers may require more stringent velocity and range resolution due to the dimensions of propellers with respect to the UAVs. Some propeller diameters to UAV body ratio could be between ½ to 1/3 [x], therefore, the range resolution values are modified accordingly. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * *