3GPP TSG SA WG 1 Meeting #112 	S1-254262

17-21 November 2025, Dallas, TX, USA	(revision of)

Source:	LG Electronics

Title:	Proposed Clarification on sensing-based positioning

Draft TS/TR:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.4.2 ISAC

Document for:	Approval

Contact:	Ki-Dong Lee (kidong.lee (at) lge.com)

Abstract: In many practical cases, the meaning of relative distance estimated via 3GPP sensing-based positioning and that via conventional 3GPP-based positioning are different. The reference point of the former case is the closest point on surface (reflection point) whereas the reference point of the latter case is the antenna reference point (APR). If the object is relatively large compared to the distance (e.g., road vehicle sensing for autonomous driving), the related KPIs need to be explicitly described reflecting such distinctions. It is proposed for discussion to put a suitable clarifying note in ISAC clauses (both in the main clause and CPR clause).

1. Introduction

In V2X, UAV, service robots, and cobots scenarios utilizing 3GPP Sensing for relative distance estimation, it's essential to recognize that the measurement output differs fundamentally from traditional 3GPP Positioning. Sensing estimates the distance from a Reference Point of Sensing (RPS) on the reference vehicle to the closest surface point of the neighboring object (e.g., vehicle). Conversely, traditional Positioning estimates the distance between the Antenna Reference Points (ARP) of the respective UEs. While this distinction is negligible for distant objects (e.g., in case of distant UAV detection), it becomes critically important for high-precision, safety-critical use cases such as autonomous driving and robotic operations such as in warehouse, dysinfection site, and residential areas where robots and humans (and companion animals / service dogs) are present. This crucial difference must be clarified and reflected in the applicable requirements proposed in the Sensing aspect of this 6G study. We propose that the relative distance estimated by 3GPP Sensing is explicitly defined as the distance from the RPS to the sensed object's surface.

The requirements for Sensing accuracy and resolution (e.g., range resolution in centimeters) are significantly tighter than traditional positioning. This high precision is meaningless if the target point (surface vs. antenna) is not explicitly defined.

For proximity-sensitive, safety-critical use cases such as autonomous driving V2X, the inherent difference in measurement output between 3GPP Sensing and 3GPP Positioning is vital. 3GPP Sensing aims to estimate the distance to the physical surface of a neighboring object, which is crucial for collision avoidance and planning. In contrast, traditional 3GPP Positioning estimates the distance to the neighboring UE's Antenna Reference Point (ARP). When the relative distance is small, the offset between the object's surface and the ARP is not negligible and must be accounted for by the Sensing requirements.

2. Reason for Change

See below.

3. Conclusions

See below.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1.

* * * First Change * * * *

# 7	Integrated Sensing and Communication

## 7.1	General

Integrated Sensing and Communication (ISAC) facilitates new applications and services that require sensing capabilities. The service includes offering wide area multi-dimensional sensing that provides spatial information about unconnected objects as well as connected devices and their movements and surroundings.

Normative service requirements for ISAC have been specified in [6].

NOTE: 	For some sensing-based relative positioning requirements, the relative distance measurement obtained through 3GPP Sensing is referenced to the surface of the sensed object (e.g., the closest physical boundary of a neighboring vehicle, robot, or some obstacle) from the Reference Point of Sensing (RPS) on the reference entity (e.g., regular UE, vehicle, and robot). This is distinct from traditional 3GPP Positioning measurements, which determine the distance between the Antenna Reference Points (ARP) of the communicating entity (e.g., UEs, vehicle, and robot)

* * * Second Change * * * *

(Moderator’s text from S1-254254)

## 7.x	Consolidated performance requirements for Integrated Sensing and Communication

The 6G system shall be able to provide sensing with the following performance requirements:

Table 7.x.1-1: Consolidated performance requirements for Integrated Sensing and Communication.

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing area (sensing target and/or description) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| High topology mapping (NOTE 1) |  | N/A | 0.10 | 0.10 | - |  | [0.4] | - | 50 | ≤ 0.2 | ≤10 | <1 | Outdoor |
| Low-altitude UAV supervision |  | ≥90 | 1-2 | 1-2 | 3-5 | 3-5 | N/A | N/A | 100~1000 | ≤1 | ≤5 | ≤5 | Outdoor (UAV trajectory tracking) |
|  |  | ≥95 | ≤10 | ≤10 | N/A | N/A | [10] | [≥5] | [≤1000] | [≤1] | [≤5] | [≤5] | Outdoor (UAV intrusion detection) |
| Environment object reconstruction |  | 95 | [0.5-5] (NOTE 3) | [0.5-5] (NOTE 3) | N/A | N/A | [0.5] | N/A | [10,000-600,000] | [10-60] | [1-5] | [1-5] | Outdoor (Building (NOTE 2) as in Table 7.6.1-1) |
|  |  |  | 0.5 | 0.5 | 1.5 | N/A | [0.5] | [0.5] | 100 | 0.1 | 5 | 5 | Outdoor (Vehicle (NOTE 2) as in Table 7.6.1-1) |
| Road digitalization |  |  | 0.5 (vehicle) (NOTE 4) 1 (pedestrian) | N/A | 0.5 | N/A | 0.2 | 0.2 m/s | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 | Outdoor (crossroad, as in Table 7.7.1-1) (NOTE 5) |
|  |  |  | 3 (NOTE 4) | N/A | 3 | N/A | 1 | 1 m/s | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 | Outdoor (highway, as in Table 7.7.1-1) (NOTE 6) |
| Ship detection and tracking (NOTE 7) |  |  | ≤100, (NOTE 8) | NA | NA | NA | ≤100 | NA | 5000 | 1 | 5 | 5 | Outdoor, (Detection of ship on the near shore waters and offshore waters) |
|  |  |  | ≤10 (NOTE 8) | NA | NA | NA | ≤10 | NA | 5000 | 1 | 5 | 5 | Outdoor, (Detection of ship on the harbour entrances, harbour approaches, and port. and coastal waters) |
|  |  |  | ≤2~10 (NOTE 9) | NA | NA | NA | ≤2 | NA | 5000 | 1 | 5 | 5 | Outdoor, (Detection of ship on the inland waterway (e.g. river, lake)) |
| Structural health monitoring |  |  | 0.1, (NOTE 19) | 0.1 | N/A | N/A | N/A | N/A | 5000 | 60 | 5 | 5 | Outdoor (e.g. detection of corner reflectors on bridges, buildings in urban scenario) |
| UAV detection, classification and counting |  |  | 1 (NOTE 20) | 1 (NOTE 20) | 1 (NOTE 20) | 1 (NOTE 20) | [0.3 – 1] (NOTE 20) | 1 (NOTE 20) | ≤1000 (NOTE 20) | ≤1 (NOTE 20) | 2 (NOTE 20) | 2 (NOTE 20) | Outdoor |
| Safety Assistance for vulnerable pedestrian |  | [95] | [1] | N/A | [0.5] | N/A | [0.2] | [0.5] x N/A | ≤ [500] | ≤ [0.1] | ≤ [5] | ≤ [5] | Outdoor (Crossing) |
| Collaborative Robots (NOTE 12) (NOTE X) |  |  | [≤ 0.1] | [≤ 0.1] |  |  |  |  | N/A |  |  |  | Indoor/outdoor |
| Infrastructure collapse monitoring |  |  | [4] | [N/A] | [N/A] | [N/A] | [4] | [N/A] | TBD | [ 1] | [1~2] | [1~2] | Outdoor (e.g. Detecting sudden collapse on infrastructure such as highway, railway, road, flyover, rural areas, farmland) |
| UAV takeoff and landing (NOTE X) |  |  | [0.1] (NOTE 13) | [0.1] (NOTE 13) | [1] | [1] | [0.5] | [1] | [100-500] | [0.2] | [5] | [5] | Outdoor (NOTE 14) |
| Gestures Recognition (NOTE 23) (NOTE X) |  | 99 | [0.1-0.3] | [0.1] | n/a | n/a | [≤0.02] | n/a | ≤20 | [≤0.02] | ≤ 1% (NOTE 22) | ≤ 10% (NOTE 22) | Indoor, Factory Environment, (hand gesture) |
|  |  |  | [0.1] | [0.1] | n/a | n/a | [≤0.002] | n/a | ≤20 | [≤0.02] | ≤ 1% (NOTE 22) | ≤ 10% (NOTE 22) | Indoor, Factory Environment, (facial gesture) |
|  |  |  | [0.1] | [0.1] | n/a | n/a | [≤0.05] | n/a | ≤20 | [≤0.02] | ≤ 1% (NOTE 22) | ≤ 10% (NOTE 22) | Indoor, Factory Environment, (head gesture) |
|  |  |  | [0.1] | [0.1] | n/a | n/a | [≤0.1] | n/a | ≤20 | [≤0.02] | ≤ 1% (NOTE 22) | ≤ 10% (NOTE 22) | Indoor, Factory Environment, (body and limb gesture) |
| Enhanced XR navigation |  | 99.9 (NOTE 11) | ≤0.5 (NOTE 10) | 0.5 (NOTE 10) | 0.5 | N/A | 0.1- 1 (NOTES 10, 11) | 0.5 (NOTE 10) | ≤100 | 0.1 – 1 (NOTE 11) | 1% | 1% | Indoor, Factory Environment, (10 m2) |
| Safe and economic UAV transport |  | 99.99 | ≤ 50 | 50 (NOTE 15) | 5 (NOTE 15) | 5 | 10 (NOTE 15) | 0.5 (NOTE 15) | ≤ 100 | 0.1 – 1 | 0.1% (NOTES 16, 17) | 1% (NOTES 16) | Outdoor (NOTE 18) |
| NOTE 1: The accuracy KPIs in Scenario “High Topology mapping” are extrapolated from [323] Table C2 as considered for autonomous navigation and remote driving. NOTE 2: A sensing target in Scenario “Environment object reconstruction” refers to a [segment/part] of the target object to be detected and/or tracked, whereas the size of each part is comparable to corresponding spatial resolution of object reconstruction. The percentage of missed detection/false alarm represent missed/falsely reconstruction of parts of the object statistically NOTE 3: Considering a variety of dimensions, shapes, and functionalities of urban buildings, a range of KPI values are needed to measure and provide the flexibility of reconstruction accuracy. NOTE 4: Vehicle, with the assumptions in Table 7.7.1-1. NOTE 5: Sensing target density for vehicles ≤ 750 per 1000 m x 24 m; sensing target density for pedestrians ≤100 per 1000 m x 24 m. Sensing target density is described in Clause 3.1 of the present document. NOTE 6: Sensing target density for vehicles ≤ 72 per 1000 m x 24 m. Sensing target density is described in Clause 3.1 of the present document. NOTE 7: The typical size (Length x Width x Height) of ship is 270 m x 30 m x 30 m in both sensing service category#1 and #2, and 100 m x 15 m x 15 m in sensing service category #3, according to reference [91] NOTE 8: The KPI values for detection of ship on these sensing service categories are referred to [192]. NOTE 9: The KPI values for detection of ship on this sensing service category are referred to [193]. NOTE 10: This positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the user NOTE 11: The KPIs are based on the Hazard Prevention in Industrial Environments for “Prediction of workers’ and machines’ actions” and “Detection of worker’s location” in [94] NOTE 12: Related usecases are captured in clause 7.2 of 3GPP TR 22.837 [9] and clause 6.2 of 3GPP TS 22.137 [6]. NOTE 13: The KPI values for accuracy of positioning are centimetre-level sourced from reference [199], [200] NOTE 14: The KPI values of this use case are effective for sensing in specific area for UAV takeoff and landing. NOTE 15: The listed positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the UAV and validate the reported location of the UAV itself. NOTE 16: Safety impact of missed detection is substantially higher compared to the false alarm, here fore the Missed Detection is assigned a more stringent value compared to the False Alarm. NOTE 17: To enable safe operation of BVLOS UAV flights in mixed airspace (manned & unmanned) and in high risk areas (e.g. above cities) the value for missed detection of 0.1% corresponds to the lower value of the missed detection range specified in Table 6.2-1 of TS 22.137 [6] for sensing service category 2. NOTE 18: Sensing service availability [%] is 99.999 NOTE 19: The typical length of corner reflector is 0.5m [327]. NOTE 20: Most of the requirements presented here are related to the sensing category 3 in [6] NOTE 21: Detection, classification of UAV features such as propellers may require more stringent velocity and range resolution due to the dimensions of propellers with respect to the UAVs. Some propeller diameters to UAV body ratio could be between ½ to 1/3 [330], therefore, the range resolution values are modified accordingly. NOTE 22: Depending on the actual application to be controlled by the gestures, Missed detection and False alarm may have different KPI values from < 1% to 10%. NOTE 23: References for hand gesture recognition are [331][332], for facial and head gesture recognition [333], and for body and limb gesture recognition [334][335]. NOTE X: The relative distance measurement obtained through 3GPP Sensing is referenced to the surface of the sensed object (e.g., the closest physical boundary of a neighboring vehicle, robot, or some obstacle) from the Reference Point of Sensing (RPS) on the reference entity (e.g., regular UE, vehicle, and robot). |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

/* none */