3GPP TSG-SA WG1 Meeting #112 	S1-254278

17-21 November 2025, Dallas, USA	(revision of S1-253050r2)

Source:	LG Uplus, SK Telecom, Thales, Novamint

New use case title:	New use case on Event-based Inter-PLMN Coordination

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.5.2

Document for:	Approval

Contact:	Taeyeoun Kim (tkim@lguplus.co.kr), Jaehyun Ahn (jaehyun80@lguplus.co.kr), nicolas.chuberre@thalesaleniaspace.com

Abstract: This use case primarily focuses on scenarios of temporal on-demand events that require inter-PLMN coordination. More specifically, it is based on an NTN scenario and cell reselection between MNOs and SNOs. Through this use case, MNOs can ask SNOs to provide the satellite-based communication only when to be needed such as during disasters, enhancing energy saving, and fulfilling other critical objectives.

---------- NEW TEXT for clause 8.X of TR 22.870 ----------

x.1	Use case on Event-based Inter-PLMN Coordination

x.1.1	Description

The concept of ubiquitous network aims to enhance connectivity and improve user experience by ensuring uninterrupted access to communication services across all times and locations. Within this framework, Non-Terrestrial Networks (NTNs) are recognized as a pivotal enabler of ubiquitous connectivity, as they expand and sustain network coverage in areas where traditional terrestrial infrastructure is limited or unavailable. Among NTNs, satellites are anticipated to play a critical role in realizing ubiquitous connectivity, particularly through the deployment of Low-Earth Orbit (LEO) satellite constellations or Geostationary Orbit (GEO) satellite systems.

In the context of 6G system, effective collaboration between Network Operators of respectively TN and the NTN components through coordination will allow to ensure ubiquitous connectivity to user devices while maximising the performance of the network (TN + NTN) and the QoS for the users. This coordination between network operators would be especially beneficial to address temporal on-demand events impacting the nominal operation of some network nodes for example due to failure, disasters, localized traffic surge, or energy saving.

This use case primarily focuses on an NTN scenario and cell reselection between MNOs and SNOs. The common theme in this use case is that events cause changes in the coverage of the TN, and the satellite network must compensate for these changes. Given the distinct characteristics of terrestrial and satellite systems, such as differences in signal power attenuation and cell size, the inter-operator coordination is required to ensure seamless connectivity. More specifically, this use case focuses on the user equipments (UEs) in idle mode. For the event area, the NTN operator broadcasts updated cell reselection priority to the idle mode UEs within its satellite beam coverage. Fundamentally, the inter-operator reselection coordination use case is to minimize unnecessary power consumption in user equipment (UE) by preventing unintended network reselections.

x.1.2	Pre-conditions

Figure 1: Pre-condition description in normal times and cell coverage comparison

Operator A(TN) is a Mobile Network Operator that is serving a region named PLACE1, and Operator B(NTN) is a Satellite Network Operator that can also serve the region PLACE1 as a complementary. Including the region of the PLACE1, the cell reselection priority is set for TN to idle mode devices for their connection stability and power efficiency. For example, in the areas where TN cells are actively serving customers, the neighbour cell list of TN cells in Operator A is configured to give high priority to other TN cells while assigning lower priority to NTN cells. the neighbour cell list of NTN cells in Operator B is configured to give lower priority to NTN cells and higher priority to TN cells.

Someday, the “event” occurred. For example, the infrastructure of Operator A is destroyed in PLACE1 due to the natural disaster. There exist user devices in idle mode that are served by Operator A: UE1 in PLACE1 and UE2 not in PLACE1, but nearby the PLACE1.

x.1.3	Service Flows

Figure 2: Example of event based inter-PLMN coordination

1. Usually, Operator A and Operator B set priority of NTN to be low in both TN and NTN, in order to prevent undesired network reselection of customers.

2. If an event such as disaster or traffic offload occurs over a region, i.e. PLACE1, Operator A sets priority of NTN higher within TN and NTN for PLACE1. Also, Operator A will request Operator B to change the priorities for PLACE1.

- * Due to the large cell size of NTN, Operator B should also adjust the priorities for PLACE2, which is located near PLACE1 and falls within the coverage area affected by Operator B’s service for PLACE1

3. The user devices (UE1 and UE2) in Operator B’s gNB coverage which include PLACE1 receive the cell selection priority information and performs reselection, wherein Operator B’s gNB coverage would be bigger than PLACE1.

- UE1 (in PLACE1): Since UE1 is in the regional information of PLACE1, perform the cell reselection to be connected to Operator B.

- **UE2 (in PLACE2): Due to the large size of the satellite beam, UE2 will receive high prioirty for NTN from Operator B. However, within PLACE2, UEs would be better to be served by Operator A in QoS experience perspective. Thus, UE2 needs to select Operator A based on the regional priority information via Operator B.

x.1.4	Post-conditions

UE1 that is in the event area PLACE1 maintains the network connection during the event by reselecting to the NTN cell in idle mode.

UE2 that is in PLACE2 during the event still connect to Operator A. Thus, UE2 prevents unnecessary power consumption when reselecting NTN. (Note that if the event area is small, limit in satellite beam size may cause unnecessary requests for priority adjustments for NTN even in surrounding areas outside the intended region.)

x.1.5	Existing features partly or fully covering the use case functionality

Clause 5.2.4.5 in TS 38.304 defines the cell reselection criteria for NR inter-frequency and inter-RAT scenarios. The UE can be provided with priorities for different frequencies or RATs, and the specification outlines the conditions under which the UE shall reselect to a higher or lower priority based on the received signal quality and level.

Clause 5.2.4.6 in TS 38.304 specifies the cell reselection criteria applicable to intra-frequency and equal priority scenarios. In these cases, the reselection decision is based on a ranking criterion that incorporates measurement offsets to determine the most suitable cell.

x.1.6	Potential New Requirements needed to support the use case

[PR x.1.6-1]: The 6G system shall provide an interface between two Public Land Mobile Networks (TN and NTN) for network operators to exchange network management related information in view of coordination in case of temporal on-demand events such as disasters, sudden traffic load increase, energy shortage, etc.

NOTE: network management related information may refer to PLMN status/capability, measurements.

NOTE: coordination may be needed in terms of PLMN reselection policies, coverage/capacity allocation over a given area.

NOTE: The coordination may require to activate inter PLMN procedures such as initiation, activation, update, termination and rollback, with acknowledgement and validity control

[PR x.1.6-2]: The 6G system shall be able to provide event-scope information including regional identifiers, validity time, and so on, which to guide UE cell selection strategy.

[PR x.1.6-3]: The 6G system shall enable a UE, when idle or inactive, to conditionally reselect a PLMN (TN or NTN) based on operator-provided priority information and the UE’s location and context to minimize power consumption.