3GPP TSG SA WG 1 Meeting #112 	S1-254393

Dallas, USA, 17 - 21 November 2025	(revision of S1-254282r1)

Source:	TNO, Airbus, ESA, Fraunhofer, Thales, Novamint

Title:	Resubmission of use case on High-altitude UAV supervision

Draft Spec:	3GPP TR 22.870 V0.4.1

Agenda item:	8.1.4.2 Resubmission of Use Cases and others

Document type:	pCR

Document for:	Discussion/Approval

Contact:	relja.djapic@tno.nl

Abstract: This document resubmits a use case on integrated sensing and communication for high-altitude UAV supervision.

1. Introduction

Integrated Sensing and Communication (ISAC) is an important element of 6G as reflected in Chapter 7 of TR 22.870. However, the use cases so far focus to the UAV ISAC cases at lower altitudes that is typically covered by the terrestrial networks. In addition to UAV operations based on the TN, uthors in ([X1], [X2], [X3]) prove the feasibility of ISAC based on non-terrestrial networks (NTN). In additoin, [X4] and [X5] develop satellite processing/computing payload to support NTN based ISAC. Accordingly, it becomes apparent that the field of ISAC from space is rapidly emerging and that non-terrestrial networks (NTNs) can support sensing application.

The following table highlights the ISAC use cases currently in TR 22.870 V0.4.1 that would benefit from NTN contributions in particular (but not restricted to) remote areas:

| ID | Use case on | Remarks |
| --- | --- | --- |
| 7.1 | coordination of search and rescue missions in large disaster areas | The satellite assets of a NTN Satellites can provide significant contributions to coordination in disaster situations in particular in remote areas and if the operation of the TN is impaired by the disaster (e.g. loss of terrestrial base stations, …) |
| 7.3 | high-resolution topographical maps | NTN can provide additional sensor layers to improve accuracy of topographical maps |
| 7.6 | road digitalization | NTN ISAC can complement by provisioning suitable traffic flow data in remote areas with a scarcity of terrestrial base stations. |
| 7.9 | advanced modern city transportation system | Analogously to 7.6 NTN can complement TN ISAC services for providing additional data to Intelligent Transportation Systems |
| 7.10 | stored sensing data handling | NTN can provide additional sensing data upon request especially in barely accessible remote regions |
| 7.14 | infrastructure collapse monitoring | Infrastructure monitoring and change detection is an estalbished Earth Observation use cas, NTN can contribute to pre-event monitoring through wide area surveillance and automated change detection |
| 7.15 | multi-sensor fusion based sensing for UAV takeoff and landing | NTN ISAC results can be fused into the results from other (terrestrial) sensors to improve accuracy |
| 7.17 | safe & economic UAV transport | UAV transport is in particular viable in rural regions where TN coverage can be sparse. NTN coverage can complement with sensing data for safe UAV operation even in these areas |
| 7.20 | autonomous driving based on network-assisted sensing | Sensing data obtained by the 3GPP network – both TN and NTN – together with the vehicle’s onboard sensors can very advantageous for the safe operation of the autonomous vehicle |

The use case 7.5 in TR 22.870 on  the Low-altitude UAV supervision is relying on ISAC with terrestrial base stations, which are limited in reach and coverage of higher altitudes. Utilizing NTN would enable supervision of UAVs also at higher altitudes (e.g., above 100m).

Consequently, this contribution proposes a new use case: High-altitude UAV supervision, based on ISAC principles in combination with NTN.

2. Reason for Change

Ensure that both TN and NTN based based Integrated Sensing and Communications for High altitude UAV supervision will be supported by 6G networks.

3. Conclusions

Add a new use case on NTN based Integrated Sensing and Communications (NTN ISAC) in TR 22.870.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.4.1.

* * * 1st Change * * * *

All New Text

# 2	References

[X1]	Matera et al, “Terrestrial and Non-Terrestrial Networks for Integrated Sensing and Communication”, International Symposium on Systems Engineering ISSSE 2024, Oct 2024.

[X2]	Y. Gu et al, "ISAC towards 6G Satellite–Terrestrial Communications: Principles, Status, and Prospects", 2024, Electronics 13, no. 7: 1369, https://doi.org/10.3390/electronics13071369.

[X3]	L. You et al, “Ubiquitous Integrated Sensing and Communications for Massive MIMO LEO Satellite Systems”. Integrating sensing and communication for ubiquitous internet of things, IEEE Internet of Things Magazine, Volume: 7, Issue: 4, July 2024

[X4]	M. Corici et al. "Repurposable payloads: concept, design, and initial results," 41st International Communications Satellite Systems Conference (ICSSC 2024), Seattle, USA, 2024, pp. 120-124, doi: 10.1049/icp.2024.4622.

[X5]	ESA Project 5GEOSiS – 5G Earth Observation Server in Space: https://connectivity.esa.int/projects/5geosis-–-5g-earth-observation-server-space

[X6]	C. K. Sheemar et al., "Joint Communication and Sensing for 6G Satellite Networks: Use Cases and Challenges"    https://arxiv.org/abs/2501.05243v2

[X7]	L. Yin, Z. Liu, M. R. B. Shankar, M. Alaee-Kerahoodi and B. Clerckx, "Integrated Sensing and Communications Enabled Low Earth Orbit Satellite Systems," in IEEE Network, vol. 38, no. 6, pp. 252-258, Nov. 2024, doi: 10.1109/MNET.2024.3371172

[X8]	N. Huang, P. Li, L. Qian, Y. Ren and Y. Wu, "Integrated Sensing and Communication for Satellite-Terrestrial Integrated Network with Multi-access Mobile Edge Computing," in IEEE Internet of Things Journal, doi: 10.1109/JIOT.2025.3625450

[X9]	Q. Wang, X. Chen, Q. Qi, M. Li and W. Gerstacker, "Multiple-Satellite Cooperative Information Communication and Location Sensing in LEO Satellite Constellations," in IEEE Transactions on Wireless Communications, vol. 24, no. 4, pp. 3346-3361, April 2025, doi: 10.1109/TWC.2025.3530083.

[X10]	Q. Kong et al., "Cooperative Precoding for Integrated Sensing and Communication in Massive MIMO LEO Satellite-Terrestrial Networks," in IEEE Wireless Communications Letters, doi: 10.1109/LWC.2025.3614432

* * * 2nd Change * * * *

All New Text

## 7.x	Use case on High-altitude UAV supervision

### 7.x.1	Description

With the development of Uncrewed Aerial Vehicles (UAV) technologies, light and small civilian UAVs have played a great role in aerial photography, agriculture, mapping and other fields. Various commercial UAV applications are now becoming a reality. Some of these UAVs operate at high altitudes (e.g. above 100m) often in the same airspace as commercial aircraft. This may produce a series of safety control problems, e.g. UAV illegal intrusion and UAV collision. In addition, surveillance of commercial air traffic in remote areas (oceans, mountains, deserts) remains a challenge and suitable data supplementary to data provided by existing ATC/ATM (air traffic control/ air traffic management) systems will be welcome by the relevant service providers and flight safety authorities. Generally, non-terrestrial network (NTN) based ISAC can provide capability to detect, locate and track objects/targets on or near the Earth surface [X6]. The integrated sensing capabilities of 6G from space using NTN [X1, X2, X3, X4, X5] can support the collection of supplementary data suitable for ATC/ATM and can thus be used to optimize air traffic flow, improve safety and simultaneously enable efficient communication between pilots and air traffic control. The associated KPIs are analogous to typical RF/radar sensing metrics like detection and false-alarm probability, mean squared error (MSE), beampattern approximation and radar mutual information [X7].

Even though, the distances in case of NTN are longer than between terrestrial base stations and UEs, the respective links have been designed for that distance (with appropriate signal strength and aperture size). Accordingly, sensing functionalities will also work in case of NTN, even if specificities still need to be optimized. While basic capabilities can already be implemented in a monostatic setting with a single satellite, sensing performance will be significantly improved when two (bistatic) or multiple (multistatic) satellites and/or user equipment cooperate ([X8], [X9]) and when MIMO techniques are employed [X10]. In principle, for this purpose GEO as well as NGSO satellites could be used. However, LEO and VLEO satellites will be able to deliver by far the best detection, localization and tracking performance. In order to improve supervision precision the sensing procedures could combine TN, NTN signals originating from satellites as well as the ones originating from the ground NTN segment.

Additional details in particular on the detection, supervision and collision avoidance of UAV can be found in clause 7.4.1 and its application to high-altitude UAVs and commercial aviation can easily be inferred.

### 7.x.2	Pre-conditions

Terrestrial and Satellite Network Operator B (T-SNOB) provides 6G sensing service in high-altitude airspace for air traffic management and UAV flight assistance service, including flight trajectory tracing, collision prediction, illegal UAV intrusion detection etc. T-SNOB can make use of TN and NTN satellites to sense the airspace within their coverage area and report the sensing information (including tracked UAV and aircraft as well as the environment around them) to the respective authorities and service suppliers such as – ATM, USS (Uncrewed Aerial System Service Supplier)/UTM.

The UAV Company UAVCY uses the USS/UTM to supervise the high-altitude UAVs..

The USS/UMT uses 6G sensing service provided by T-SNOB to track the flight trajectory and also for potential UAV illegal airspace intrusion detection and UAV collision prediction.

Air Navigation service providers (ANSPs) use the 6G sensing service provided by the T-SNOB as an auxiliary source of data for providing ATM/ATC services

The UAV Operator/UTM provides specific details to the UAVCY, including the characteristics of the UAV that will be tracked, along with details about the time and location for flight tracing. This information includes regulated flight paths as well as potential areas where the UAV might temporarily deviate from its route.

ANSPs may provide specific details to T-SNOB about air traffic environment, flight paths and other characteristics.

T-SNOB can realize 6G data collection, 6G data transmission, 6G data processing, 6G data storage within the network, and provide sensing results to the ATM/USS/UMT/UAV.

### 7.x.3	Service Flows

For high-altitude UAV supervision:

1.	The UAV Company (UAVCY) requests 6G sensing service for high-altitude UAV from the USS/UTM.

2.	The USS/UTM transmits the request to the 6G Terrestrial and Satellite Network Operator B (T-SNOB).

3.	T-SNOB utilizes its 6G TN and NTN assets to track the high-altitude UAV of UAVCY constantly collecting sensing data of the UAVs location.

4.	The 6G Network aggregates, forwards and transmits the data generated by the base stations, and processes the data to obtain the sensing results.

5.	The 6G Network exposes the sensing result to the USS/UTM. The USS/UTM could trigger to send warning and control messages to the UAV and if necessary to take corrective action and redirect the UAV.

For supplementary ATM data provision:

1.	When the scheduled time for tracking begins, T-SNOB activates the aircraft trajectory tracing service within the designated area until the tracking session ends.

2.	Using radio sensing, a network of 6G TN and NTN base stations and connected devices (UEs) detect aircraft and continuously gather data on its position and movement, such as distance, velocity and angle. These metrics, also known as 3GPP sensing data, are sent to a 6G processing unit for real-time analysis.

3.	During the flight, if the aircraft leaves the coverage range of one TN or NTN base station and enters a new coverage zone, the 6G system could let the old base station stop radio sensing, and switch to a new base station for sensing until it is out of coverage. This transition is based on the aircraft's estimated position and velocity, which the 6G processing unit calculates. The network can automatically adjust the sensing operations at base stations depending on this data or based on a pre-defined time frame. In certain cases, sensing handover may be triggered to maintain continuous coverage. For instance, if the current base station's connection weakens or if another nearby base station can offer better coverage for the aircraft, the system proactively shifts the sensing function to this new station to ensure uninterrupted tracking.

4.	The 6G processing unit can aggregate sensing data from multiple sources, including RANs and UEs, to estimate aircraft's location and velocity. Similar approach can also be applied to multiple aircraft, in the case there are multiple aircraft service in the area. This real-time information is then transmitted to the ANSP, who monitors the aircraft's trajectory.

5.	If the respective aircraft deviate from prescribed routes, the ANSP receives alerts, allowing them to take corrective action and redirect the aircraft as necessary. This is especially valid for remote airspaces such as over oceans.

### 7.x.4	Post-conditions

For high-altitude UAV supervision: UAV#1 efficiently and safely follows the tracked flight route to its destination; any off-route movements are detected and can be quickly corrected. Thanks to the wide-area and constant sensing capability of the 6G TN and NTN, and the efficient data transmission, processing and storage by the 6G core network, the safe supervision of the high-altitude UAV of UAVCY is improved.

For supplementary ATM data provision: The supplementary ATM data are included in the ATM activities of the ANSPs to provide safe and efficient air traffic control and to detect and correct any deviations in a timely manner.

### 7.x.5	Existing features partly or fully covering the use case functionality

In TS 22.137 [6], there are requirements specified for 5G system on integration of sensing and communication:

Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

Subject to user consent, regulation, and operator’s policy, the 5G system should support the joint processing of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result.

Subject to operator’s policy, the 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing service based on specific parameters (e.g., refresh rate, period of time, sensing KPIs, geographical location) and to receive the corresponding sensing results.

### 7.x.6	Potential New Requirements needed to support the use case

[PR 7.x.6-1] The 6G system shall be able to provide high altitude (>100m) UAV supervision through 6G system sensing capabilities with the following KPIs:

Table 7.x1.6-1:	Performance requirements for High-altitude UAV supervision

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [s] | Refreshing rate (Hz) | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s] |  |  |  |  |
| High altitude UAV supervision | Outdoor | ≥95 | ≤10 | ≤10 | ≤5 | ≤5 | 10 | 3 | < 1.5 Note 3 | 1 Application configurable | ≤5 Note 2 | 5 |
| NOTE 1: The listed positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the UAV operating on higher (>100m) altitudes where the expected density of the UAVs is expected to be lower compared to altitudes <100m. NOTE 2: Table 6.2-1 of TS 22.137 [6] for sensing service category 2 specifies a range of possible values for missed detection in the range of 0.1%-5%. Possible usage of NTN for High altitude UAV supervision implies the selection of ≤5% missed detection value in this use-case. NOTE 3: Maximal sensing service latency requirement equals 1.5s to cover latencies induced by possible usage of NTN for high altitude UAV sensing. |  |  |  |  |  |  |  |  |  |  |  |  |