3GPP TSG-SA WG1 Meeting #112 	S1-254418

17-21 November 2025, Dallas, USA	(revision of S1-254086)

Source:	Qualcomm Incorporated, T-Mobile USA

New use case title:	Use case on Joint QoS handling for 6G Service

Draft TS/TR:	3GPP TR 22.870 v.041

Agenda item:	8.1.6.2

Document for:	Approval

Contact:	Lola Awoniyi-Oteri<oawoniyi@qti.qualcomm.com>

Abstract: This contribution is the re-submission of a new use case which focuses on the coordination of multimodal traffic associated with a user application.

1. Introduction

Addressing comments from the last SA1 preparation call,

2. Reason for Change

- Revised based on comments from:

  - Ericsson asked about how many endpoints were used for the communication- Added the text in the preconditions to clarify single end points are assumed: “In this use case, single communication endpoints are assumed.”

  - Nokia suggested some text change “coordination needs” and “influencing policies”. These texts were added in PR1

  - Huawei and Nokia wanted clarification on where the info was coming from the UE or user application. We removed the user application since the use case only supports UE.

- Adding TMO as co-sourcing company.

- Incorporated additional offline comments from Huawei.

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1.

* * * ALL NEW TEXT * * * *

## 9.x	Use case on joint QoS handling

### 9.x.1	Description

Mobile devices, including wearables, XR devices, laptops, smartphones, cameras, or other IoT devices, are increasingly incorporating computing capabilities. These devices often run compute-intensive applications such as Large Language Models (LLM), scene understanding, graphics rendering, avatar generation.

While these devices’ capabilities are getting better to support such applications, it is inevitable that some devices will be much more powerful than others. For example, a laptop’s computing capability is more powerful than wearables. Therefore, there is the need for distributed computing operation, where UE application clients can distribute their computing workloads (applications) to other UEs, edge servers, and/or cloud servers available within their network. For example, for an LLM workload, there could be different compute service modes: compute locally on device, compute remotely on edge server, or compute remotely on cloud server. The 6G network can assist distributed compute for client applications via richer compute discovery and dynamic selection of compute serving modes to balance power vs user experience.

Distributed compute for workloads such as AI inference, XR, and AR virtual assistance often involve multiple simultaneous data flows with variety of traffic types. For example, compute for an XR application may involve offloading 3D reconstruction (3DR), hand tracking, and rendering tasks (as shown in Fig. 9.x-1):

- Uplink input stream: head tracking information, hand tracking information, camera images.

- Downlink output stream: Rendered video, and 3D mesh.

Fig. 9.x-1:  XR application – example of compute related service flows

In another example, Image/Video Generation via LLMs can include:

- Uplink input stream: Text Prompt

- Downlink output stream: Generated image/video, and audio summary

In the above examples, there could be different groups of flows, where e.g.:

- Each flow may be uplink or downlink.

- Each flow may correspond to different traffic type, e.g., text, audio, image, video, etc.

- Each flow can have different QoS, e.g., data rate, requirements.

QoS requirements for multiple flows can be interdependent, i.e., satisfying QoS for one flow (or some flows) only can be insufficient to meet application-level requirements. Furthermore, there can be multiple combinations of QoS profiles, for different groups of data flows, that may be acceptable for the application.

To address those QoS challenges, there should be the ability to provide a UE indication or preference, to the network, to assist managing groups of flows associated to a service, e.g. ensuring that all flows in the group are admitted simultaneously and corresponding resources allocated based on joint QoS requirements. Furthermore, the UE could also provide alternative groups of data flows with associated QoS profiles, and the network can dynamically select the optimal group / QoS based on requirements and resources availability.

### 9.x.2	Pre-conditions

John and Samantha use wireless communication services provided by their MNO.

Samantha is a medical worker, using an AI-powered ultrasound scanner application on her smartphone, e.g. when operating off-site.

John is an XR gamer using smart glasses connected to the mobile network, located in the same area as Samantha. His application involves scene rendering computation which can only be computed at one remote mode at an edge server. In this use case, single communication endpoints are assumed.

### 9.x.3	Service Flows

- Samantha needs to run a medical scan. She needs to run LLM-based diagnostics, which has two communication service options because of the computation demands and the location of the compute resource. The options are;

  - Option 1: remotely at a cloud server with medium-resolution images on UL requiring 5Mbps and processed images and audio on DL requiring 10Mbps; and

  - Option 2: remotely at an edge server with high-resolution images on UL requiring 15Mbps and processed images and audio on DL requiring 30Mbps.

Samantha’s UE indicates to the network the two acceptable communication service options, as well as the associated UL and DL performance requirements.

- The network optimizes its resources and admits Samantha’s UEs request with option 2, guaranteeing its UL and DL requirements, simultaneously. This allows Samantha to run the LLM-based diagnostics remotely with excellent performance using option 2.

- Meanwhile, John decides to play his favorite XR game using his smart glasses. He puts on the glasses and starts the game, which involves scene rendering computation. His scene rendering task was admitted with only one communication service option, the rendering tasks are performance remotely on an edge server with control information (e.g. pose info.)  is sent on the UL requiring 1Mbps and rendered video frame on DL requiring 25 Mbps. John’s UE indicates to the required group of DL and UL communication data flows with their associated requirements to the network. The network – which is initially not congested - optimizes its resources and admits John’s connection, guaranteeing the requested UL and DL requirements. This allows John’s glasses to run scene rendering remotely at the edge with excellent user experience.

- As time goes by, the network starts to get congested. Since the network knows about Samantha’s alternate acceptable communication service option (i.e. Option 1 with lower communication requirements), the network switches Samantha’s phone connectivity from Option 2 to Option 1 to ensure her user experience is acceptable while preserving John’s user experience.

### 9.x.4	Post-conditions

John’s glasses continue running scene rendering remotely at the edge with requested QoS met and Samantha’s phone running the LLM-based diagnostics remotely with acceptable QoS.

### 9.x.5	Existing features partly or fully covering the use case functionality

### Existing requirements in TS 22.261 [14]:

The 5G system shall enable an authorized 3rd party to provide policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

The 5G system shall support a means to apply 3rd party provided policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

NOTE:    The policy can be used by a 3rd party application for coordination of the transmission of multiple UEs’ flows (e.g., haptic, audio and video) of a multi-modal communication session.

### 9.x.6	Potential New Requirements needed to support the use case

[PR 9.x.6-1] Based on operator policy, the 6G system shall enable a UE to indicate to the network coordination information (e.g. influencing policies for QoS handling) for transmitting the set of data flows for a multimodal communication session associated with an application.

NOTE:    One example of influencing policies can be alternative sets of data flows with different QoS requirements for the multimodal communication session, so that the network can dynamically determine optimal QoS handling, e.g. based on network resource availability.

* * * End of  Change * * * *