3GPP TSG SA WG 1 Meeting #112 	S1-254449

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254285r2)

(revision of S1-254285r1)

(revision of S1-254285)

(revision of S1-254038)

(revision of S1-253112)

Source:	CEWiT, Tejas Network Limited, Softbank Corp.

pCR Title:	Pseudo-CR on Real time traffic offloading using HAPS in a time specific requirement on a geographically challenging area

Draft Spec:	3GPP TR 22.870 v.0.3.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	ankita.lachhwani@cewit.org.in

Abstract: This contribution proposes a new use case(re-submission) for real time on demand connectivity and traffic offloading on a geographically challenging area

* * * First Change * * * *

## 8.X	Use case for ensuring essential services using real time traffic offloading with HAPS in a time specific requirement on a geographically challenging area

### 8.X.1	Description

During mass gatherings or temporary events in a geographically challenging or remote areas, existing terrestrial network infrastructure often becomes insufficient to meet the surge in service demand. The deployment of additional or backup infrastructure in such ad-hoc scenarios is typically not cost-efficient due to high logistical and operational complexities. Furthermore, the increased load significantly raises energy consumption, making traditional approaches less sustainable. Following challenges are faced doing these situations

Limited Scalability for Sudden Traffic Surges:

- Terrestrial networks are typically designed based on average or predictable usage patterns.

- During large gatherings or events, the sudden influx of users can overwhelm the available network capacity, leading to congestion, degraded performance, and service outages.

High Deployment and Operational Costs:

- Expanding coverage or capacity temporarily (e.g., using portable base stations) involves significant logistical and financial resources.

- These solutions are often not cost-effective for short-duration or one-time events.

Challenging Terrain and Inaccessible Locations:

- In mountainous, forested, or remote areas, installing or upgrading terrestrial infrastructure is complex, slow, and expensive.

- It may require building roads, laying fiber cables, or using helicopters, all of which increase costs and delays.

Energy Constraints:

- Increased load on the network leads to higher energy consumption, especially in areas without access to reliable power grids.

- Temporary power solutions (e.g., diesel generators) are not sustainable and contribute to environmental concerns.

Limited Coverage and Service Availability:

- Even with portable units, the coverage radius is limited, and service quality may be inconsistent, especially with obstructions or interference.

- Ensuring seamless connectivity across large or irregular terrains becomes technically difficult.

Long Setup Times and Low Flexibility:

- Traditional deployments require extensive planning, permitting, and physical setup time.

- They lack the flexibility needed for rapid deployment or dynamic scaling during unpredictable situations.

To address these challenges, this use case proposes the deployment of High Altitude Platform Stations (HAPS) to rapidly establish or restore communication services. HAPS can provide supplemental or standalone coverage with minimal ground infrastructure, especially in areas where:

- Network congestion leads to service degradation

- Temporary events result in short-term spikes in demand

HAPS could be utilized to provide at least essential voice and data connectivity to the users in such scenarios. HAPS can be deployed on-demand, providing temporary or supplemental coverage as needed. HAPS platforms can operate using solar energy, aligning with sustainability requirements.

To enable this, the network should configure additional mobility events based on the location and link quality. Furthermore, network requires to establish the wireless link for the backhaul with core network which necessitate to update the existing or introduce addition procedures. In addition to this, HAPS link may not be capable of all QoS handling that terrestrial network could handle. This requires modification of QoS handling procedures in 6G network based on access to ensure service connectivity.

### 8.X.2	Pre-conditions

- A mass gathering event has occurred, overloading terrestrial communications.

- HAPS platform(s) are available for deployment or are already airborne near the designated area.

- The base station on the ground and the base station on board a HAPS platform are connected to the same 6G Core Network.

- Authorized spectrum is available for the usage.

- A mechanism exists to establish backhaul connectivity for the HAPS (e.g. via satellite link (HTS), a connection to an intact ground gateway outside the zone, or through another nearby HAPS (HTH)).

- Coordination with relevant authorities (e.g. aviation, emergency management) is established.

### 8.X.3	Service Flows

- Upon notification/authorization, HAPS platform(s) are deployed/re-tasked to the target area.

- HAPS activates communication payload, establishing service links over the designated area.

- Backhaul connectivity (HTS, HTG and/or HTH) is established.

- UEs register and connect to the network either via HAPS or terrestrial network.

- User has opted for smart switching between TN and NTN based on availability to avoid manual switching

- Voice, data, and potentially location services are provided. HAPS should prioritize traffic for emergency services.

- Network monitoring and management are performed remotely.

### 8.X.4	Post-conditions

- Essential communication services are established/restored in the desired area via HAPS.

- Network status is monitored, and operational logs are maintained.

- The essential services (voice, data) are provided with acceptable QoS.

- The emergency services are provided to the user.

- Thanks to solar power usage the sustainability of the system is maintained.

### 8.X.5	Existing features partly or fully covering the use case functionality

Table 8..X.5-1: Gap analysis for this use case

| Specifications and clauses | Examples of Existing Requirements | Gap Analysis |
| --- | --- | --- |
| TR 22.261[14] Clause 6.3.2 Clause 6.5.2 | Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic (e.g. from 3GPP RAT to non-3GPP access technology), taking into account traffic load and traffic type. The 5G system shall be able to support mobility between the supported access networks (e.g. NG-RAN, WLAN, fixed broadband access network, 5G satellite access network). The 5G network can also support multiple wireless backhaul connections (e.g. satellites and/or terrestrial), and efficiently route and/or bundle traffic among them | These requirement in 5G system requires to be extended to support HAPS. In case of HAPS, the backhaul could be integrated with wireless access |

### 8.X.6	Potential New Requirements needed to support the use case

[PR 8.X.6-1]: Subject to regulation, operator(s) policy, the 6G system shall support load balancing between a base station on the ground and a base station on board a HAPS platform under certain scenario (e.g. congestion of base station on the ground).

Editor’s Note: This requirement is FFS