3GPP TSG-SA WG1 Meeting #112	S1-254503

17-21 November 2025, DALLAS, USA	was S1-254320

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0855 | rev | 2 | Current version: | 19.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the end-to-end latency via satellite in clause 7.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI 17, 5GSAT |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
|  | T |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | With the approved Tdoc SP-241760, CR 0820 (S1-244781), the value of propagation delay via satellite in table 7.4.1-1 has been updated since version 19.9.0. But the content which has dependency on changed values in 7.4.2 was not change accordingly. Details in below: In Version 19.8.0, the yellow highlighted part in clause 7.4.2 align with table 7.4.1-1. Since version 19.9.0, the yellow highlighted part does not align with green highlighted part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating end-to-end latency values and notes in clause 7.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The end-to-end latency will not align with propagation delay as explained in the notes. Especially, the MEO case has a wrong value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *****************   First Change ***********************

7.4.2	Requirements

A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 277 ms end-to-end latency.

NOTE 1:	 5 ms network latency is assumed and added to UE to ground station via satellite propagation delay as defined in Table 7.4.1-1.

A 5G system providing service with satellite access shall be able to support MEO based satellite access with up to 203 ms end-to-end latency.

NOTE 2:	 5 ms network latency is assumed and added to UE to ground station via satellite propagation delay as defined in Table 7.4.1-1.

A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 31 ms end-to-end latency.

NOTE 3:	 5 ms network latency is assumed and added to UE to ground station via satellite propagation delay as defined in Table 7.4.1-1.

A 5G system shall support negotiation on quality of service taking into account latency penalty to optimise the QoE for UE.

The 5G system with satellite access shall support high uplink data rates for 5G satellite UEs.

The 5G system with satellite access shall support high downlink data rates for 5G satellite UEs.

The 5G system with satellite access shall support communication service availabilities of at least 99,99%.

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pedestrian (note 2) | [1] Mbit/s | [100] kbit/s | 1,5 Mbit/s/km2 | 150 kbit/s/km2 | [100]/km2 | [1,5] % | Pedestrian | Handheld |
| Public safety | [3,5] Mbit/s | [3,5] Mbit/s | TBD | TBD | TBD | N/A | 100 km/h | Handheld |
| Vehicular connectivity (note 3) | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | 50 % | Up to 250 km/h | Vehicle mounted |
| Airplanes connectivity (note 4) | 360 Mbit/s/ plane | 180 Mbit/s/ plane | TBD | TBD | TBD | N/A | Up to 1000 km/h | Airplane mounted |
| Stationary | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | N/A | Stationary | Building mounted |
| Video surveillance (note 4a) | [0,5] Mbit/s | [3] Mbit/s | TBD | TBD | TBD | N/A | Up to 120km/h or stationary (note 4b) | Vehicle mounted or fixed installation |
| Narrowband IoT connectivity | [2] kbit/s | [10] kbit/s | 8 kbit/s/km2 | 40 kbit/s/km2 | [400]/km2 | [1] % | [Up to 100 km/h] | IoT |
| Note 1: Area capacity is averaged over a satellite beam. Note 2: Data rates based on Extreme long-range coverage target values in clause 6.17.2. User density based on rural area in Table 7.1-1. Note 3: Based on Table 7.1-1 Note 4: Based on an assumption of 120 users per plane 15/7.5 Mbit/s data rate and 20 % activity factor per user Note 4a: Refer to video surveillance data transmitted (in UL) from a UE on the ground (e.g. picture or video from a camera) using satellite NG-RAN to connect to 5GC, and video surveillance-related configuration or control data sent (in DL) to the UE/device. 0.5 Mbit/s for DL experienced data rate is based on MAVLINK protocol that is widely used for UAV control. 3 Mbit/s for UL experienced data rate is based on the assumed sum from 2.5 Mbit/s for video streaming and 0.5 Mbit/s for data transmission. Note 4b: Up to 120km/h applies to vehicle mounted while stationary applies to fixed installation. Note 5: All the values in this table are targeted values and not strict requirements. Note 6: Performance requirements for all the values in this table should be analyzed independently for each scenario. |  |  |  |  |  |  |  |  |

## *****************   End of Changes ***********************