3GPP TSG SA WG1 Meeting #112                                                	   S1-254276r2

17-21 November 2025, Dallas, USA	 (revision of S1-253341r2)

Source:	Philips International B.V., Qualcomm Incorporated

pCR Title:	Resubmission of use case on improved communication services for cooperating UEs

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.5.2

Document for:	Approval

Contact:	Walter Dees (walter.dees@philips.com)

Abstract: This pCR proposes a new use case for clause 8 Ubiquitous connectivity.

1. Introduction

Clause 8 in TR 22.870 contains several use cases related to enhancing connectivity everywhere. This contribution proposes a new use case for this clause. This is a resubmission of S1-253341r2, with several clean ups (e.g. by using the definition of cooperating UEs as specified in TR 22.870 v0.4.1), and with further clarifications of the benefits to perform coordinated procedures for cooperating UEs.

2. Reason for Change

In current cellular systems, two or more UEs work independently of each other, even if carried by the same user and sharing a common subscription and able to cooperate, such as a smartphone and smartwatch or smart glasses. If two or more UEs are co-located these are likely to be connected to the same base station. However, each of these devices would have to set up and maintain connection with the base station and incur the overhead of these connections. Also these devices are likely to move together and hence would be subject to the same handover context and conditions. Similarly, oftentimes users may reside or move together as a group. By coordinating a set of co-located/cooperating devices the connection handover and also the connection quality can be improved, and hence the overall user experience can be enhanced to ensure ubiquitous connectivity.

3. Proposal

Include the following use case in Clause 8.

* * * First Change * * * *

---------- Use Case template ----------

## 8.x 	Improved communication services for a group of cooperating UEs

#### 8.x.1			Description

More and more users utilize multiple UEs of different types, such as a smartphone, smartwatch and/or smart glasses. In current cellular systems, those UEs work independently of each other, even if carried by the same user, and thus, subject to a similar environment in terms of the cellular infrastructure, mobility and/or user interactions. Each of these devices would have to set up and maintain connection with the base station and incur the overhead of these connections. If these devices are near each other or move together, the devices would be subject to the same communication context and conditions.  Furthermore, there are scenarios when those devices can experience different connection quality, e.g. connectivity of the smart glasses may be poorer (e.g. less powerful antennas) or the connectivity of the phone may be poorer (e.g. because it is inside a user’s pocket/backpack), thus certain traffic could be better routed via another surrounding device (e.g. smartphone), improving overall user experience.

These scenarios apply to both TN as well as NTN and help to improve QoS/QoE and service availability in various network conditions, including network conditions in scarcely covered areas.

#### 8.x.2 	Pre-conditions

Rob is a subscriber of Operator A and registers his UEs, namely his mobile phone and smart glasses, as cooperating UEs.

Both his mobile phone as well as his smart glasses can work independently from each other, but they can act as cooperating devices if they are in proximity of each other. The proximity can be detected e.g. by using location or ranging services or e.g. through a non-3GPP connection between the two devices. Detection of proximity by the UEs or the network may be done with low frequency (e.g. once per 5 minutes) to save energy, or proximity changes may only be reported by the UE(s) when the proximity changes are persistent, e.g., when the devices are stationary or detected not to be close anymore.

Operator A uses a cellular system that provides enhanced QoS and improved service availability for cooperating UEs.

#### 8.x.3	Service Flows

1.	Rob decides to go for a bike tour, leaving from an urban area where good coverage is available and going towards a rural area, where coverage is worse. Before starting to ride, Rob shares a link with his friend Anne to keep streaming his position from his smart glasses and sending alerts in case of an accident.

2.	Rob is talking to his friend Anne, while cycling towards an area where the coverage of his smart glasses is getting worse. Similarly, the coverage of the smart phone is getting worse, but because it has a stronger transmitter and more capable antenna configuration the signal has not deteriorated so much as for the smart glasses. In order to prolong the service availability and overall reliability for both UE, the cellular system coordinates the cooperation between the UEs by (1) routing or duplicating part of the communication via the smart phone, or by (2) coordinating e.g. the mobility procedures of both UEs by using the stronger signal of the mobile phone for more reliable communication with both UEs. The cellular system may use measurements, UE capability information, proximity information and other information about the UEs to configure the UEs to trigger/perform the coordination.

3.	After cycling 30 Km Rob takes a break, and he would like to share a video he made with his friend Anne. The battery of his glasses is at a very low level, the coverage is poor, and the video is bulky. However, the smart phone can cooperate in the data transfer to increase reliability and performance, e.g. using cooperative transmission or by transmitting (a subset of) the data via direct and/or indirect network connection. The cellular system coordinates the cooperation between UEs to perform the data transfer via both the smart phone and smart glasses increasing the reliability and speed of the data transfer so that Rob can share the video timely.

#### 8.x.4	Post-conditions

Rob enjoys high quality of service when making a call and sharing the video.

#### 8.x.5	Existing features partly or fully covering the use case functionality

TS 22.261 [14] has several requirements related to optimizing performance for multiple UEs, however, these requirements are more related to broadcasting downlink data and group communication, not related to cooperation/coordination between UEs to optimize QoS and service availability. For example, the following requirements have been specified in clauses 6.2.2, 6.4.2.2, 6.40.2.1, 6.43.2 of TS 22.261 [14]:

The 5G network shall allow operators to optimize network behaviour (e.g. mobility management support) based on the mobility patterns (e.g. stationary, nomadic, spatially restricted mobility, full mobility) of a UE or group of UEs.

…

The 5G network shall optimize the resource use of the control plane and/or user plane to support bulk operation for high connection density (e.g. 1 million connections per square kilometre) of multiple UEs.

The 5G system shall support a timely, efficient, and/or reliable mechanism to transmit the same information to multiple UEs.

…

The 5G system shall support the relaying of traffic between a remote UE and a gNB using one or more relay UEs.

The 5G system shall support same traffic flow of a remote UE to be relayed via different indirect network connection paths.

The 5G system shall support different traffic flows of a remote UE to be relayed via different indirect network connection paths.

The connection between a remote UE and a relay UE shall be able to use 3GPP RAT or non-3GPP RAT and use licensed or unlicensed band.

…

The 5G system shall be able to expose aggregated QoS parameter values for a group of UEs to an authorized service provider.

…

The 5G system shall enable an authorized 3rd party to provide policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

The 5G system shall support a means to apply 3rd party provided policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

NOTE:	The policy can be used by a 3rd party application for coordination of the transmission of multiple UEs’ flows (e.g., haptic, audio and video) of a multi-modal communication session.

TS 22.261 [14] clause 6.9.2 further defines requirements for relay communication, such as:

…

The 5G system shall support the relaying of traffic between a remote UE and a gNB using one or more relay UEs.

The 5G system shall support same traffic flow of a remote UE to be relayed via different indirect network connection paths.

The 5G system shall support different traffic flows of a remote UE to be relayed via different indirect network connection paths.

The connection between a remote UE and a relay UE shall be able to use 3GPP RAT or non-3GPP RAT and use licensed or unlicensed band.

The purpose of relay communication is to provide indirect network connection to out-of-coverage UEs. This is different from two UEs having a direct connection being coordinated by the network to improve QoS/QoE and communication availability.

As specified in TS 38.300 [221] clause 16.21, a non-3GPP connection may exists between UEs in case of an indirect connection path, and the presence of such non-3GPP connection is conveyed to the base station. This implies that the the base station is aware that the two devices are in close proximity of each other without having to rely on 3GPP location or ranging services.

TS 38.300 [221] also specifies Multi-TRP operation for 5G NR (similar to CoMP in LTE). However, multi-TRP operation involves only describes cooperative transmission by a group of base stations, not by a group of UEs.

#### 8.x.6	Potential New Requirements needed to support the use case

[PR 8.x-1] Subject to operator policy, the 6G network shall support a means for coordinating cooperating UEs to enhance QoS/QoE and service availability for one or more of the cooperating UEs of communication with the 6G network, compared to the QoS/QoE and service availability of communication by each UE individually.

NOTE: the cooperating UEs are all expected to have a direct connection with the network.

[PR 8.x-2] Subject to operator policy and user permission, the 6G network shall allow configuring the addition and removal of a UE to a set of cooperating UEs.

Editor’s Note: the term “user permission” is FFS.

* * * End of Change * * * *