3GPP TSG SA WG 1 Meeting #112 	S1-254302r2

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254231)

Source:	Huawei, HiSilicon, China Mobile

pCR Title:	Pseudo-CR on TTFT and its importance for AI inference latency

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.3.2

Document for:	Approval

Contact:	Shuang ZHANG, zhang.shuang2 AT Huawei DOT com

Abstract: Add clarification to the AI inference latency aspects captured in TR22870 v.041.

1. Introduction

Introduce the definition of E2E TTFT (i.e. round-trip communication latency, and TTFT in Service Hosting Environment) in SA1. In the meantime, complementing AI inference latency requirements (based on agreed content in TR22870 v.040) with E2E TTFT being required to be less than 200ms, in order to meet the requirement of basic human brain reaction time.

Details refer to discussion paper S1-254230.

2. Reason for Change

TTFT reflects the time for the AI system's first response, i.e., the system's response time. Within the scope of TR 22.870, use cases and requirements are captured related to AI applications based on AI models dealing with input incl. images/videos/audios (e.g. visual models /VM, multi-modal language models /MLM, visual language action models /VLA), and the corresponding performance requirements are captured, i.e. Joint E2E latency (ms), which is describes in various NOTEs (e.g. KPI table Table 6.10.6-1, Table 6.49.6-1) as:

/// excerpt from TR 22.870, v0.4.1 ////

Joint E2E latency (i.e. round-trip communication latency, and AI inference latency in Service Hosting Environment). and UE is only considered to contribute to the communication service latency.

//// end of excerpts ///////////////////////

The common aspect these use cases/AI models share is the AI inference result being only one token. Therefore:

joint E2E latency (ms) = round-trip communication latency + TTFT

It is important to capture this clarify in the stage1 performance requirement of “Joint E2E latency (ms)” = round-trip communication latency + TTFT.

3. Conclusions

Propose to define E2E TTFT for AI inference service latency, and enhance the related KPI parameter with TTFT.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v041.

Change in r2:

Based on offline suggestions, adding the text in green for better clarify: “the first output token fulfils the AI inference request”

* * * First Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

//// skip ///

joint E2E latency: the performance parameter for 6G AI Service when performing AI inference, comprised of round-trip communication latency (e.g. communication latency for UE transferring inference input to AI model in Service Hosting Environment, as well as the communication latency for transferring inference result from the AI model back to the UE), plus the AI inference latency (e.g. TTFT) inside the Service Hosting Environment.

Time to First Token (TTFT): the time an AI model takes to generate the first token from the moment the inference input is received.

NOTE: It reflects how fast the AI model can start responding, which includes the tokenization of input prompts and the model's initial computations (forward propagation) to generate the first output token.

* * * Second Change * * * *

### 6.10.6	Potential New Requirements needed to support the use case

[PR 6.10.6-1] Subject to operator’s policy, the 6G network shall be able to provide AI service (e.g. AI model inference) to a UE.

[PR 6.10.6-2] Subject to operator’s policy, the 6G system shall be able to support negotiation of the service performance (e.g. latency, inference accuracy), between UE and 6G network, when providing AI service (e.g. AI model inference).

[PR 6.10.6-3] Subject to operator’s policy, the 6G network shall be able to support mechanism to guarantee the service performance (e.g. latency, inference accuracy) when providing AI service (e.g. AI model inference).

[PR 6.10.6-4] Subject to operator policy and user consent, the 6G network shall support mechanisms for scheduling and provisioning network resources and computing resources in Service Host Environment for distributed AI inference service.

[PR 6.10.6-5] Subject to operator policy, the 6G network shall support mechanisms to ensure a guaranteed service performance of the distributed AI inference service.

NOTE:	The UE is not involved in distributed AI inference service.

[PR 6.10.6-6] The 6G system shall be able to provide the communication and AI service for a user with the KPI requirements in Table 6.10.6-1 below.

Table 6.10.6-1: KPIs for home robot perform perception for general task in open environments via 6G network

| Use case | Traffic type | Average packet size (Byte) | Transfer interval (ms) | Data Rate (Mbps) | Joint E2E latency (ms) (NOTE 6) | Reliability |
| --- | --- | --- | --- | --- | --- | --- |
| home robot perceiving overall context of the scene (NOTE 5) | UL camera data (NOTE 1) | [<1000] | [10] | [20-60] (NOTE 2) | [150] (NOTE 3) | [99.9 %] |
| for identifying individual objects in the scene (NOTE 5) | UL camera data (NOTE 1) | [<1000] | [10] | [20-60] (NOTE 2) | [200-300] (NOTE 4) | [99.9 %] |
| NOTE 1: 6 RGB cameras are equipped for robot “Figure 02” [180]. NOTE 2: 20 Mbps is for six 1080p cameras, and 60 Mbps is for four 1080p cameras plus two 4K cameras. Data Rate is calculated from video resolution*(Bits per colour*3) *Refresh rate/Compression ratio. Compression ratio of 240 and 8 bits per colour is assumed, thus data rate is around 3 Mbps for 1080p 15Hz, and around 24Mbps for 4K 30Hz. Compression ratio, bits per colour and refresh rate for 1080p refers to the similar cases for real time video uploading of a vehicle as per YD/T 4778-2024 [182]. NOTE 3: Based on psychophysical and neurophysiological studies [281], [282], human beings can perceive the gist (e.g. overall meaning) of complex visual scenes within around 150ms after stimulus onset. This rapid initial perception enables us to grasp the general context of a scene, even if the stimulus is briefly presented (around 10ms). NOTE 4: For human beings, it takes longer for a human being to identify individual objects, e.g. finer object identification requires larger than 200 ms [283] and around 300ms [284]. NOTE 5: In the target scenarios, robots are expected to have similar perception time as an average human being. NOTE 6: Joint E2E latency (i.e. round-trip communication latency, and AI inference latency in Service Hosting Environment), and UE is only considered to contribute to the communication service latency. NOTE 7: In this use case, due to the AI models being used for inference, AI inference latency equals TTFT. For example, certain VLA AI models (e.g.π0) inherently only take one forward propagation per inference cycle, and the first output token fulfils the AI inference request. |  |  |  |  |  |  |

* * * Third Change * * * *

### 6.49.6	Potential New Requirements needed to support the use case

[PR 6.49.6-1] The 6G system shall be able to provide the communication and AI service (assuming AI inferencing in Service Hosting Environment) for users with the KPI requirements in Table 6.49.6-1 below.

Table 6.49.6-1: KPIs for physical AI robot to perform cognitive functions to interact with a human user via 6G network

| Use case | Traffic type | Average packet size (Byte) | Transfer interval (ms) | Data Rate (Mbps) | Joint E2E latency (ms) (NOTE 4) | Reliability |
| --- | --- | --- | --- | --- | --- | --- |
| Support robot conscious awareness for interacting with human user | UL camera data (NOTE 1) | [<1000] | [10] | [20-60] (NOTE 2) | [<200] (NOTE 3) (NOTE 5) | [99.9 %] |
| NOTE 1: 6 RGB cameras are equipped for robot “Figure 02” [180]. NOTE 2: 20 Mbps is for six 1080p cameras, and 60 Mbps is for four 1080p cameras plus two 4K cameras. Data Rate is calculated from video resolution*(Bits per colour*3) *Refresh rate/Compression ratio. Compression ratio of 240 and 8 bits per colour is assumed, thus data rate is around 3 Mbps for 1080p 15Hz, and around 24Mbps for 4K 30Hz. Compression ratio, bits per colour and refresh rate for 1080p refers to the similar cases for real time video uploading of a vehicle as per YD/T 4778-2024 [182]. NOTE 3: For physical AI robot interacting with a human user, the robot is expected to mimic the similar basic human brain reaction time including conscious awareness/recognition and decision-making based on various stimuli. Such human brain reaction time ranges from mean auditory reaction time 140-160ms, touch 155ms, to visual reaction time 180-200ms [279]. NOTE 4: Joint E2E latency (i.e. round-trip communication latency, and AI inference latency in Service Hosting Environment). and UE is only considered to contribute to the communication service latency. NOTE 5: The human to robot latency (vice versa) is not included. NOTE 6: In this use case, due to the AI models being used for inference, AI inference latency equals TTFT. For example, certain VLA AI models (e.g.π0) inherently only take one forward propagation per inference cycle, and the first output token fulfils the AI inference request. |  |  |  |  |  |  |

* * * End of Change * * * *