3GPP TSG-SA WG1 Meeting #60 	 S1-124075

# Edinburgh, UK, 12th – 16th November 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.228 | CR | 0175 | rev | - | Current version: | 12.3.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for RTCWeb client access to IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMS_RTCWeb |  |  |  |  |  |  | Date: |  | 2012-10-24 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | RTCWeb is a free, open project that enables web browsers with Real-Time Communications (RTC) capabilities via simple Javascript APIs. IETF and W3C make joint efforts on related architecture, protocols and interfaces. Normally, browser based client access IMS through SIP-plugins. The user experience is poor because of the installation of plugins often failed, such as configuration error and version error etc. However, RTCWeb client is plugin-free, enabling us to deliver the service without any installation in the first place with user-transparent upgrading capability on a continuous basis, resultant in ever improving user experience. Based on the suggestion of protocol selection from IETF, RTCWeb client will use HTTP/Websocket protocol to communicate. Based on the suggestion of codecs selection from IETF, RTCWeb client will use Opus and G.711 as ‘mandatory to implement’ audio codecs, and so far the video codec selection are TBD. However, IMS does not support clients speaking in HTTP/Websocket, and usually does not support Opus codec. So it needs to enhance the capability of IMS to support protocol and media translation for RTCWeb client access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The capability of supporting RTCWeb client access to IMS is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#####First change#####

7	User service requirements

7.16	Support for Push Notification Service in IMS

The IMS shall be able to support SIP based Push Notification Service as defined in [28].

7.17	Support for RTCWeb client access to IMS

RTCWeb client is a browser based client with Real-Time Communications (RTC) capabilities. IMS shall be able to provide the capabilities as specified to support RTCWeb client access.

-	Shall be able to support RTCWeb clien to use SIP-like signalling protocol over HTTP/Websocket to access.

-	Shall be able to support RTCWeb client to communicate with a IMS client, both for originating and terminating calls.

-	Shall be able to support RTCWeb client to realise most IMS services, such as presence, group, conference etc.

Note: The implementation of the RTCWeb client is to be considered out of scope for 3GPP.

#####End change#####