3GPP TSG-SA1 Meeting #93e	S1-210020

Electronic Meeting, 22 February - 04 March 2021	(revision of S1-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | XXXX | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ranging based service requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  |  |  |  | Date: |  |  | 2021-02-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Ranging based service WID has been agreed, the new requirements for ranging based service need to be captured in TS22.261. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new section including the general description and functional requirement for ranging based services are added in chapter 6. A new section including KPIs for Ranging based services is added in chapter 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x, 7.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************   Start first modification  ***************

3.1 	Definitions

…

Ranging: same as Ranging service, refers to the determination of the distance between two UEs and/or the direction of one UE from the other one via direct communication connection.

******************   End first modification  ***************

******************   Start second modification  ***************

6.x 	Ranging based services

6.x.1	Description

Ranging-based services are the applications utilizing the distance between two UEs and/or the direction of one UE from the other one. In 3D case, direction includes horizontal direction and elevation direction. Ranging-based services can be applied in a variety of verticals, such as consumer, smart home, smart city, smart transportation, smart retail, and industry 4.0. Some Ranging-based services may only require the distance measurement, some may only require direction measurement, the others may require both distance and direction measurement. In case both distance and direction are required, although distance and direction can be used to calculate relative position, what is different from relative position is that it does not necessarily require the accuracy of distance and direction are in the same level.

Ranging service can be supported with or without 5G coverage, i.e. in coverage, partial coverage and out of coverage. If licensed band is used for ranging, it shall be fully under operator control.

6.x.2	Requirements

The 5G system shall be able to support for a UE to discover other UEs supporting Ranging.

The 5G system shall be able to authorize Ranging for each individual UE.

The 5G system shall be able to protect privacy of a UE and its user, ensuring that no identifiable information can be tracked by undesired entities during Ranging.

The 5G system shall be able to enable or disable the Ranging service.

The 5G system shall support mutual ranging, i.e. the two UE’s involved shall be able to range each other.

The 5G system shall be able to ensure that the use of ranging, if in licensed spectrum, is only permitted in network coverage under the full control of the operator who provides the coverage.

The 5G system shall support energy efficient UE ranging services and operation.

******************   End second modification  ***************

******************   Start third modification  ***************

7.x 	KPIs for Ranging based services

In several scenarios, it can be beneficial to determine the distance between two UEs and/or the direction of one UE from the other one via direct communication connection. The functional requirements related to Ranging based services can be found in clause 6.x. Performance requirements for Ranging based services in different scenarios can be found in table 7.x-1.

Key performance indicators and key attributes for Ranging are defined as follows:

-	Ranging accuracy: describes the closeness of the measured position of the target UE to its true position value. It can be further divided into distance accuracy and direction accuracy (See the Annex).

-	Confidence level: describes the percentage that true position is in the confidence interval considering the Ranging accuracy.

-	Effective Ranging distance: the largest distance between the UE who initiates the Ranging and target UEs in the Ranging operation.

-	Environment of use: the physical environment between the UE who initiate the Ranging and target UEs, such as LOS environment and NLOS environment. Also the physical environments of the UE who initiate ranging, such as in coverage and out of coverage.

-	Relative UE velocity: the target UE can be either static or mobile relative to the UE who initiates the Ranging. In the latter, the attribute shall also provide some elements about its motion, e.g. maximum speed, trajectory.

-	Availability: percentage of time when a ranging system is able to provide the required Ranging-related data within the performance targets or requirements.

-	Latency: time elapsed between the event that triggers the determination of the Ranging-related data and the availability of the Ranging-related data at the Ranging system interface.

-	Power consumption: electrical power used by Ranging during Ranging operation.

-	Ranging interval: number of Ranging operations during a given time unit.

Table 7.x-1: Performance requirements for Ranging based services

| Ranging service level | Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| 1 | Distance based smart home TV control | 10cm up to 3 meter separation | ±2° horizontal direction accuracy at 0.1 to 3 meter separation and AoA coverage of (-60°) to (+60°); ±2° Elevation direction accuracy at 0.1 to 3 meter separation and AoA coverage of (-45°) to (+45°) | 99 % | 10ms | 10m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | - | - |
| 2 | Picture and video sharing based on ranging results | 10cm | 2° | 99 % | 50ms | 10m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | - | - |
| 3 | Distance based smart device control | 10cm | - | 99 % | 50ms | 20m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | 20 | - |
| 3 | Smart Vehicle Key | 10 cm | - | 99 % | 50ms | 30m | IC/PC/OOC | - | Static/ Moving (<2m/s) | - | - | 50UEs/ (104m2) |
| 3 | Touchless Self-checkout Machine Control | 10cm | - | 99% | 50ms | 1m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | - | - | = |
| 3 | Hands Free Access | 10cm | - | [99 %] | 100ms | 10 m | IC/PC/OOC | - | Static/ Moving (1 m/s) | - | - | 20 UEs/3.14*100m2 |
| 3 | Smart Transportation Metro/Bus Validation | 10cm | - | 99 % | 50ms | 2m | IC/PC/OOC | LOS | Static/ Moving (3km/h) | 50ms | 20 | 100 in the area of 8 m2 |
| 3 | Ranging of UE’s in front of vending machine | 20cm | 10° | - | 150ms | 5m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | 50ms | - | 10 |
| 3 | Search and rescue | 20m | 5° | 99 % | 1s | 100m-1km | IC/PC/OOC | LOS | Static/ Moving (up to 10m/s) | 5s | - | - |
| 3 | Finding Items in a supermarket | 50 cm | 5 degree | 95 % | 500ms | 100m | IC/PC/OOC | - | Static/ Moving (<1m/s) | - | - | 100 UEs/ (3.14*104m2) |
| 4 | distance based intelligent perception for public safety | 50cm | - | 99 % | - | 20m | IC/PC/OOC | LOS | Static/ Moving (<20km/h) | - | 100 | - |

******************   End third modification  ***************