3GPP TSG-SA WG1 Meeting #98e 	S1-221070r2

Electronic Meeting, 9 – 19 May, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.874 | CR | 0009 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use case of direct device connection assisted Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AIML_ph2 |  |  |  |  |  | Date: |  |  | 2022-4-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add a new use case for FS_AIML_Ph2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Using Direct Device Connection to assist FL (i.e. get more UE members involved for FL task) so that the FL performance can be increased. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new use case is not added |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change *********

2	References

[……]

[xx]	M. Chen, K. Huang, W. Saad, M. Bennis, A. V. Feljan, and H. V. Poor, “Distributed Learning in Wireless Networks: Recent Progress and Future Challenges”IEEE JOURNAL ON SELECTED AREAS IN COMMUNICATIONS, VOL. 39, NO. 12, DECEMBER 2021

[xy]	M. Chen, H. V. Poor, W. Saad, and S. Cui, “Wireless communications for collaborative federated learning,” IEEE Commun. Mag., vol. 58, no. 12, pp. 48–54, Dec. 2020

********** Second Change *********

11.x	Direct device connection assisted Federated Learning

11.x.1	Description

In many circumstances, an application server holding a Federated Learning (FL) task has a transmission delay requirement and limited FL coverage. An FL coverage means an area in which UEs the Application server can organize for federated learning.

the Application server has a transmission delay requirement for each FL member (UE). Some of UEs are actually holding valuable dataset but cannot fulfil transmission delay requirement, which lead to a decreasing of FL performance. However, if a UE’s direct network connection cannot fulfil the transmission delay requirement (i.e. an QoS on Uu), using direct device connection helps to involve more UEs holding valuable dataset for the FL task:

A UE-A with bad transmission condition interact with Application server via PC5 relay (the relay UE-B need not be a FL member). It helps to fulfil the transmission requirement. In such way, the total transmission latency depends on the delay of direct device connection + direct network connection.

It is an obvious method that a UE with bad condition leverage direct device connection-based relay to acquire the expected transmission latency so as to finish the FL task [11] [14].

11.x.2	Pre-conditions

Figure-1

As depicted in Figure-1, there is an Application server for federated learning which need to communicate the UEs in a FL coverage for FL task.

To achieve an ideal performance (i.e. fast convergence and high model accuracy), there is a transmission latency requirement to each FL member UE’s data transmission.

Alice and Bob are FL members but Alice’s direct network connection cannot fulfil the latency requirement.

Carl is not a FL member, but his cell phone has a good signal for direct network communication. And Carl is also willing to provide relay service for the neighbouring cell phones.

Alice, Bob and Carl are neighbouring to each other within a FL coverage.

11.x.3	Service Flows

- Alice established a direct network connection in 5G network however it cannot fulfil the transmission latency as requested by application server due to she is in a bad coverage.

- Alice discovers Carl, who is neighbouring to Alice, is willing to be act as a relay for the FL related data transmission. Thus, Alice requests Carl to establish a direct device connection-based relay to the 5G network so that Alice can acquire the training model and upload training result to application server via Carl’s cellophane.

- Alice can communicate with application server to perform the FL task fulfilling the communication latency requirement.

11.x.4	Post-conditions

With leveraging direct device connection, it helps Application server to involve more UEs to join the FL task so that to optimize the FL performance.

By leveraging direct device connection based relay, Alice can join the FL task with the transmission latency requirement.

11.x.5	Existing features partly or fully covering the use case functionality

22.261 v18.6.0 6.40.2

Based on operator policy, the 5G system shall be able to provide means to allow an authorized third-party to monitor the resource utilisation of the network service that is associated with the third-party.

NOTE 1:	Resource utilization in the preceding requirement refers to measurements relevant to the UE’s performance such as the data throughput provided to the UE.

Based on operator policy, the 5G system shall be able to provide an indication about a planned change of bitrate, latency, or reliability for a QoS flow to an authorized 3rd party so that the 3rd party AI/ML application is able to adjust the application layer behaviour if time allows. The indication shall provide the anticipated time and location of the change, as well as the target QoS parameters.

Based on operator policy, 5G system shall be able to provide means to predict and expose predicted network condition changes (i.e. bitrate, latency, reliability) per UE, to an authorized third party.

Subject to user consent, operator policy and regulatory constraints, the 5G system shall be able to support a mechanism to expose monitoring and status information of an AI-ML session to a 3rd party AI/ML application.

NOTE 2:	Such mechanism is needed for AI/ML application to determine an in-time transfer of AI/ML model.

Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to expose information (e.g. candidate UEs) to an authorized 3rd party to assist the 3rd party to determine member(s) of a group of UEs (e.g. UEs of a FL group).

11.x.6	Potential New Requirements needed to support the use case

Functionality Requirements:

[P.R.5.x-001] Subject to user consent and operator policies, the 5G system shall support a UE-a to discover another UE-b for direct device connection-based relay so that the data transmission latency required by FL application can be fulfilled.

NOTE: how the transmission latency is split to direct device connection and direct network connection is by implementation.

KPI requirement for direct device communication

[P.R.5.x-002] The 5G system shall support the following KPI as defined in Table 5.x.6-1

NOTE: The table above refers the same AI/ML model which has been discussed in clause 7.1 for FL for image recognition (i.e. a 7-bit CNN model VGG16_BN using 2242243 images as training data). The transmission requirement of the FL model can also apply to the total latency of direct device connection + direct network connection (i.e. PC5 + Uu).

Table 5.x.6-1: Latency and user experienced UL/DL data rates for uncompressed Federated Learning

| Mini-batch size (images) |  | Maximum latency for trained gradient uploading and global model distribution (see note 1) | User experienced UL/DL data rate for trained gradient uploading and global model distribution (see note 2) |
| --- | --- | --- | --- |
| 64 |  | 3.24s | 325Mbit/s |
| 32 |  | 1.9s | 55Mbit/s |
| 16 |  | 1.3s | 810Mbit/s |
| 8 |  | 1.1s | 960Mbit/s |
| 4 |  | 1.04s | 1.0Gbit/s |
| NOTE 1: Latency in this table is assumed 20 times the device GPU computation time for the given mini-batch size. NOTE 2: Values provided in the table are calculative needs for an 8-bit VGG16 BN model with 132MByte size, given mini-batch sizes and a duration of [FFS] seconds per iteration. Necessary user experience UL/DL data rates can be reduced by e.g. setting longer times per iteration, applying compressed FL, or using another AI/ML model. |  |  |  |

Editor's Note:	it is FFS whether other potential requirements will be identified.

*******************End of Change*****************