3GPP TSG-SA WG1 Meeting #98e 	S1-221103r1

Electronic Meeting, 9 – 19 May 2022	(revision of S1-22xxxx)

Source:	vivo, China Unicom

pCR Title:	Pseudo-CR on use case of mobility for non-N2 shared network

Draft Spec:	3GPP TR 22.851v0.0.0

Agenda item:	7.5

Document for:	Approval

Contact:	Yanchao Kang <kangyanchao@vivo.com>

Abstract: introduce the mobility scenarios and potential requirements for the non-N2 shared network in TR22.851.

1. Introduction

Study FS_netshare aims to study use cases and potential requiremnts for the non-N2 shared network.

2. Reason for Change

Mobility scenarios and potential requirements needs to be discussed and agreed for the non-N2 shared network.

3. Conclusions

none

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.851.

* * * First Change * * * *

5.A.1	Description

It is worth mentioning that 5G networks have been designed to provide shared facilities at the beginning of the deployment. This means that if the 4G network has independent E-UTRAN coverage and shared E-UTRAN coverage at the same time, there will be more than one operator's wireless access technology simultaneously supporting coverages:

- Independent E-UTRA coverage,

- Shared E-UTRA coverage,

- Independent 5G NR coverage,

- Shared 5G NR coverage.

Therefore, interoperability with existing technologies needs to be considered when introducing non-N2 network sharing.

5.A.2	Pre-conditions

1. It is assumed that, OP1’s home network uses the same PLMN-ID for both 4G and 5G; and OP2’s home network uses same PLMN-ID for both 4G and 5G network.

NOTE: The home 4G and 5G network of OP1 and OP2 may be deployed with independent and shared wireless access technology.

- OP 1 is Hosting RAN Operator, Only OP1’s 5G RAN is "shared" with Participating Operator 2.

- OP 2 is Participating Operator.

- UE subscribe to OP2’s home PLMN.

- UE may register successfuly to OP1’s shared 5G network.

- Both operators agree to share the network via non-N2 direct connection between the shared radio access network and the OP2's core network.

3. PLMN-ID broadcasted by the shared RAN is different from home PLMN-IDs of OP1 and OP2.

4. Potential scenario1: OP1 shared 5G network may overlap with OP2’s 4G home network; may also overlap with OP2’s 5G home network (i.e. at OP1’s border).

The following coverage scenarios are possible:

- OP1-5G (shared) only.

- OP1-5G (shared) + OP2-4G (e.g. OP1 5G preferred by UE of OP2).

- OP1-5G (shared) + OP2-5G (e.g. OP2-5G preferred by UE of OP2).

- OP2 5G only.

NOTE:  if UE preferred OP1’s 5G, OP2 UE should stay on OP1’s shared network, unless OP2’s 5G is available.

- Potential scenario2: OP1 shared 5G network may overlap with OP2’s 4G home network (i.e. at OP1 border); may also overlap with OP2’s 5G home network (i.e. at OP1 border).

NOTE: The overlap between OP2’s 4G and OP1’s 5G home network happen in the border.

5.A.3	Service Flows

- OP2_4G

- OP2_5G

- OP2_5G

- OP1_5G

- OP1_5G

- OP1_5G

②

②

①

①

Figure 5. A.3-1: Non-N2 shared network mobility scenario

The UE moves between the OP1’s shared 5G network and the OP2’s home network.

NOTE 1: OP1_5G is OP1’s shared 5G network, via non-N2 direct connection between the shared radio access network and the OP2's core network.

NOTE 2: OP2_5G is OP2’s home network, may be MOCN networks or independent network.

NOTE 3: OP2_4G  is OP2’s home network, may be a MOCN network or independent network.

Scenario 1: shared (OP1) 5G   - >  home (OP2) 5G

-UE is registered in OP1 and get some 5G services, and moves from shared network to home 5G network. As shown as ① in figure, that is OP1_5G(shared)  - > OP2_5G.

Scenario 2: shared (OP1) 5G - > home (OP2) 4G

-UE is registered in OP1 and get some 5G services, and moves from shared network to home 4G network. As shown as ② in figure, that is OP1_5G(shared)  - > OP2_4G.

Scenario 3: home (OP2) 4G - > shared (OP1) 5G

-UE is registered in OP2 and get some 4G services, and moves from home 4G network to shared network. As shown as ② in figure, that is OP2_4G - >  OP1_5G(shared).

Scenario 4: home (OP2) 5G - > shared (OP1) 5G

-UE is registered in OP2 and get some 5G services, and moves from home 5G network to the shared network. As shown as ① in Figure, that is OP2_5G - >  OP1_5G(shared) .

More details of mobility is shown in the table 5.A.3-1.

Table 5.A.3-1: Mobility for non-N2 network sharing

| Scenario | 1 (OP1-5G shared - > OP2-5G) | 2 (OP1-5G shared- > OP2-4G) [no OP2-5G] | 3 (OP2-4G - > OP1-5G shared) [no OP2-5G] | 4 (OP2-5G - > OP1-5G shared) |
| --- | --- | --- | --- | --- |
| a) Inter-PLMN selection | Yes | Yes | Yes | Yes |
| b) Inter-PLMN cell reselection | Yes (via ePLMN + I-freq priority) | Yes (via ePLMN), only at OP1 border | Yes (via PLMN + IRAT priority) | Yes (via ePLMN), only at OP2 border |
| c) Inter-PLMN handover | Yes (via ePLMN) only at OP2 border | Yes (via ePLMN), | Yes (via ePLMN) | Yes (via ePLMN), only at OP2 border |
| NOTE: PLMN-ID broadcasted by the shared RAN is different from home PLMN-IDs of OP1 and OP2 |  |  |  |  |

5.A.4	Post-conditions

5.A.5	Existing feature partly or fully covering use case functionality

SA1 has performed various studies on mobility and network sharing in previous releases, where related normative stage 1 requirements are introduced in 3GPP TS 22.101 [x1] and 22.261[x3].

3GPP TS 22.261 [x3] introduces requirements of Diverse mobility management, stated as follows:

The 5G system shall support inter- and/or intra- access technology mobility procedures within 5GS with minimum impact to the user experience (e.g. QoS, QoE).

3GPP TS 22.101 [x1] introduces requirements of mobility of network sharing, stated as follows:

It shall be possible to support different mobility management rules, service capabilities and access rights as a function of the home PLMN of the subscribers.

The above requriements are based on MOCN.

5.A.6	Potential New Requirements needed to support the use case

[PR 5.A.6-001] Mobility shall be supported if a user crosses the border between the shared network managed by Hosting operator and the Participating Operator’s home network.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>