SA WG2 Meeting #S2#106	S2-143938

17 - 21 November 2014, San Francisco, USA	(revision of S2-143059)

Source:	Ericsson

Title:	Scenario for coordination of max latency between the application and the network

Document for:	Approval

Agenda Item:	7.17

Work Item / Release:	FS_HLCom / REL-13

Abstract of the contribution: Proposes a scenario for optimization

***** First Change *****

## 4.z	Scenario z: Coordination of max latency between the application and the network

Editor’s Note: Description of a scenario where optimization is considered and motivation for the optimization.

### 4.z.1	Description

Different MTC applications have different max latency requirements. “Max latency” is from the application point of view the time from that an application residing in an application server initiates communication with a 3GPP constrained device until the device has received the communication from the application. Contributors to the latency are for example PSM, long DRX and low bitrate bearers. Generally “Max latency” should also relate to the opposite direction when the constrained device initiates communication with an application server.

The max latency requirement is highly application dependent. For example for a device that measures and controls a water flow, a response time (i.e. max latency) of 1 hour may be sufficient for increasing or reducing the water flow. For another device that measures and controls a gas flow, a response time of 1 minute may for example be required in order to switch off the gas in case of emergency. This max latency requirement the application has need to be communicated to the network. The network needs this max latency information to set parameters correctly for the different affected mechanisms (see the list below) e.g. the PSM.

Potential Solutions for this Scenario should state if they are applicable to multi-application environments or not. And how coordination of max latency is done if multiple application environments are supported.

Application

IP connection

Application

UE

Application Server

[Property: Max latency requirement]

Figure 4.z-1:   An IP connection with a certain max latency property between two application peers

### 4.z.2	General considerations for solutions to this scenario

This scenario assumes that it is possible to change the max latency. Applications may also need to change the current max latency, e.g. based on application state or changed conditions, with the overall goal to minimize power consumption. For constrained devices having multiple applications with different max latency requirements, it is assumed that the lowest max latency is applied for the common IP connection.

***** End of Changes *****