SA WG2 Meeting #S2#106	S2-143943

17 - 21 November 2014, San Francisco, USA	(revision of S2-14xxxx)

Source:	Ericsson

Title:	Architecture Aspects

Document for:	Approval

Agenda Item:	7.6

Work Item / Release:	DECOR / REL-13

Abstract of the contribution: Clarifying architecture aspects of Dedicated Core Networks

## Discussion

The following updates should be needed:

- The usage of the terms “user plane nodes” and “control plane nodes” in clause 4.2 are not clear. A SGSN is for example both a control plane node and a user plane node. It is proposed to use the node names. Also PCRF is added as a node to be considered for Dedicated CN.

- A paragraph describing and clarifying the context of the architecture requirements is added in clause 4.3. The purpose with a dedicated core network is to provide specific characteristics and/or functions for a category or group of UEs or subscribers.

- The architecture requirement is clarified to allow dedicated core networks to be used to provide specific characteristics and/or functions for specific categories or groups of UEs or subscribers.

***** First Change *****

# 4	Assumptions and Architecture Requirements

## 4.1	General

Editor’s Note: This section contains general definition if needed.

## 4.2	Assumptions

-	A dedicated core network is assumed to serve only specific subscriber(s).  A dedicated core network comprises one or more SGSN/MME and it may comprise one or more SGW/PGW. When the PGW are part of a dedicated core network, the PCRF may be shared or specific for the dedicated core network.

Editor’s Note: DÉCOR impacts on PCC are FFS

-	If the network deploys dedicated core network selection based on both LAPI indication and subscription information (i.e. Décor feature), then dedicated core network selection based on subscription information by MME/SGSN overrides selection based the LAPI-indication by RAN.

## 4.3	Architecture Requirements

The purpose with a dedicated core network is to provide specific characteristics and/or functions for a category or group of UEs or subscribers, e.g. M2M subscribers, subscribers belonging to a specific enterprise or separate administrative domain, etc.

The following architecture requirements shall be met by all proposed solutions:

-	There shall be no impacts to UE.

-	Support of dedicated core networks shall not require the use of additional PLMN-IDs, e.g. to separate different dedicated core networks.

-	It shall be possible to deploy multiple dedicated core-networks sharing the same RAN.

-	The architecture should support dedicated core networks for specific categories or groups of UEs or subscribers.

-	The architecture should support scenarios where the dedicated core-network is only deployed in a part of the PLMN e.g. only one RAT or in a part of the PLMN area. Each dedicated MME/SGSN supports the décor related functionality over its entire service area.

-	Mechanisms shall be supported to enable the redirection of attached UEs to appropriate dedicated core network, e.g. when a dedicated core network is commissioned in a PLMN.

-	If the dedicated core network is not deployed to serve a particular RAT or service area of PLMN, the UE in that RAT or service area is served by MME/SGSN from non-dedicated core network and may be served by user-plane network elements from the dedicated core network.

***** End of Changes *****