SA WG2 Meeting #S2-113	S2- 160751

25 - 29 January 2016, Frigate Bay, Saint Kitts and Nevis, KN

Source:	Nokia Networks, AT&T, Deutsche Telekom AG, China Mobile, Ericsson, Intel, Qualcomm Incorporated, ZTE, ETRI, KT

Title:	Key issue: QoS Framework

Document for:	Approval

Agenda Item:	NextGen/6.10

Work Item / Release:	NextGen/ Rel-14

Abstract of the contribution: This contribution propose to add a new key issue on QoS framework

# Introduction

NGMN Vision serves as a baseline for 5G architecture discussion and definition. According to NGMN vision in the NGMN white paper, “5G is an end-to-end ecosystem to enable a fully mobile and connected society. It empowers value creation towards customers and partners, through existing and emerging use cases, delivered with consistent experience, and enabled by sustainable business models”

5G is a system both for “Things”, i.e. cars, smart meters, machines, etc. and for human-centric devices and services.

Supporting Quality of Service, Quality of user experience is an important factor to ensure consistent experience with the next generation network architecture.

NGMN whitepaper lists several key requirements relevant to 5G System and Packet Core including a need for functional decoupling of Core and RAN network domains, a RAN technology agnostic architecture where introduction and connection of new radio technology will be possible in a plug and play manner and a need for independent evolution of core and RAN network domains. 4G Americas whitepaper has listed several requirements similar to it as well

- The 5G network architecture should be such that the air interface and the core network can evolve and scale independently of each other.

- Changes/enhancements to one domain should not mandate changes/enhancements to the other.

- 5G networks must also support multi-RAT connectivity efficiently and effectively.

- Provide access agnostic packet core across multiple Radio Technologies (e.g. cellular, Wi-Fi, etc.) to support uniform Authentication, Session Continuity, and Security across radio technologies.

- Provide Plug and Play capability where a new access technology may be attached to the packet core without any modifications.

Based on these requirements and 5G study item objectives to “minimize access dependencies” 5G CN should minimize access specific aspects. QoS is a key access network characteristic. Up until now signaling of QoS needs to the RAN has been access specific within 3GPP core network. This key issue will look into solutions that will make the QoS concepts of a NexGen core network general purpose to allow ease of reuse of packet core and to allow its independent evolution from various access technologies that it could support.

# Proposal

It is proposed to add the following Key Issue to the TR 23.799 “Study on Architecture for Next Generation System”.

* * * Start of changes * * * *

# 5	Key Issues

## 5.x	Key issue on QoS framework

Editor's Note: This clause will identify key architectural issues and the corresponding candidate solutions during the design of the next generation system architecture.

### 5.x.1	Description

This key issue should study the QoS framework for the system architecture i.e. the required functions (including both CP and UP) and a functional split between UE, Access Networks and CN, and any necessary QoS related signaling between those functions. The QoS framework that enables the operator to provide QoS for the wide range of use cases is expected to be fulfilled by the NextGen architecture.

The architecture should support QoS in such a manner that it fulfils the following principles:

- Solution for QoS framework should allow ease of reuse of Next Generation core and allow independent evolution of core and access technologies (i.e. 3GPP access, non-3GPP access).

- Solution for QoS framework within NexGen core network is not access specific.

- Solution for QoS framework enables optimal service level quality as per application needs, optimizing network capacity utilization.

- Add a note to state that network capability exposure.

- A QoS framework that can provide adequate QoS handling for services/flows whose characteristics have been explicitly provided to the 3GPP system as well as for services/flows whose characteristics have been implicitly deduced (not deduced) by the 3GPP system (e.g. by subscription). – Allot communication (Sebastian,

- Editor’s note: level of interworking between NextGen and EPS needs to be determined.

- Solution for QoS framework should identify proper granularities (e.g. per-UE, per-flow) of traffic aggregation and their QoS parameters (e.g. maximum bit rate, guaranteed bit rate, priority level).

NOTE: The need and the solutions for such inter system change between NextGen and EPS are expected to be studied in other key issues, e.g. those on migration and coexistence with legacy systems.

In terms of description the candidate solutions should clarify the following items:

-	Functions: the required functions (including both CP and UP) and a functional split between UE, Access Networks and CN.

-	Functional entities: indication of QoS control points, QoS enforcement points and the associated reference points.

-	QoS model: how is QoS handled on each reference point (e.g. per packet, per flow, aggregation of flows etc.)

-	QoS characteristics: the list of parameters (e.g. maximum bit rate, guaranteed bit rate, priority level) needed for the QoS framework.

-	QoS related information exchange: how is QoS information conveyed to entities where it is enforced (e.g. to UE, to RAN, or to user plane gateways).

-	Traffic separation: how is traffic separation achieved (in the core as well as on the CN-RAN interface) for QoS treatment; solutions shall also clarify the granularity of the traffic separation.

-	Traffic identification: how is traffic identified at the various QoS provisioning/enforcement points and at what level (e.g. per packet, per flow or as an aggregate of flow etc.)

* * * End of Changes * * *