3GPP TSG SA WG-2 Meeting #115 	S2-162590

Nanjing, China, 23-27 May 2016 	(revision of S2-16xxxx)

Source:	T-Mobile USA

Title:	New Work Item Description on Service Domain Centralization

Document for:	Approval

Agenda Item:	7.1

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 	Feasibility Study Service Domain Centralization

## Acronym: 	FS_SeDoC

## Unique identifier: 	--

## 1	3GPP Work Area

|  | Radio Access |
| --- | --- |
| X | Core Network |
|  | Services |

## 2	Classification of WI and linked work items

### 2.0	Primary classification

This work item is a …

| X | Study Item (go to 2.1) |
| --- | --- |
|  | Feature (go to 2.2) |
|  | Building Block (go to 2.3) |
|  | Work Task (go to 2.4) |

### 2.1	Study Item

| Related Work Item(s) (if any] |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 370025 | ICSRA | Derivative |
| 410135 | IMS_SCC-ICS | Derivative |

Go to §3.

### 2.2	Feature

| Related Study Item or Feature (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.3	Building Block

| Parent Feature (or Study Item) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

This work item is …

|  | Stage 1 (go to 2.3.1) |
| --- | --- |
|  | Stage 2 (go to 2.3.2) |
|  | Stage 3 (go to 2.3.3) |
|  | Test spec (go to 2.3.4) |
|  | Other (go to 2.3.5) |

#### 2.3.1	Stage 1

| Source of external requirements (if any) |  |  |
| --- | --- | --- |
| Organization | Document | Remarks |

Go to §3.

#### 2.3.2	Stage 2

| Corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other source of stage 1 information |  |  |
| --- | --- | --- |
| TS or CR(s) | Clause | Remarks |

If no identified source of stage 1 information, justify:

Go to §3.

#### 2.3.3	Stage 3

| Corresponding stage 2 work item (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Else, corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other justification |  |  |
| --- | --- | --- |
| TS or CR(s) or external document | Clause | Remarks |

If no identified source of stage 2 information, justify:

Go to §3.

#### 2.3.4	Test spec

| Related Work Item(s) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

Go to §3.

#### 2.3.5	Other

| Related Work Item(s) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship | TS / TR |

Go to §3.

### 2.4	Work task

| Parent Building Block |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

## 3	Justification

As part of Release 8 3GPP developed the concept of IMS Centralized Services which was then further extended in Release 9.

The goal of ICS was to enable an operator to provide on a per-subscriber basis, communication services such that all services and service control are only based on IMS mechanisms and enablers. ICS therefore enables IMS services, particularly voice, to be provided using the CS domain access to transport the media including by legacy UEs.

An effective deployment of services centralization will result in a number of benefits for the network operator:

- Service layer: a full implementation of ICS for all the subscribers will result in a fully consistent service delivery as well as remove the problems related to the configuration and synchronization of supplementary service settings in the two domains.

- Network layer: it will be possible for an operator to modernize the network moving towards a pure All-IP architecture. Furthermore centralization of services in IMS and use of a single service domain will also allow the decommissioning of the traditional GSM network elements that are reaching the end of life such as HLR, SMSC thus leading to network simplification.

- On the interconnection side, once the call control is realized in IMS by all interconnection partners there would be no need to keep supporting SS7 interfaces in the core network. Note that the support of SS7 interfaces may be needed during a transitory phase, e.g. in order to support inbound and outbound roamers who still rely on CS call control.

However the functionality that has been specified in TS 23.292 has the following limitations:

a)ICS services are provided by the HPLMN of a subscriber. If a subscriber roams in a VPLMN and the HPLMN does not support ICS services then the VPLMN is unable to offer services to the roaming subscriber based on ICS;

b) ICS assumes emergency call (including eCall) originated in 2/3G CS RAN will be established via legacy means (MSC Call Control only, ISUP/TDM respectively BICC/RTP links towards PSAP) without usage of IMS Service Domain.

A work item is needed to study  solutions that overcome the limitations of ICS, in order to enable operators to deploy an architecture that supports  services  always being provided, regardless of the access technology in use, via IMS.

## 4	Objective

The objective of this work item is to study modifications to the architecture in order to enable all services to be controlled via IMS rather than via CS legacy mechanisms (single service domain), for all subscribers of a network and also for inbound roamers.

The study will be conducted to document solutions aiming to satisfy the following conditions:

- There shall be no impacts on the UE. This implies that for a UE using a CS bearer:

- All NAS level signalling will continue to support TS 24.008 messages and flows.

- Emergency calls (including eCall) shall be routed via the IMS service domain as per requirement in TS 22.101 introduced by CR 0497	rev2.

- User profile storage functionality offered by HLR and VLR should not be required for the support of IMS service logic

- The solution will support inbound roamers whose HPLMN does not support ICS.

- The solution will support outbound roamers served by a PLMN that does not support ICS

- The solution should study if it is feasible to replace SS7 interfaces with IP interfaces (e.g. DIAMETER).

Solutions found during the study phase will be evaluated against the above criteria and against existing mechanisms available in 3GPP specifications before deciding whether to make changes to the relevant technical specifications.

## 5	Service Aspects

No additional service aspects compared to the original ICS

## 6	MMI-Aspects

None identified

## 7	Charging Aspects

The use of CDRs produced by CS domain nodes will need to be investigated.

## 8	Security Aspects

Once the study phase has been concluded, SA3 will be asked to investigate as appropriate (e.g. security, authentication, LI).

## 9	Impacts

| Affects: | UICC apps | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 10	Expected Output and Time scale

| New specifications [If Study Item, one TR is anticipated] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title | 1st rsp. WG | 2nd rsp. WG(s) | Presented for information at plenary# | Approved at plenary # | Comments |
| TR 23.7bc | Feasibility Study on Service Domain Centralization | SA2 | - | SA#72 (June 2016) | SA#73 (Sept 2016) | The TR may be abandoned once agreement is reached on the preferred alternative in favour of starting the normative work. |

| Affected existing specifications [None in the case of Study Items] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Spec No. | CR | Subject of the CR | Approved at plenary# | Comments |

## 11	Work item rapporteur(s)

Chris Joul (T-Mobile USA)

chris (dot) joul (at) t (dash) mobile (dot) com

## 12	Work item leadership

TSG SA WG 2

## 13	Supporting Individual Members

Supporting IM name

Deutsche Telekom

T-Mobile USA

TeliaSonera

Telenor

NEC

Huawei

China Mobile

ZTE