SA WG2 Meeting #116	S2-163732

11 - 15 Jul 2016, Vienna, AT	(revision of S2-163490 and S2-163408)

Source:	NTT DOCOMO, MediaTek Inc.

Title:	Capturing Architecture Options in the TR

Document for:	Approval

Agenda Item:	6.10.0

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: The contribution adds the architecture options proposed by SA and RAN plenary #72 in their joint meeting in Busan.

# Proposal

The following updates are proposed

MODIFIED TEXT for TR 23.799

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.203: "Policy and charging control architecture".

[4]	3GPP TR 22.861: "Feasibility Study on New Services and Markets Technology Enablers for Massive Internet of Things; Stage 1 (Release 14)".

[5]	3GPP TR 22.862: "Feasibility Study on New Markets and Technology Enablers - Critical Communications; Stage 1(Release 14)".

[6]	3GPP TR 22.863: "Feasibility Study on New Services and Markets Technology Enablers - Enhanced Mobile Broadband; Stage 1 (Release 14)".

[7]	3GPP TR 22.864: "Feasibility Study on New Services and Markets Technology Enablers - Network Operation; Stage 1 (Release 14)".

[8]	NGMN Alliance: "Description of Network Slicing Concept", Version 1.0, January 13, 2016 (http://www.ngmn.org/publications/technical.html).

[9]	3GPP TR 22.891: "Feasibility Study on New Services and Markets Technology Enablers; Stage 1 (Release 14)".

[x]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[y]	SP-160464/RP-161266, “5G Architecture Options- Full Set”. Joint RAN/SA Meeting, Busan, S. Korea, June 2016.

[z]	SP-160460/RP-161269, “Tasks”, Joint RAN/SA Meeting, Busan, S. Korea, June 2016.

[xx]	3GPP TR 38.801: " Study on New Radio Access Technology; Radio Access Architecture and Interfaces"

Next Change

# Annex X: Deployment Scenarios for consideration in this TR

## X.1	General

This section provides high level deployment scenarios corresponding to the options listed in SP-160464/RP-16266 [x]. See TR 38.801 [xx] for further details of the NextGen RAN architecture.

As part of the present TR, SA2 will focus on the study of the next generation system supporting deployment scenarios2, 4, 5 and 7. RAN groups in addition will study Option 3 (see SP-160460/RP-161269 [y]). For Option 3, SA2 will study interworking and migration scenarios with the Next Generation system. Architecture Option 1 is EPC with EUTRAN, and is not captured in this annex as its support is already specified.

NOTE 1: Options 6 and 8 SP-160464/RP-16266 [x]are not reflected in this Annex as they were deemed irrelevant at TSG#72 plenaries.

Some of the reference points names and definitions between the UE and NGC and between NR and NGC are given in Annex G. Reference point for EPC and their definitions are provided in TS 23.401 [2].

NOTE 2:	In the figures below the RAN is represented by a box with the RAN type (E-UTRAN or NG RAN) followed by radio access technology in brackets (NR, E-UTRA or Evolved E-UTRA)

NOTE 3: 	The interface names between RAN nodes and between the UE and the RAN node are in the remit of RAN groups and are left unnamed in the figures in this Annex.

## X.2 Deployment Option 2 – Standalone NR in NextGen System

UE

NGC

NG2

NG3

NG1

NG RAN (NR)

Figure X.2-1: Deployment Option 2.

This deployment option is for NG RAN with the radio access based on NR in a stand-alone configuration in NextGen System.

## X.3 Deployment Option 3 – Non-standalone NR in EPS

UE

EPC

EPC NAS

S1-MME

S1-U

eNB

NR Node

UE

EPC

EPC NAS

S1-MME

S1-U

S1-U

(3)

(3a)

eNB

NR Node

Figure X.3-1: Deployment Option 3. RAN-level split-bearer option and 3a. SCG bearer option.

This deployment option is a dual connectivity deployments with E-UTRA as the anchor RAT  and NR as the secondary RAT  in a non stand-alone configuration in EPS. .

## X.4 Deployment Option 4 – Non-standalone Evolved E-UTRA in NextGen System

NG1

NG1

Evolved eNB

NGC

Evolved eNB

NG3

NGC

NG2

NR Node

NG2

NR Node

NG3

UE

NG3

UE

NG RAN

NG RAN

(4)

(4a)

Figure X.4-1: Deployment Option 4. RAN-level split-bearer option and 4a. SCG bearer option.

This deployment option is a dual connectivity deployment with NR as the anchor RAT and Evolved E-UTRA as the secondary RAT in a non stand-alone configuration in NextGen System.

## X.5 Deployment Option 5 – Standalone Evolved E-UTRA in NextGen System

UE

NG RAN   (Evolved        E-UTRA)

NGC

NG1

NG2

NG3

Figure X.5-1: Deployment Option 5.

This deployment is for NG RAN with the radio access based on Evolved E-UTRA in a stand-alone configuration in NextGen System.

UE

NGC

NG1

NG2

NG3

NR Node

Evolved eNB

UE

NGC

NG1

NG2

NG3

NG3

NR Node

Evolved eNB

(7)

(7a)

X.6 Deployment Option 7 – Non-standalone NR in NextGen System

NG RAN

NG RAN

Figure X.6-1: Deployment Option 7. RAN-level split-bearer option and 7a. SCG bearer option

This deployment option is for dual connectivity deployments with Evolved E-UTRA as the anchor RAT and NR as the secondary RAT in a non stand-alone configuration in NextGen system.