SA WG2 Meeting #116	S2-164066

11 - 15 Jul 2016, Vienna, AT	(revision of S2-163370)

Source:	LG Electronics

Title:	Solution - Session Management on the PDU session(s) via different accesses

Document for:	Approval

Agenda Item:	6.10.4

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: This paper proposes the solution for the Session Management on the PDU session(s) via different accesses

Introduction

This contribution considers

-	SM_WT_#5: PDU session(s) via different accesses

The proposed solution identifies the architecture options with identifying the combination of access networks Also it proposes the initial definition of “a multi-access PDU session”

Proposal

It is proposed to add the following changes to TR 23.799.

* * * * Start of 1st Change * * * *

All NEW text to TR 23.799

6.4.X	Solution 4.X: Session Management on the PDU session(s) via different accesses

This solution applies to key issue 4 on session management. Specifically, the solution addresses the following work tasks:

-	SM_WT_#5: PDU session(s) via different accesses

6.4.X.1	Architecture description

This clause describes a session management architecture focusing on the following three cases:

- Multiple PDU sessions via different accesses to the different data networks

- Multiple PDU sessions via different accesses to  the same data network

- A PDU session via different accesses to the same data network (a.k.a. multi-access PDU session)

Editor’s note: In the following high level architectures, it is FFS if the UE has one or two NG1 connections with the NextGen core. How to route over multiple accesses to the same CP function is FFS.

The proposed solution assumes the following session management model principles:

- 	A UE can have multiple PDU sessions via simultaneous multiple accesses to the different data networks or same data networks (as showed in the figure 6.4.X.1-1 and figure 6.4.X.1-2)

-	A UE can have multiple PDU sessions via NG RAN and non-3GPP access network

-	The establishment and release for the PDU Session via NG RAN (NR or Evolved E-UTRA) is performed using NG1 signalling.

- 	The establishment and release for the PDU Session via non-3GPP access may be performed using NG1 signalling.

- The SM context of NextGen core network should keep the access network types for multiple PDU sessions.

- A UE can have a PDU session via different accesses to the same data network (a.k.a. multi-access PDU session) (as showed in the figure 6.4.X.1-3)

- This solution defines « a multi-access PDU session » as follows;

A PDU session whose traffic can be routed over either a NG RAN or a non-3GPP access or both.

Editor's note: The definition of « a multi-access PDU session » is needed to be refined.

- The SM context of NextGen core network should keep the access network types for a multi-access PDU session.

Figure 6.4.X.1-1: Multiple PDU sessions via different accesses to the different data networks

Figure 6.4.X.1-2: Multiple PDU sessions via different accesses to the same network

Editor’s note: In case session bitrate limitation across all PDU sessions to the same data network is required (to be determined by the QoS key issue), then for UE with multiple PDU sessions towards the same DN there is the need of a Core Network User plane “convergence point” to support across session bitrate enforcement and charging. Across-session bitrate enforcement has to be run in a unique entity that handles all the DL traffic from the same DN. DL traffic Charging is to be performed  after potential packet discard by across-session bitrate enforcement.

Figure 6.4.X.1-3: Multi-access PDU session to the same network

6.4.X.2	Function description

Editor's note:	This clause will contain function descriptions and the interactions among the network functions.

6.4. X.3	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

* * * * End of Changes * * * *