SA WG2 Meeting #120	S2-172062

27-31 March 2017, Busan

Source:	Huawei, HiSilicon

Title:	TS 23.501: Re-Registration procedure initiated by UE after NSSAI changes

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	5GS_ph1/Rel-15

Abstract of the contribution: This contribution proposes to update the description of re-Registration procedure initiated by UE after NSSAI changes in TS 23.501.

# 1.     Discussion

In the last meeting, S2-171622 has been approved and the following text is captured in clause 5.15.5 of TS 23.501.

(How Temporary ID is used in RRC layer for RAN to route the NAS message to the AMF)

[…]

Upon successful Registration, the UE is provided with a globally unique Temporary ID by its serving AMF. The UE includes the locally unique Temporary ID in the RRC Connection Establishment during subsequent initial accesses to enable the RAN to route the NAS message to the appropriate AMF, as long as the Temp ID is valid. In addition, the serving PLMN may return the latest Accepted NSSAI of the slices permitted by the serving PLMN for the UE. The Accepted NSSAI includes the S-NSSAI values of the slices permitted by the UE's serving PLMN.

When receiving the NSSAI and a complete locally unique Temporary ID in RRC, if the RAN can reach to the AMF corresponding with the locally unique Temporary ID, then RAN forwards the request to this AMF. Otherwise, the RAN selects a suitable AMF based on the NSSAI provided by the UE and forwards the request to the selected AMF. If the RAN is not able to select an AMF based on the provided NSSAI, then the request is sent to a default AMF.

[…]

(NSSAI changing will trigger a re-Registration procedure)

The network, based on local policies, subscription changes and/or UE mobility, can change the set of Network Slices that is being used by a UE by providing the UE a notification of Accepted NSSAI change indicating a new value of NSSAI. This then triggers a UE initiated re-Registration procedure including in RRC and NAS Signalling the value of the new NSSAI the network has provided.

[…]

Based on the above description, the UE can initiate re-Registration procedure with UE’s Temporary ID after UE receives the new NSSAI from the network as long as the Temporary ID is valid. The RAN node forwards the re-Registration request to the old AMF corresponding to the Temporary ID. Then the redirection between AMFs is performed.

However, if the new set of network slice indicated by the new (accepted) NSSAI is fully isolated with the old set of network slices which currently the UE registered to (e.g. the UE subscription changed from slice [1,2,3] of tenant A to slice[4,5] of tenant B), the context transferring and relocation from the old AMF to the new AMF cannot be performed. For that case, the RAN should not connect to the old AMF any more. To avoid the RAN routing the re-Registration request to the old AMF, the temporary ID shall not be included in the re-Registration procedure.

Proposal:

For a UE that is already registered, if the subscribed NSSAI changes, and the serving AMF cannot decide a target AMF corresponding with the new NSSAI due to subscription changes, the network provides the UE a notification of Accepted NSSAI change indicating a new value of NSSAI and also indicates UE that UE’s Temporary ID is invalid. The UE’s context (e.g., MM context, SM context, etc.) stored in the current set of Network Slices is released. This then triggers a UE initiated re-Registration procedure with UE’s SUPI in the NAS and new NSSAI both in RRC and NAS signalling.

# 2	Proposal

It is proposed to update the following text into clause 5.15.5 for TS 23.501

***** BEGIN 1st CHANGE *****

### 5.15.5	Detailed Operation Overview

When a UE Registers with a PLMN, the UE shall provide to the network in RRC and NAS layer either the Configured-NSSAI, the Accepted NSSAI or sub-set of those, if stored in the UE.

Editor's note:	Whether NSSAI in RRC and NAS are exactly the same, is to be determined.

The NSSAI is used to select the AMF, whereas, the S-NSSAI is used to assist the selection of a Network Slice instance.

The UE shall store a Configured and/or Accepted NSSAI per PLMN.

-	The Configured NSSAI is configured in a UE by the HPLMN to be used in a PLMN when no PLMN-specific Accepted NSSAI is stored in the UE.

-	The Accepted NSSAI is the NSSAI provided by the PLMN to the UE in registration procedures and the UE shall use this in that PLMN until the next registration from that PLMN. The Registration Accept message may include the Accepted NSSAI. The accepted NSSAI may be updated by a subsequent Registration Procedure.

If the UE has been provided a Configured or Accepted NSSAI for the selected PLMN, the UE shall include this NSSAI in RRC Connection Establishment and in NAS. The RAN routes the initial access to an AMF using the provided NSSAI.

If the UE has not yet received any Accepted NSSAI for the selected PLMN, but the UE has been provided with a Configured NSSAI for the selected PLMN, the UE may provide the Configured NSSAI or sub-set in RRC Connection Establishment and in NAS. The RAN uses the NSSAI for routing the initial access to an AMF.

If the UE does not provide any NSSAI (Accepted or Configured) for the selected PLMN in RRC Connection Establishment and in NAS, the RAN sends NAS signalling to a default AMF.

Upon successful Registration, the UE is provided with a globally unique Temporary ID by its serving AMF. The UE includes the locally unique Temporary ID in the RRC Connection Establishment during subsequent initial accesses to enable the RAN to route the NAS message to the appropriate AMF, as long as the Temp ID is valid. In addition, the serving PLMN may return the latest Accepted NSSAI of the slices permitted by the serving PLMN for the UE. The Accepted NSSAI includes the S-NSSAI values of the slices permitted by the UE's serving PLMN.

When receiving the NSSAI and a complete locally unique Temporary ID in RRC, if the RAN can reach to the AMF corresponding with the locally unique Temporary ID, then RAN forwards the request to this AMF. Otherwise, the RAN selects a suitable AMF based on the NSSAI provided by the UE and forwards the request to the selected AMF. If the RAN is not able to select an AMF based on the provided NSSAI, then the request is sent to a default AMF.

The network operator may provision the UE with network slice selection policy (NSSP). The NSSP includes one or more NSSP rules each one associating an application with a certain S-NSSAI. A default rule which matches all applications to a S-NSSAI may also be included. When a UE application associated with a specific S-NSSAI requests data transmission, then:

-	If the UE has one or more PDU sessions established with this specific S-NSSAI, the UE routes the user data of this application in one of these PDU sessions, unless other conditions in the UE prohibit the use of these PDU sessions. If the application provides a DNN, then the UE considers also this DNN to determine which PDU session to use.

If the UE does not have a PDU session established with this specific S-NSSAI, the UE requests a new PDU session with this S-NSSAI and with the DNN that may be provided by the application. In order for the RAN to select a proper resource for supporting network slicing in the RAN, RAN needs to be aware of the Network Slices used by the UE.

Editor's note:	how the RAN is made aware of the network slices used by the UE is FFS.

The network, based on local policies, subscription changes and/or UE mobility, can change the set of Network Slices that is being used by a UE by providing the UE a notification of Accepted NSSAI change indicating a new value of NSSAI. This then triggers a UE initiated re-Registration procedure including in RRC and NAS Signalling the value of the new NSSAI the network has provided.

Change of set of slices used by a UE (whether UE or Network initiated) may lead to AMF change subject to operator policy.

NOTE 1:	Changing the set of network slices accessible by the UE will result in termination ongoing PDU sessions with the original set of network slices if these slices are no longer used (Some slices are still retained, potentially).

Editor's note:	The condition is under which the UE is able to request the change of the network slices, and what it is able to request is FFS.

During the initial Registration procedure, in case the network decides that the UE should be served by a different AMF, then the AMF that first receives the initial Registration Request shall redirect the initial Registration request to another AMF via the RAN or via direct signalling between the initial AMF and the target AMF. The redirection message sent by the AMF via the RAN shall include information about the new AMF to serve the UE.

For a UE that is already registered, the system shall support a redirection initiated by the network of a UE from its serving AMF to a target AMF.

-	Operator policy determines whether redirection between AMFs is allowed.

-	If the network decides to redirect the UE due to NSSAI change, the network send the updated/new NSSAI to the UE using an RM procedure and an indication for the UE to initiate an Registration Update procedure with the updated/new NSSAI. The UE then initiates the Registration Update procedure with the updated/new NSSAI.

For a UE that is already registered, if the subscribed NSSAI changes, and the serving AMF cannot decide a target AMF corresponding with the new NSSAI, the network provides the UE a notification of Accepted NSSAI change indicating a new value of NSSAI and also indicates UE that UE’s Temporary ID is invalid. The UE’s context (e.g., MM context, SM context, etc.) stored in the current set of Network Slices is released. This then triggers a UE initiated re-Registration procedure with UE’s SUPI in the NAS and new NSSAI both in RRC and NAS signaling.

The AMF selects an SMF in a network slice instance based on S-NSSAI, DNN and other information e.g. UE subscription and local operator policies. The selected SMF establishes a PDU session based on S-NSSAI and DNN.

* * * * End of Changes * * * *