3GPP TSG SA WG2 Meeting #121	S2-173285

15 - 19 May 2017, Hangzhou, China	(revision of S2-17XXXX)

Source:	China Telecom, China Mobile, CATT, CATR

Title:	TS23.501: Functional description for EIR

Document for:	Approval

Agenda Item:	6.5.11

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: This contribution proposes to add the service-based interface and functional descriptions for EIR in TS23.501.

- Proposal

The ME Identity Check service provided by EIR is defined in section 5.2.4 of TS 23.502.It is proposed to add the service-based interface and functional descriptions for EIR to TS23.501.

***** START of 1st CHANGE *****

4.2.2	Network Functions and entities

The 5G System architecture consists of the following network functions (NF). The functional description of these network functions is specified in clause 6.

-	Authentication Server Function (AUSF)

-	Core Access and Mobility Management Function (AMF)

-	Data network (DN), e.g. operator services, Internet access or 3rd party services

-	Structured Data Storage network function (SDSF)

-	Unstructured Data Storage network function (UDSF)

-	Network Exposure Function (NEF)

-	NF Repository Function (NRF)

-	Policy Control function (PCF)

-	Session Management Function (SMF)

-	Unified Data Management (UDM)

-	User plane Function (UPF)

-	Application Function (AF)

-	User Equipment (UE)

-	(Radio) Access Network ((R)AN)

-  Equipment Identity Register (EIR)

***** START of 2nd CHANGE *****

4.2.6	Service-based interfaces

The 5G System Architecture contains the following service-based interfaces:

Namf:	Service-based interface exhibited by AMF.

Nsmf:	Service-based interface exhibited by SMF.

Nnef:	Service-based interface exhibited by NEF.

Npcf:	Service-based interface exhibited by PCF.

Nudm:	Service-based interface exhibited by UDM.

Naf:	Service-based interface exhibited by AF.

Nnrf:	Service-based interface exhibited by NRF.

Nausf:	Service-based interface exhibited by AUSF.

Neir:	Service-based interface exhibited by EIR.

***** START of 3rd CHANGE *****

6.2.X	EIR

The EIR is an optional network function that supports the following functionality:

-	Check the status of Mobile Equipment's identity (e.g. to check that it has not been stolen, or, to verify that it does not have faults).

***** End of CHANGE *****