SA WG2 Meeting #123	S2-177321

23 - 27 October 2017 Ljubljana, Slovenia

Source:	Huawei, HiSilicon

Title:	TS 23.502: Deactivation of NGAP association with release of UE-TNLA-binding for all UEs served by an AMF

Document for:	Approval

Agenda Item:	6.5.2

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution proposes to add the NGAP UE association release procedure.

Introduction

Per TS23.501 the NGAP UE association/ NGAP UE-TNLA-binding are defined as below,

NGAP UE association: The logical per UE association between a 5G AN node and an AMF.

Due to AMF management, it is possible to deactivated NGAP association with release of TNLA binding for all UE served by one AMF as described in TS23.501 clause 5.21.2. The corresponding procedure in TS 23.502 need to be added. It is proposed to add the NGAP UE association deactivation per node procedure into TS 23.502.

Also to support the AMF failure handling, the backup AMF may also be notified to the 5G AN during the N2 setup procedure.

Proposal

It is proposed to update TS 23.502 as follows:

* * * Begin of Changes * * *

4.2.7.1	N2 Configuration

At power up, restart and when modifications are applied, the 5G-AN node and AMF use non-UE related N2 signalling to exchange configuration data. Full details of this configuration data are specified in TS 38.300 [9], but the following highlights some aspects.

The AMF supplies the 5G-AN node with information about:

a)	the AMF Name and the GUAMI(s) configured on that AMF Name;

b)	the set of TNL associations available for reception of the initial N2 message, e.g. N2 INITIAL UE MESSAGE;

NOTE:	The AMF chooses whether or not to use the same TNL association for the initial N2 message and subsequent messages for that UE.

c)	the set of TNL associations that are not permitted for the initial N2 message but which the AMF can use for subsequent UE related N2 signalling. The AMF may leave this set empty;

d)	weight factor of the AMF within the AMF set; and

e)	(optional) weight factor of the TNL association within the AMF.

f) (optional) for each GUAMI(s) configured on that AMF the corresponding backup AMF name.

* * * 2nd Changes * * *

4.2.7.2.x	NGAP UE association deactivation per node procedure

If AMF failure is detected by 5G AN, all NGAP UE associations for UEs served by that AMF are deactivated.

If AMF becomes unavailable due to planned maintenance, the AMF notifies the 5G AN about the unavailable GUAMI(s) and provides optionally a target AMF Name corresponding to each unavailable GUAMI. The 5G AN deactivates all NGAP UE associations of the UEs related to the indicated unavailable GUAMI(s) unless the notification from the AMF includes an indicator that the AMF will deactivate the NGAP UE associations on a per UE-basis. In that case, if 5G AN supports, the 5G AN waits with the deactivation until the timer expires so that the AMF may deactivate NGAP UE associations on per UE-basis.

For the deactivated NGAP UE association, the TNLA binding is released while keeping the affected UEs in CM-CONNECTED state and keeping the corresponding N3 interface.

* * * End of Changes * * *