SA WG2 Meeting #123 meeting	S2-177482

23 – 27 November 2017, Ljubljana	(was S2-17----)

Source:	CATT

Title:	Changes on Modification of Set of Network Slices

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to change the content in clause 5.15.5.2.2, TS 23.501v1.4.0.to include the two cases of both valid and invalid 5G-GUTI if it is necessary for UE to perform a registration procedure.

- Discussion

For a UE registered with a network, if the modification of set to Network Slice leads to the change of AMF, a Registration request is always necessary whether the current Serving AMF can determine the Target AMF or not. It is proposed to change the content in clause 5.15.5.2.2, TS 23.501v1.4.0.

- Proposal

The following changes are proposed in 3GPP TS 23.501 v1.4.0.

*************** Start of change*********************

5.15.5.2.2	Modification of the Set of Network Slice(s) for a UE

The set of Network Slices for a UE can be changed at any time while the UE is registered with a network, and may be initiated by the network, or the UE under certain conditions as described below.

NOTE:	In this release of the specification it is assumed that the registration area allocated by the AMF to the UE shall have homogeneous support for network slices.

The network, based on local policies, subscription changes and/or UE mobility, operational reasons (e.g. a Network Slice instance is no longer available), may change the set of Network Slice(s) to which the UE is registered and provides the UE new Allowed NSSAI. The network may perform such change during a Registration procedure or trigger a notification towards the UE of the change of the Network Slices using a Generic UE Configuration Update procedure as specified in TS 23.502 [3], clause 4.2.4. The new Allowed NSSAI is determined as described in clause 5.15.5.2.1 (an AMF Relocation may be needed). The AMF provides the UE with the new Allowed NSSAI and TAI list, and:

-	If the changes to the Allowed NSSAI do not require the UE to perform a registration procedure:

-	The AMF indicates that acknowledgement is required, but does not indicate the need to perform a registration procedure;

-	The UE responds with a UE configuration update complete message for the acknowledgement.

-	If the changes to the Allowed NSSAI require the UE to perform a registration procedure:

-	The serving AMF indicates to the UE whether current 5G-GUTI is valid or not(e.g. based on whether the new S-NSSAIs require a separate AMF that cannot be determined by the current serving AMF) and the need for the UE to perform a registration procedure after entering CM-IDLE state. The AMF shall release the NAS signalling connection to the UE to allow to enter CM-IDLE based on local policies (e.g. immediately or delayed release);

-	The UE initiates a registration procedure after the UE enters CM-IDLE state. The UE shall include SUPI if current 5G-GUTI is invalid and new Allowed NSSAI in the registration in this case.

When a Network Slice used for a one or multiple PDU Sessions becomes no longer available for a UE, in addition to sending the new Allowed NSSAI to the UE, the following applies:

-	In the network, if the Network Slice becomes no longer available under the same AMF (e.g. due to UE subscription change), the AMF indicates to the SMF(s) corresponding to the relevant S-NSSAI to autonomously release the UE's SM context

-	In the network, if the Network Slice becomes no longer available with AMF relocation (e.g. due to Registration Area change), the new AMF indicates to the old AMF that the PDU Session(s) associated with the relevant S-NSSAI shall be released. The old AMF informs the corresponding SMF(s) to autonomously release the UE's SM context.

-	In the UE, the PDU session(s) context is implicitly released after receiving the Allowed NSSAI in the Registration Accept message.

The UE uses UE Configuration (e.g. NSSP) to determine whether ongoing traffic can be routed over existing PDU sessions belonging to other Network Slices or establish new PDU session(s) associated with same/other Network Slice.

In order to change the set of S-NSSAIs being used, the UE shall initiate a Registration procedure as specified in clause 5.15.5.2.1.1.

Change of set of S-NSSAIs to which the UE is registered (whether UE or Network initiated) may lead to AMF change subject to operator policy, as described in clause 5.15.5.2.1.

Editor's note:	The condition under which the UE is able to request the change of the Network Slices, and what it is able to request, are FFS.

*************** End of changes *********************