SA WG2 Meeting #123		S2-177696

23 – 27 October, 2017, Ljubljana, Slovenia	(revision of S2-176902)

Source:	Ericsson

Title:	TS 23.503: clause 6.1.2.1 access and mobility policy control

Document for:	Approval

Agenda Item:	6.5.6

Work Item / Release:	5GS_ph1

Abstract of the contribution: This contribution proposes to move the content of A.3.1.5 Access and Mobility Policy Enforcement into normative text. It covers issue nr. 10 in the ”to do list for 5G policy framework”.

# Introduction

This document proposes to move the content of the clause A.3.1.5 Access and Mobility Policy Enforcement in Annex A into the normative text as follows:

- The functional description of the AMF access and mobility policy control is updated to include both the management of the service area restrictions and the management of the RFSP Index by the PCF.

- The terminology is aligned to:

  - AMF Access and Mobility related policy control information to describe the information sent by PCF to AMF.

  - Subscribed service area restrictions to describe the information stored in UDM. Authorized service area restrictions to describe the information provided by the PCF.

  - Subscribed RFSP Index to describe the information stored in UDM. Authorized RFSP Index to describe the information provided by the PCF.

- The AMF Access and Mobility related policy control information is refined to show conditional information elements and the possibilities for the PCF to modify the information elements.

- The functional description in the AMF Access and Mobility policy related control information clause is moved to the AMF Access and Mobility policy control.

# Proposal

It is proposed to update 23.503 as described below.

* * * First Change * * *

#### 6.1.2.1	Access and mobility policy control

Editor's note:	This clause will cover functionality that is independent from PCC related functionality.

The access and mobility policy control encompasses the management of service area restrictions and the management of the RFSP functionalities.

The management of service area restrictions enables the PCF to modify the service area restrictions used by AMF as described in TS 23.501 [2] clause 5.3.4.

A UE's subscription may contain service area restrictions, which may be further modified by PCF based on operator defined policies at any time, either by expanding a list of allowed TAIs or by reducing a non-allowed TAIs or by increasing the maximum number of allowed TAIs. Operator defined policies in the PCF may depend on input data such as UE location, time of day, information provided by other NFs, etc.

The AMF may report the subscribed service area restrictions received from UDM during Registration procedure or when the AMF changed, the conditions for reporting are that local policies in the AMF indicate that Access and Mobility Control is enable. The AMF reports the subscribed service area restrictions to the PCF also when the service area restrictions change event trigger is met. The AMF receives the modified service area restrictions from the PCF. The AMF stores them then use it to determine mobility restriction for a UE. The PCF may indicate the AMF that there is an unlimited service area.

The service area restrictions consist of a list of allowed TAI(s) or a list of non-allowed TAI(s) and optionally the maximum number of allowed TAIs.

NOTE 1: 	The enforcement of the service area restrictions is performed by the UE, when the UE is in CM-IDLE state or in CM-CONNECTED state when in RRC Inactive, and in the RAN/AMF when the UE is in CM-CONNECTED state.

The management of the RFSP Index enables the PCF to modify the RFSP Index used by the AMF to perform radio resource management functionality as described in TS 23.501 [2] clause 5.3.4.

UEs' subscription may contain the RSFP Index which may be further adjusted by the PCF based on policies at any time.

For radio resource management, the AMF may report the subscribed RFSP Index received from UDM during the Registration procedure or when the AMF changed. The conditions for reporting are that local policies in the AMF indicate that Access and Mobility Control is enable. The AMF reports the subscribed RFSP Index to the PCF when the subscription to RFSP Index change to the PCF is met. The AMF receives the modified RFSP Index from the PCF.

Upon change of AMF, the source AMF informs the PCF that the UE context was removed in the AMF.

NOTE 2:	The enforcement of the RFSP Index is performed in the RAN.

* * * Next Change * * *

## 6.5	Access and mobility related policy control information

Editor's note:	This clause will cover functionality that is independent from PCC related functionality.

To enable the enforcement in the 5GC system of the access and mobility policy decisions made by the PCF for the control of the service area restrictions and RFSP Index, the 5GC system may provide the Access and mobility related policy control information from the PCF to the AMF.

Table 6.5-1 lists the AMF access and mobility related policy information

Table 6.5-1: Access and mobility related policy control information

| Information name | Description | Category | PCF permitted to modify in a UE context in the AMF | Scope |
| --- | --- | --- | --- | --- |
| Service Area Restrictions | This part defines the service area restrictions |  |  |  |
| List of allowed TAIs. | List of allowed TAIs (NOTE 3) (NOTE 4). | Conditional (NOTE 1) | Yes | UE context |
| List of non-allowed TAIs. | List of non-allowed TAIs (NOTE 3). | Conditional (NOTE 1) | Yes | UE context |
| Maximum number of allowed TAIs | The maximum number of allowed TAIs. (NOTE 4) | Conditional (NOTE 1) | Yes | UE context |
| RFSP Index | This part defines the RFSP index |  |  |  |
| RFSP Index | Defines the RFSP Index that applies for a UE | Conditional (NOTE 2) | Yes | UE context |
| NOTE 1: If service area restrictions is enable. NOTE 2: If RFSP index is enable. NOTE 3: Either the list of allowed TAIs or the list of non-allowed TAIs are provided by the PCF. NOTE 4: Both the maximum number of allowed TAIs and the list of allowed TAIs may be sent by PCF. |  |  |  |  |

The list of allowed TAIs indicates the TAIs where the UE is allowed to be registered, see TS 23.501 [2] clause 5.3.4 for the description on how AMF uses this information.

The list of non-allowed TAIs indicates the TAIs where the UE is not allowed to be registered, see TS 23.501 [2] clause 5.3.4 for the description on how AMF uses this information.

The Maximum number of allowed TAs indicates the maximum number of allowed Tracking Areas, the list of TAI is defined in the AMF and not explicitly provided by the PCF.

The RFSP Index defines the RFSP Index for radio resource management functionality.

* * * End of Changes * * *

* * * End of Changes * * *