3GPP TSG-SA WG2 Meeting #127Bis	S2-185179

Newport Beach, USA, 28 May – 1 June, 2018	(revision of S2-xxxxxx)

Source:	Huawei, HiSilicon

Title:	Solution for KI 1#: Redundant Transmission for URLLC service

Document for:	Approval

Agenda Item:	6.20

Work Item / Release:	FS_URLLC /Rel-16

Abstract: This paper proposes solution to Key Issue #1: Supporting high reliability by redundant transmission in user plane.

# 1	Introduction

This contribution proposes a solution on how to establish the redundant transmission paths for URLLC service.

# 2	Proposal

It is proposed to agree the following P-CR to TR 23.725.

* * * * Start of Change, all new texts * * * *

## 6.X	Solution for Key Issue #1: Supporting redundant data transmission in user plane for URLLC services

### 6.X.1	Description

This solution addresses the KI#1 of How to “Supporting high reliability by redundant transmission in user plane”.

Typically telecom-level equipments are able to provide up to five-nine reliability. However, in many cases the end-to-end reliability of real deployed network might be lower than that considering the susceptible deployment environments, especially in the last-mile.

To realize ultra-high reliability for URLLC services in the commercial deployed network, this solution proposes to use redundant transmission over different physical transport paths in CN and RAN to enhance the reliability of upper layer service.

In this solution, whether redundant transmission need to be activated for a QoS Flow can be decided by the SMF based on local policies for the DNN or slice, or by the PCF based on its QoS requirement, UE’s subscription and condition of network deployment.

Based on different deployment environments, two different redundant transmission (RT) modes are proposed in this solution:

- RT Mode 1: In case a single RAN node cannot provide enough reliability in air interface for the QoS flow, the redundant packets will be transferred between UPF and UE via two independent N3 tunnel and RAN nodes to enhance the reliability of service.

- RT Mode 2: In case a single RAN node can provide enough reliability in air interface for the QoS flow, but the reliability of single N3 tunnel is considered not high enough, e.g. due to the deployment environment of backhaul network, the redundant packets will be transferred between UPF and RAN via two independent N3 tunnel over different transport layer path to enhance the reliability of service.

Editor’s Note: How to ensure the two N3 tunnels can be transferred via different transport layer paths are FFS.

Editor’s Note: Whether and, if yes, how to switch between RT Mode 1 RT and Mode 2 is FFS.

Editor’s Note: The scenario that N3 UPF involved is FFS.

Editor’s Note: How to support redundant transmission in roaming scenario is FFS.

Editor’s Note: How to support redundant transmission during Handover procedure is FFS.

RAN can determine which mode shall be used if the PCF determines that redundant transmission shall be activated for a QoS Flow.

#### 6.x.1.1	RT Mode 1 - Redundant transmission between a UPF and two RAN nodes

RT Mode 1 supports the redundant transmission between the UE and the UPF via two independent NG-RAN nodes based on the DC architecture. With two different NG-RAN nodes, separate transport layer paths are used for redundant data transmission in user plane.

Figure 6.x.1-1: RT Mode 1 - Redundant transmission with two N3 tunnels between a UPF and two NG-RAN nodes

In RT Mode 1, a HRP (High Reliability Protocol) layer are implemented on the UE and UPF to support packet replication and elimination function.

Figure 6.x.1-2: User Plane Protocol Stack for RT Mode 1

#### 6.x.1.2	RT Mode 2 - Redundant transmission with two N3 tunnels between the UPF and a single NG-RAN node

RT mode 2 supports the redundant transmission based on two N3 tunnels between a single NG-RAN mode and the UPF. The RAN node and UPF shall support the packet replication and elimination function.

Figure 6.x.2-1: RT Mode 2- Redundant transmission with two N3 tunnels between the UPF and a single NG-RAN

In RT Mode 2, the HRP (High Reliability Protocol) layer is implemented on the RAN and UPF to support packet replication and elimination function.

Figure 6.x.2-2: User Plane Protocol Stack for RT Mode 2

Editor's note: Whether the protocol stacks of Mode 1 and Mode2 can be unified is FFS.

Editor's note: The format of HRP header and whether the HRP layer can be realized by enhancing existing protocol layers (e.g. GTP-U, SDAP) are FFS.

### 6.x.2	Procedures

#### 6.x.2.1	Activate Redundant Transmission during PDU Session Establishment procedure

The redundant transmission will be established during a PDU Session establishment procedure or a PDU Session Modification procedure.

Figure 6.x.2.1-1 depicts the establishment of redundant transmission during the PDU session establishment procedure in the non-roaming and roaming with local breakout cases.

Figure 6.x.2.1-1: Activate Redundant Transmission during PDU Session Establishment procedure

1. Steps 1-12 from clause 4.3.2.2.1(UE requested PDU Session Establishment) in TS 23.502 [3] are performed with the following differences:

The PCF indicates the SMF and the NG-RAN to activate redundant transmission for the QoS flow by the 5QI if the UE requested PDU Session Establishment procedure is initiated for a URLLC service.

The SMF then initiates an N4 Session Establishment procedure with the selected UPF and CN Tunnel Info 1& CN Tunnel Info 2 are allocated by the SMF or UPF.

The N2 SM information includes the QFI(s), QoS Profile(s), CN Tunnel Info 1& CN Tunnel Info 2 corresponding to each N3 tunnel.

2. RRC Connection Reconfiguration takes place with the UE establishing the necessary NG-RAN resources related to the QoS Rules for the PDU Session request. The NG-RAN determines whether RT Mode 1 can be used. In case of RT Mode 1, the Secondary NG-RAN node Addition procedure can be performed as defined in TS 37.340 [x].

3. Steps 14-20 from clause 4.3.2.2.1(UE requested PDU Session Establishment) in TS 23.502 [3] are performed with the following differences:

The AN Tunnel Info provided by the NG-RAN includes AN Tunnel Info 1 and AN Tunnel Info 2. In case of RT Mode 1, each AN Tunnel Info includes a tunnel endpoint for each involved NG-RAN node, and the QFIs assigned to each tunnel endpoint.

The SMF provides AN Tunnel Info 1 and AN Tunnel Info 2 to the UPF as well as the corresponding forwarding rules. The forwarding rules indicates the UPF to replicate the downlink packets and send each replicate via one of DL N3 tunnels separately to NG-RAN, and eliminate the duplicate uplink packets received from the two UL N3 tunnels.

Editor’s Note:   How to associate a CN tunnel info and an AN tunnel Info is FFS.

#### 6.x.2.2	Activate Redundant Transmission during PDU Session Modification procedure

Figure 6.x.2.1-2 depicts the establishment of redundant transmission during the UE or network requested  PDU session modification procedure in the non-roaming and roaming with local breakout cases.

#### Figure 6.x.2.1-2: Activate Redundant Transmission during PDU Session Modification procedure

1. Steps 1-12 from clause 4.3.3.2 (UE or network requested PDU Session Modification) in TS 23.502 [3] are performed with the following differences:

The PDU Session Modification procedure is triggered by the UE or the SMF to add a QoS flow for a URLLC service.

The PCF indicates the SMF and the NG-RAN to activate redundant transmission for the QoS flow by the 5QI.

The SMF then initiates an N4 Session modification procedure with the UPF and CN Tunnel Info 1& CN Tunnel Info 2 are allocated by the SMF or UPF.

The N2 SM information includes the QFI(s), QoS Profile(s), CN Tunnel Info 1& CN Tunnel Info 2 corresponding to each N3 tunnel.

2. RRC Connection Reconfiguration takes place with the UE establishing the necessary NG-RAN resources related to the QoS Rules for the PDU Session request. The NG-RAN determines whether RT Mode 1 can be used. In case of RT Mode 1, the Secondary NG-RAN node Addition procedure can be performed as defined in TS 37.340 [x].

3. Steps 6-12 f rom clause 4.3.3.2 (UE or network requested PDU Session Modification) in TS 23.502 [3] are performed with the following differences:

The AN Tunnel Info provided by the NG-RAN includes AN Tunnel Info 1 and AN Tunnel Info 2. In case of Mode 1, each AN Tunnel Info includes a tunnel endpoint for each involved NG-RAN node, and the QFIs assigned to each tunnel endpoint.

The SMF provides AN Tunnel Info 1 and AN Tunnel Info 2 to the UPF as well as the corresponding forwarding rules. The forwarding rules indicates the UPF to replicate the downlink packets and send each replicate via one of DL N3 tunnels separately to NG-RAN, and eliminate the duplicate uplink packets received from the two UL N3 tunnels.

### 6.X.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

### 6.X.4	Solution Evaluation

Editor's note:	This clause provides an evaluation of this solution.

* * * * End of Changes * * * *