3GPP TSG-SA2 Meeting #130 	S2-1900614

# Kochi, India, 21st January 2019 - 25th January 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0968 | rev | - | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Bindings between the consumer and producer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2018-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the procedures to establish binding between consumer and producer as concluded in TR 23.742. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for procedures to establish bindings between the consumer and producer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 23.502 CR XXX |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************** First Change ****************

*** BEGIN CHANGES ***

## 4.x Binding between consumer and producer

### 4.x.1 General

This clause describes the procedures to establish binding with the service producer/consumer set, or producer/consumer set and service zone, or the service producer/consumer instance.

In the procedures the interaction between two Network Functions (Consumer and Producer) may either be direct or indirect (see Annex x). In case of indirect communication, a Common Framework Support Functions (CFSF) is located between the NF service consumer and NF service producer.

### 4.x.2 Binding created as part of service response

When the service consumer instance communicates with the service producer instance, the service producer instance may return a binding information to the service consumer. The service consumer stores the received binding information. Based on the received binding information the binding is established with the service producer set, or producer set and service zone, or the service producer instance.

Figure 4.x.2-1: Binding created as part of service response

1. 	NF Service Consumer sends a service request to the NF service. If indirect communication mode is used, the consumer provides within the service request to the CFSF metadata required to discover, select, and route the message to a suitable producer.

2. 	The NF Service Producer sends a response to the consumer. If the request in step 1 creates a resource in the producer, the response identifies the created resource, and the response may indicate a binding with a producer set or producer instance, optionally for a given service zone.

3.	The NF Service Consumer uses the indicated binding information received in the previous step for subsequent requests regarding concerned resource. The following applies when the producer is selected:

-		In case of the binding is established with the producer instance, the indicated producer instance is selected. If the indicated producer instance become unavailable, a different producer instance within the same Set is selected.

-	In case of the binding is established with the producer set, one of the producer instance from the indicated producer set is selected.

-	In case of the binding is established with the producer set for a given service zone, one of the producer instance from the indicated producer set corresponding to the service zone is selected.

If direct communication mode is used, the consumer selects the producer instance and sends the request to the selected producer instance.

If indirect communication mode is used, the consumer provides within the service request to the CFSF metadata required to select and route the message to a suitable producer, e.g. parameters to identify the NF service instance set, service zone identifier. The CFSF selects the producer instance and routes the request.

4. 	The producer sends a response to the consumer. The producer may update the binding in the response; the response may indicate binding with different producer instance or change the binding from a producer set to prodcer instance or vice versa, or may remove the binding.

### 4.x.3 Binding created as part of service request

If the service consumer instance can also be as a service producer for later communication from the contacted service producer, it may include the binding information in the service request sent to the service producer.

Figure 4.x.3-1: Binding created as part of service request

1. 	Instance A, as an NF Service Consumer sends a service request to the NF service. If the Instance A can also be as a service producer for later communication from the contacted service producer, it may include the binding information in the service request sent to the service producer, and the request may indicate binding with a producer set or producer instance, optionally for a given service zone.

2. 	The NF Service Producer sends a response to the consumer.

3.	The instance B becomes a consumer and sends a service request to the NF service as describes in Steps 3-4 in Figure 4.x.2-1.

*** END CHANGES ***