3GPP TSG-SA WG2 Meeting #131	S2-1901466

# Santa Cruz, Tenerife, Spain, 25 February – 1 March 2019 	(revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0991 | rev | - | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for adding second access to MA PDU Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is a need to describe how a second access can be added to a PDU Session |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description is added for how a second access can be added to a PDU Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.x.3	UE Requested PDU Session Establishment with Network Modification to MA-PDU Session

### 4.x.3.1 	Non-roaming and Local Breakout

When a UE or SMF wants to add an access to a MA PDU Session, the PDU Session Modification procedure is used, as described in clause 4.3.3.2 with the following differences:

- In step 1a, if the request is initiated by the UE, the UE provides an indication in the PDU Session Modification Request message that the UE requests to add an access to the MA PDU Session.

- In step 3a, if the UE included an indication to add an access to the MA PDU Session in step 1a, the SMF indicates to the UE whether it was accepted. The SMF also includes the N3 tunnel information for the User Plane connection in the N2 SM information.

- In step 3b, if the request is initiated by the SMF, the SMF provides an indication in the PDU Session Modification Command message that the SMF requests to add an access to the MA PDU Session. The SMF also includes the N3 tunnel information for the User Plane connection in the N2 SM information.

#### 4.x.3.2	Home-routed roaming

Editor’s note: The support of home routed roaming scenarrios for ATSSS is FFS

When a UE or H-SMF wants to add an access to a MA PDU Session, the PDU Session Modification procedure is used, as described in clause 4.3.3.2 with the following differences:

- In step 1a, if the request is initiated by the UE, the UE provides an indication in the PDU Session Modification Request message that the UE requests to add an access to the MA PDU Session.

-	Step 3:

If the UE included an indication to add an access to the MA PDU Session in step 1a, the SMF indicates to the UE whether it was accepted. The SMF also includes the N3 tunnel information for the User Plane connection in the N2 SM information

If the request is initiated by the SMF, the SMF provides an indication in the PDU Session Modification Command message that the SMF requests to add an access to the MA PDU Session. The SMF also includes the N3 tunnel information for the User Plane connection in the N2 SM information.

**** End of Changes ****