SA WG2 Meeting #131 	S2-1901689

# 25 February - 1st March 2019, Santa Cruz de Tenerife, Spain				 (revision of S2-1901689)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0944 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Nature of the information sent over N16a to support a local UL CL / BP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarify the nature of the information sent over N16a to support a local UL CL / BP |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR starts from CR 0848 rev2 as agreed at the last meeting and updates it according to following principles |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The way for SMF to control traffic offload via a I-SMF is unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New (5.x.d, 5.x.e, 5.x.f) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE all text is new

### 5.x.d	Usage of an UL Classifier for a PDU Session controlled by I-SMF

This clause applies only in case of non-roaming or LBO roaming as control of UL CL/BP in VPLMN is not supported in HR case.

When I-SMF is involved for a PDU Session, it is possible that the UL CL controlled by I-SMF is inserted into the data path of the PDU Session. The usage of an ULCL controlled I-SMF in the data path of a PDU Session is depicted in Figure 5.x.d-1.

Figure 5.x.d-1 User plane Architecture for the Uplink Classifier controlled by I-SMF

The I-SMF determines whether UL CL will be inserted based on information received from SMF, and the I-SMF selects the UPFs acting as UL CL and/or PDU Session Anchor providing local access to the Data Network.

### 5.x.e	Usage of IPv6 multi-homing for a PDU Session controlled by I-SMF

This clause applies only in case of non-roaming or LBO roaming as control of UL CL/BP in VPLMN is not supported in HR case.

When I-SMF is involved for a PDU Session, it is possible that the BP controlled by I-SMF is inserted into the data path of the PDU Session. The usage of a BP controlled I-SMF in the data path of a PDU Session is depicted in Figure 5.x.e-1.

Figure 5.x.e-1: Multi-homed PDU Session: Branching Point controlled by I-SMF

The I-SMF determines whether BP will be inserted based on information received from SMF, and the I-SMF selects the UPFs acting as BP and/or PDU Session Anchor providing local access to the Data Network.

### 5.x.f	Interaction between I-SMF and SMF for the support of traffic offload by UPF controlled by the I-SMF

This clause applies only in case of non-roaming or LBO roaming as control of UL CL/BP in VPLMN is not supported in HR case. It applies for the architectures described in clauses 5.x.d and 5.x.e

When the I-SMF is inserted into a PDU Session, e.g. during PDU Session establishment or due to UE mobility, the I-SMF provides the DNAI list it supports to the SMF. Based on the DNAI list information received from I-SMF, the SMF may provide information for local traffic steering to the I-SMF e.g. immediately or when a new or updated or removedPCC rule(s) is/are received. The information for local traffic steering is derived from PCC rules and refers to the DNAI(s) of interest for the PDU Session that are served by the I-SMF.

The I-SMF is responsible of the insertion, modification and removal of UPF(s) to ensure local traffic steering. The SMF does not need to have access to local configuration or NRF output related with these UPF(s). Based on the information for local traffic steering and UE location the I-SMF determines whether traffic can be diverted to a local access to the Data Network. If the traffic can be diverted, the I-SMF selects UPF(s) acting as UL CL/BP and/or PDU Session Anchor based on selected DNAI, and inserts the UPF(s) into the data path of the PDU Session.

When a UL CL/BP has been inserted, changed or removed, the I-SMF indicates to the SMF that traffic offload has been inserted, updated or removed for a DNAI, providing also the IPV6 prefix that has been allocated in case a new IPV6 prefix has been allocated for the PDU Session.

From now on the SMF takes control of the traffic offload at the local UPF (UPF controlled via the I-SMF): this may entail

- Notifying the UE with the new Prefix (multi-Homing case): Issuing Router advertisement to the UE including the priorities provided to the UE to select among the prefixes related with the PDU Session.

- N4 interactions related with traffic offloading. The SMF may provide N4 information to the I-SMF for how the traffic shall be detected, enforced, monitored in UPF(s) controlled by the I-SMF: the SMF issues requests to the I-SMF containing N4 information creating / updating /removing PDR, FAR, QER, URR, ….

- Receiving N4 notifications related with traffic usage reporting

NOTE x: How the SMF decides what traffic steering and enforcement actions are enforced in UPF(s) controlled by I-SMF is left for implementation.

Editor’s Note; it is FFS how the I-SMF maps between PDR, FAR, QER, URR, … received from SMF on N16a and actual PDR, FAR, QER, URR, ….sent on N4 to the UPF(s) it controls.

When SMF has requested usage reports, the I-SMF sends the usage report information to SMF, and SMF aggregates and constructs usage reports towards PCF/CHF.

NEXT CHANGE

END OF CHANGES