SA WG2 Meeting #131 	S2-1902334

# 25 February – 1 March 2019, Santa Cruz - Tenerife, Spain		(revision of S2-1902273)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0931 | rev | 2 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access Network Performance Measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Oppo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces the Access Performance Measurements related aspects for ATSSS. It includes general principles about measurement Assistance Information and the list of measurements that apply for this release of the specification. It also includes - details on the RTT measurements and on the Access Availability/Unavailability reporting; - the protocol stack used to convey measurement messages and reporting messages between UE and UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause Access Performance Measurements is added to 23.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance Access Measurements would not be specified and ATSSS feature cannot be implemented for switching and smallest delay steering modes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 5.x.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.x.x	Access Network Performance Measurements

#### 5.x.x.1	General principles

When the UE requests an MA-PDU Session Establishment and indicates it is capable to support ATSSS, the UE may receive Measurement Assistance Information, during the establishment of an MA-PDU session. This information assists the UE in determining which measurements shall be performed over both accesses, as well as whether and when measurement reports need to be sent to the network.

Measurement Assistance Information shall include

the IP address and the UDP port of a Performance Measurement Function (PMF) in the UPF, with which the UE can send PMF protocol messages.

The following PMF protocol messages can be exchanged between the UE and the PMF:

-	Messages for Round Trip Time (RTT) measurements, e.g. when the "Smallest Delay" steering mode is used;

-	Messages for reporting Access availability/unavailability by the UE to the UPF.

Editor's Note:	Before any PMF protocol messages can be exchanged between the UE and UPF, whether a PMF session between the UE and the UPF should be established is FFS.

When the UE requests a MA-PDU session and indicates it is capable to support the MPTCP functionality only, the network should not send Measurement Assistance Information as in this case the UE can use measurements available at the MPTCP layer.

#### 5.x.x.2	Round Trip Time measurement

If required by the ATSSS rules of a MA-PDU session, RTT measurement can be conducted by the UE and UPF independently. There is no measurement reporting from one side to the other.

The estimation of the RTT by the UE or by the UPF is based on the following mechanism: one entity (UE or UPF) sends an PMF-Echo Request to the destination address/port of the PMF function in the peer entity, which replies with an PMF-Echo Response to the source address/port of the PMF function in the originating entity. The originating entity derives an estimation of the average RTT over an access type by averaging the RTT measurements obtained over this access.

Editor's note:	How the RTT thresholds per access may be included in ATSSS rules is FFS.

Editor's Note:	Whether the 5QI value used to convey the PMF-Echo Request / PMF-Echo Response messages is determined by Measurement Assistance Information or is pre-configured in the UE is FFS.

#### 5.x.x.3	Access Availability/Unavailability Report

If required by the network in the Measurement Assistance Information, the detection of the unavailability and of the availability of an access, as well as and the decision of reporting it, can be performed by the network or the UE, based on implementation. When the UE has decided, based on implementation,  to report the unavailability/availability of an access, it shall:

-	build a PMF-Access Report containing the access type and an indication of availability/unavailability of this access;

-	send the PMF-Access Report to the UPF via the user plane.

#### 5.x.x.4	Protocol stack for user plane measurements and measurement reports

Figure 5.x.x.4-1: UE-PMF / UPF-PMF protocol stack for 3GPP access

Figure 5.x.x.4-2: UE-PMF / UPF-PMF protocol stack for non-3GPP access

Editor's note:	The protocol stack in case of Ethernet MA PDU Session is FFS.

END OF CHANGES