3GPP TSG-SA WG2 Meeting #131	S2-1902050

# Santa Cruz, Tenerife, Spain, February 25 – March 1, 2019		 (revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1102 | rev | - | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding User-Plane Resources for a MA-PDU Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Need to add a procedure that supports second access addition for a MA-PDU Session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is added that specifies the procedure for second access addition based on Service Request procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for second access addition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 4.X is added in 23.502 CR0866 |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.X.7	Adding User-Plane Resources for a MA-PDU Session

When the UE establishes a MA-PDU Session using a procedure in clause 4.X.3, the MA-PDU Session may have an user plane resources over an single access. In addition, the user plane resources of the MA-PDU Session may be released due to data inactiviy or deregistration over an access. In some cases, the MA-PDU Session may not have user plane resources over any access. Then the UE initiates this procedure to request activation of user plane resources.

4.X.7.1 Non-roaming and Roaming with Local-Breakout

The procedure for adding adding user plane resources for a MA-PDU Session and the UE is non-roaming or roaming with local breakout, is based on the signalling flow in clause 4.2.3.2, with the following clarifications and modifications:

-	In step 1, the UE includes a PDU Session ID of a MA-PDU Session in the List Of PDU Sessions To Be Activated.

-	In step 11, the SMF provides N2 SM information in the PDUSession_UpdateSMContext Response to establish a user plane resources over an deactivated access. The SMF includes Access Type in the response to provide routing information for the N2 SM information.

Editor’s note: It is FFS whether the SMF establishes user plane resources in both access when there is no user plane resources in any access or the UE initiates Service Request in each access.

-	In step 12, the AMF sends N2 Request to the access network indicated in the Access Type.

4.X.7.2 Home-routed Roaming in the same PLMN

Editor’s note: It is FFS how to support home routed roaming in the same PLMN.

* * * * End of Changes * * * *